3GPP TSG-SA5 Meeting #163	S5-254375

Wuhan, China, 13.-17. October 2025

Source:	China Unicom

Title:	Rel-20 pCR TR 28.886 Add new use case on radio resource optimization based on per SSB usage

Document for:	Approval

Agenda item:	6.20.7

Spec:	3GPP TR 28.886

Version:	0.1.0

Work Item:	FS_eMDAS_Ph4

Comments

Current use cases in TS 28.104 do not have radio resource optimization based on per SSB usage. This use case is proposed to provide intelligently adjust the SSB beam configuration to improve the efficiency of radio resource.

Proposed Changes

* * * First Change * * * *

# 4	Use cases

## 4.y	Use case#y: Radio resource optimization based on per SSB usage

### 4.y.1	Description

5G NR adopts massive MIMO technology, where the base station employs directional beams for both UE initial access on common channels and sequent connection during the RRC_CONNECTED state. Common signals/channels for initial access are transmitted within the Synchronization Signal Block (SSB). SSBs are designed with different patterns, corresponding to the number of SSB beams and their distinct directions.

The following two figures describe different beam management scenarios. Figure 1 illustrates an SSB beam serving a user accessing the cell. Figure 2 depicts time-shared SSB beams fulfilling access requirements based on multi-user distribution. Time-shared implies modifying the coverageShape within targeted time periods. It calculates Number of UE related the SSB beam Index (mean), DL PRB Usage per SSB,etc within the serving cell, and estimates user distribution to modify the coverageShape.

The area where a large outdoor stadium holds some competitions are characterized by a high number of users generating low-volume traffic. .It’s challenging to accurately modify the coverage  based solely on traffic statistics. It is necessary to consider utilize intelligent technology to improve radio source usage.The network management system acquires the target cell's beam-level data, commands the gNodeB to update the target cell's coverage shape according to these beam-level data as shown in the figure,. The beam-level data includes Number of UE related the SSB beam Index (mean), DL PRB Usage per SSB,etc.

It is anticipated that MDA (Multi-Dimensional Analysis) can be utilized to obtain predictions for the usage of each SSB at the granularity level of geographical coordinates. This information can then be used to intelligently adjust the final coverage shape to improve radio resource efficiency.

| Figure 1: Illustration of SSB beams covering full cell coverage area. | Figure 2: Illustration of increased number of SSB beams covering to meet the UE centralized coverage. |
| --- | --- |

### 4.y.2	Potential requirements

REQ-RRO-FUN-1: MDA capability for radio resource optimization should provide the predicted coverage shape for the area to recommend the beam forming configuration.

4.y.3		Potential solutions

4.y.4	Evaluation of solutions

* * * End of Changes * * * *