3GPP TSG-SA5 Meeting #163	S5-254396

Wuhan, China, 13. - 17. October 2025

Source:	Huawei

Title:	Rel-20 pCR TR 28.883 Enhancement on NDT reporting method

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.0.0

Work Item:	FS_NDT_Ph2

Comments

It is proposed to add use case for enhancement on NDT reporting method, related to work task:

WT-4: Investigate new use cases and requirements that require enhancement of NDT in 3GPP management system.

In TS 28.561, the operations (e.g., createMOI operations) and notifications (e.g., notifyMOIAttributeValueChanges) of generic provisioning MnS defined in TS 28.532 is used for NDT report management. However, different NDT job scenario may indicate different requirements (e.g. timeliness) of reports and MnS consumer may also have their own preference on reporting methods. For example, in the scenario of verification of network response to signalling storm, timely simulation/emulation results may be needed to reflect real time network impacts. Therefore, streaming data reporting service defined in TS 28.532 may need to support NDT reporting which can be requested by NDT MnS consumer.

Proposed Changes

* * * First Change * * * *

5.X	Use Case #X: Enhancement on NDT reporting method

5.X.1	Description

In TS 28.561 [x], the reporting method of NDT output is notification based reporting. The NDT MnS consumer receives an NDT report from NDT producer by invoking generic provisioning management service operations and notifications in TS 28.532 [y]. However, different NDT job may have different requirement on reporting method. For example, in the scenario of signalling storm, it is more appropriate to use streaming data reporting service to timely report the simulation/emulation information on potential network impact.

It is proposed to enable the NDT MnS consumer to select the reporting method of simulation/emulation output based on different requirement. In addition to notification based reporting, streaming data reporting service also need to be supported for NDT output reporting.

5.X.2	Potential requirements

REQ-NDTRM-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to request NDT report to be provided by streaming data reporting service.

REQ-NDTRM-02: The 3GPP management system should support a capability for NDT MnS producer to support delivery of NDT report by streaming data reporting service.

5.X.3	Potential solutions

5.X.4	Evaluation of potential solutions

* * * End of Changes * * * *