3GPP TSG-SA5 Meeting #163	S5-254425

Wuhan, China, 13 - 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on Time Issue of External Management Data

Document for:	Approval

Agenda item:	6.20.7

Spec:	3GPP TR 28.887

Version:	0.1.0

Work Item:	FS_MADCOL_Ph3

Comments

The group is requested to discuss and approve the pCR below.

The content of clause 4.1.1.2 has been agreed in S5-253500. However, it has not been implemented correctly in the draft TR 28.887 in S5-254123. This pCR is to correct this. Additionally, some editorial corrections on Editor’s Note’s are proposed for clauses 4.1.1.3 and 4.1.1.4.

Proposed Changes

* * * First Change * * * *

## 4.1 	Request and Report of External Management Data

### 4.1.1	Use Case#1: Time Issue of External Management Data

#### 4.1.1.1      Description

External management data are modelled in TS 28.622[X] by ExternalDataType IOC (clause 4.3.73). This IOC defines attributes to indicate the type of external management data and related meta data.

Specified meta data are

- mediaLocation: “address from which the described external management data can be retrieved. “

- externalDataTypeSchema: “URI where the MnS consumer can get the schema to parse the external management data.”

- externalDataScope: “concrete scope (e.g., geographical areas) which the external management data is applicable.”

#### 4.1.1.2      Problem Statement

The attribute mediaLocation of IOC ExternalDataType specifies the endpoint where to retrieve the external management data. However, there is no indication on the validity of this data. E.g. for which period of time applies the provided event schedule; is the described UE path a record of the past, or a prediction for the future. The information of time is of importance in case of a request for historical data as well as for present data or for future data.

The explicit information of time regarding the requested or reported external management data is missing.

Note that external management data has the format and structure as provided by the external source and SA5 has no means to modify it.

#### 4.1.1.3      Potential Requirements

Editor’s Note: This clause is to describe potential requirements for the identified issue.

#### 4.1.1.4      Potential Solution

Editor’s Note: This clause is to discuss possible solutions for the identified issue.

* * * End of Changes * * * *