3GPP TSG-SA5 Meeting #163	S5-254573

Wuhan, China, 13. -17. October 2025

Source:	Ericsson

Title:	Rel-20 pCR TR 28.883 Defining the Lifecycle and Runtime Behaviour of NDT Jobs

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.0.0

Work Item:	FS_NDT_Ph2

Comments

This contribution proposes studying and addition of the ProcessMonitor <<datatype>> from TS 28.622 Clause 4.3.43. to the NDTjob <<InformationObjectClass>> from TS 28.561 Clause 6.2.1.3.2

Proposed Changes

* * * First Change * * * *

# 5	Use cases

## 5.X	Use Case #1 Defining the Lifecycle and Runtime Behaviour of NDT Jobs

### 5.X.1	Description

The definition of the NDTJob state machine is not fully completed, there are some items open to interpretation which are described below. There is an existing requirement that specifies that the MnS Consumer should be able to control the lifecycle of the simulation activities see REQ-DTLCM-02 in TS 28.561, Clause 5.1.3.

The NDT MnS Producer receives a request from the MnS Consumer to create a job.

Upon receiving the request, the NDT MnS Producer instantiates and executes the job simulation/emulation immediately.

According to TS 28.561, clause 4.4, creation and execution is handled in a single use-case. Whereas in TS 28.561, clause 6.4, there is a separate creation and execution interaction between the MnS Consumer and MnS producer.

There is ambiguity in the specification regarding whether the job persists after completion or is removed:

Option 1: The job persists after finishing, allowing re-running simulations or further configuration.

Option 2: The job is removed immediately after completion, implying creation and execution are inseparable.

This use-case suggests to define the NDTJob state machine by including the ProcessMonitor <<datatype>> from TS 28.622 Clause 4.3.43.

### 5.X.2	Potential requirements

REQ-DTLCM-X: The 3GPP management system should support a capability to enable an authorized MnS consumer to monitor the lifecycle state of an NDTJob instance.

### 5.X.3	Potential solutions

tbd

### 5.X.4	Evaluation of potential solutions

tbd

# 6	Conclusions and Recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *