3GPP TSG-SA WG6 Meeting #69	S6-254235

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#3 on Use Sensing Results for Creating Spatial Map

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #3 (TR 23.700-15 clause 4.3), the study aspects on use of sensing results for spatial maps include:

- Whether and how the enabler layer can use sensing results for determining the presence of objects (e.g. humans, animals, and vehicles) within an area of interest in a spatial map.

- Whether and how the enabler layer can associate a sensing result with an object within a spatial map.

- Whether and how the enabler layer can localize objects and detect object movements within a spatial map from sensing results.

- How the enabler layer can support SM server with creating SM.

The Spatial Map Server can benefit from 3GPP and non-3GPP sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map. Thus, this pCR proposes to also consider non-3GPP sensing results and sensing application enablement service (SAE, which is introduced in S6-254233) for creating spatial map.

2. Reason for Change

To address the study aspect on “How the enabler layer can support SM server with creating SM” in Key Issue #3, this pCR proposes a new solution on use combination of 3GPP and non-3GPP sensing results for creating spatial map.

3. Conclusions

This paper proposes solution on combination of 3GPP and non-3GPP sensing results for creating spatial map.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X1 |  |  | X |  |

* * * Next Change * * * *

## 6.X1	Solution #X1: Use Sensing Results for Creating Spatial Map

### 6.X1.1	Solution Description

The Spatial Map Server can benefit from 3GPP and non-3GPP sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map.

The following clauses specify use combination of 3GPP and non-3GPP sensing results for creating spatial map.

#### 6.X1.1.1	Procedure

Figure 6.X1.1.1-1: Sensing results for creating spatial map

- The Spatial Map Server sends sensing result request for consolidated sensing results from the SAE Server.

- The SAE Server authenticates the request, and determines the sensing entities (e.g., SAE client, 5GC NFs, 3rd party SAE server) for collection of sensing data.

- The SAE Server responds to the SM server for the sensing result request.

- The SAE Server interacts with the SAE Client (deployed on a non-3GPP sensor) for non-3GPP sensing data collection.

4a.	The SAE Server sends sensing data request to the SAE Client, which is deployed on a non-3GPP sensor, to collect the non-3GPP sensing data measured by the non-3GPP sensor.

4b.	The SAE Client requests the Sensing Entity to measure the surrounding environment, and forwards the sensing data to the SAE Server.

- The SAE Server interacts with the NEF to collect the sensing result measured by the 3GPP network.

- The SAE Server interacts with the 3rd party SAE server to collect third party sensing data measured.

- The SAE Server consolidates the sensing data/results from different sources in steps 4-6 to generate sensing result (e.g. draft spatial map).

- The SAE Server sends sensing result response to the SM server with the consolidated sensing results.

- The Spatial Map Server creates spatial map based on the consolidated sensing result.

#### 6.X1.1.2	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X1.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X1.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***