3GPP TSG-SA6 Meeting #70	S6-255058

# Dallas, USA, 17th - 21st Nov 2025			                                         (revision of S6-xxxxxx)

Source:	Airbus

Title:	Comments on the proposed MC service UE disable procedure

Spec:	3GPP TS 23.280

Agenda item:	10.2

Document for:	Discussion and agreement

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This discussion paper provides Airbus comments to S6-254307 - MC service UE remote management – Disable procedure.

Our comments are summarized for each change (in S6-254307) and also shown with track changes.

S6-254307 : * * * First Change * * * *

Airbus comments :

- minor clarifications

- note that ambient listening information flows currently do not contain “MC service UE id” i.e. should not (yet) be mentioned here

- note also that discreet listening feature does not exist yet i.e. should not be mentioned here

### 8.1.X	MC service UE identifier

The MC service UE identifier is a unique identifier enabling to distinguish between different MC service UEs in use by the same MC service ID. The MC service UE identifier enables the authorized MC service user or MC service administrator to target certain MC service operations (e.g. Remote management) to a particular MC service UE when multiple MC service UEs are in use by the same MC service user. MC service client shall share the MC service UE identifier to the MC service server while performing the MC service authorization. MC service server shall maintain the mapping of MC service IDs, MC service UE identifier and public GRUUs and this mapping is made available to the authorized MC service user or MC service administrator when requested.

NOTE:	The MC service UE identifier may be provided during initial MC service UE configuration, see clause A.6.

S6-254307 : * * * Next Change * * * *

Airbus comments :

- In our view, since disable/re-enable operations impact the whole “MC service UE” i.e. all MC clients in that UE, these operations should not be mixed to the MC service client-server procedures. Otherwise, to follow the logic in 3GPP MC specifications, disabling/re-enabling a MC service UE may require sending disable/re-enable command separately for MCPTT, MCVideo, MCData clients.

- Furthermore, this kind of solution would mean that an administrator/authorized user MUST have all clients (MCPTT, MCVideo, MCData) active to be able to execute disable/re-enable commands.

- In our view, a much better solution is to isolate this feature into a separate ‘layer’. Compare this e.g. to the Recording admin user concept.

- The “next change” of S6-254307 is revised accordingly, see below.

## 10.X	Disabling and Re-enabling of MC service UE

### 10.X.1	General

Remote management of MC service UE provides a mechanims for the MC service administrator or authorized MC service user to disable an MC service UE of another MC service user temporarily from accessing the MC services or to re-enable the MC service UE which is temporarily disabled. When an MC service UE is temporarily disabled, the MC service clients log off from the MC services and the MC service UE enters into a temporarily disabled state. When the MC service UE is in temporarily disabled state, no MC service operations are possible.. This subsclause describes the generic procedures for disabling and re-enabling of an MC service UE by MC service administrator or authorized MC service user.

### 10.X.2	Information flows

#### 10.X.2.1	MC service UE disable request (MC device management admin client – MC device management server and MC device management server – target MC device management client)

Table 10.X.2.1-1 describes the information flow MC service UE disable request from MC device management admin client to an MC device management server and from MC device management server to the target MC device management client.

Table 10.X.2.1-1: MC service UE disable request

| Information element | Status | Description |
| --- | --- | --- |
| MC device management ID | M | The identity of the MC device management administrator requesting to disable the MC service UE. |
| MC service ID | M | The identity of the MC service user whose MC service UE needs to be disabled. |
| MC service UE identifier | M | Unique identifier to identity the target MC service UE that needs to be disabled. |

#### 10.X.2.2	MC service UE disable response (MC device management server – MC device management admin client and target MC device management client – MC device management server)

Table 10.X.2.4-1 describes the information flow MC service UE disable response from MC device managementserver to requesting MC device management admin client and from the target MC device management client to the MC device management server.

Table 10.X.2.2-1: MC service UE disable response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Indicate the MC service ID of the target user whose device is being disabled. |
| MC service UE identifier | M | Indicates the MC service UE that is being disabled. |
| Success result | O (see NOTE 1) | Indicates MC service UE disable request is successful and also indicates whether the disable request is delivered to the target MC service UE or not. |
| Failure result | O (see NOTE 1) | Indicates that MC service UE disable request is failure and the reason. |
| NOTE 1: Only one of these IE shall be present. |  |  |

S6-254307 : * * * Next Change * * * *

Airbus comments :

•	MC device management admin client instead of MC service client 1

•	MC device management server instead of MC service server

•	MC device management client instead of MC service client 2

### 10.X.3	Disabling MC service UE

Figure 10.X.3-1 below illustrates the procedure of disabling MC service UE by an authorized MC service user or MC service administrator.

Figure 10.X.3-1: Disabling MC service UE by an authorized MC service user or MC admin

1.	MC DM admin client of the authorized user retrieves the MC service ID – MC service UE binding information of the target MC service user. This information includes the binding of all MC service IDs of the target user (MCPTT id, MCVideo id, MCData id).

2.	MC DM admin client sends the request to the MC device management server to disable a MC service UE. This request shall include the MC device management identity of the requesting MC device management admin user, the MC service identity of the target user whose MC service UE has to be disabled and a unique identifier to identify the MC service UE which has to be disabled.

3.	MC device management server checks whether MC device management admin client is authorized to perform the MC service UE disable operation.

4.	MC device management server stores the information that the MC service UE identified by the identifier provided in step 2 is disabled, and shares this information to the MC service servers and all other servers in the MC system. MC service servers shall not allow the MC service clients residing on the disabled UE to access the MC services.

5.	MC device managment server sends the MC service UE disable request to the particular MC service UE based on the UE identifier received in step 2. If the MC service UE disable request is not delivered to the target MC device management client due to connectivity issues, the MC device management server shall attempt to re-send the request on periodic basis or when the MC service UE becomes online.

6.	MC device management client acknowledges the receipt of MC service UE disable request by sending the MC service UE disable response to the MC device management server.

7.	The MC service clients of the target MC service UE log off from the MC services without user intervention and all clients of that MC service UE enter into the temporarily disabled state.

Editor’s Note: Impacts of the temporarily disabled state to the MC service authorizations shall be defined in TS 33.180 by SA3.

8.	MC device management server sends MC UE disable response to the MC device management admin client which requested to disable the MC service UE.

S6-254307 : * * * Next Change * * * *

Airbus comments :

•	MC service server is not managing the binding information but only providing (the info can be stored e.g. in a database)

•	Ambient listening call does not utilize MC device ID (yet) and Discreet listening call feature does not yet exist.

## 10.Y	MC service ID binding information

### 10.X.1	General

MC system may allow an MC service user being logged into multiple devices (MC service UEs) and to access the MC services from one or more MC service UEs simultaneously. The binding information between MC service IDs and the MC service UE identifiers is received by the MC service servers during the client registration phase. The MC service servers may store this binding information into a database and/or share it with other relevant servers in the MC system. Whenever MC service user is logged into or logged out from an MC service UE, this binding information is updated by the MC service server. The MC service server shall update the binding information also when a different MC service user logs in from an MC service UE already in use by another user. When this binding information is made available to the authorized MC service user or MC service administrator, they can request the MC device management server to target a particular MC service UE for certain MC operations (e.g. Remote management of MC service UE). This subsclause describes the generic procedures for retrieval of the MC service ID binding information from the MC device management server by the authorized MC service user.

### 10.Y.2	Information flows

#### 10.Y.2.1	Retrieve MC service ID mapping request (MC device management admin client – MC device management server)

Table 10.Y.2.1-1 describes the information flow retrieve MC service ID mapping request from MC device management admin client to an MC device management server.

Table 10.Y.2.1-1: Retrieve MC service ID mapping request

| Information element | Status | Description |
| --- | --- | --- |
| MC device management ID | M | The identity of the MC device management admin user requesting to retrieve the mapping information of the particular MC service ID. |
| MC service IDs | M | The MC service identities of the MC service user whose mapping information is requested. |

#### 10.Y.2.2	Retrieve MC service ID mapping response (MC device management server – MC device management admin client)

Table 10.Y.2.2-1 describes the information flow retrieve MC service ID mapping response from MC device management server to the requesting MC device management admin client.

Table 10.Y.2.2-1: Retrieve MC service ID mapping response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the retrieve MC service ID mapping request. |
| MC service ID mapping information | O | This IE shall be present if the result is success. It contains the mapping information of the MC service IDs which includes the list of MC service UE identifiers and the corresponding MC service UE labels and public GRUUs etc. |

S6-254307 : * * * Next Change * * * *

Airbus comments :

-this procedure is between MC device management admin client and MC device management server

### 10.Y.3	Retrieval of MC service ID mapping information

Figure 10.Y.3-1 below illustrates the procedure of retrieval of MC service ID mapping information by authorized MC service user or MC service administrator from the MC service server.

Figure 10.Y.3-1: Retrieval of MC service ID mapping information

1.	MC service administrator or authorized MC service user, using a MC device management admin client, requests the MC device management server to return the binding information for a particular MC Service ID or IDs. This request carries the MC device management ID of the MC service administrator or authorized MC service user, MC service IDs of the target user whose mapping information is required.

2.	MC device managemen server checks whether the requestor in step 1 is authorized to fetch the binding information of the target MC service ID(s) specified in the request.

3.	MC service server returns either success or failure response. In case of success, the response includes binding information for the target MC service ID(s) as specified in Table 10.Y.2.2-1.