3GPP TSG-SA WG6 Meeting #70	S6-255082

Dallas, United States 17th – 21st November 2025

Source:	InterDigital

Title:	Solution #8 and #9 updates

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Quang Ly, <quang.ly@interdigital.com>

1. Introduction

Propose updates to solution 8 and 9.

2. Reason for Change

In solution 8, it is unclear why the SM server initiates sensing result request. It is proposed to clarify that the sensing result request is triggered by a spatial map create or a spatial map update as described in Clause 6.11.

In solution 9, added an option for a VAL Server/SEAL SM Server that knows about the availability of a sensing service within a SEAL SM server to include a sensing service policy Create Spatial Map Request message as described in Clause 6.11.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

#### 6.9.1.1	Procedure

Figure 6.9.1.1-1: Sensing results for creating spatial map

1.	The Spatial Map Server sends sensing result request for consolidated sensing results from the SEAL Sensing Server. The Spatial Map Server is triggered to send the sensing result request due to a spatial map create or update procedure as described in clause 6.11.

2.	The SEAL Sensing Server authenticates the request, and determines the sensing entities (e.g., 5GC NFs, 3rd party sensing server) for collection of sensing data.

3.	The SEAL Sensing Server responds to the SM server for the sensing result request.

4.	The SEAL Sensing Server interacts with the NEF to collect the sensing result measured by the 3GPP network. The SEAL Sensing Server determines whether the sensing result fulfils the request from the consumer provided in step 1, and determines whether additional sensing data is needed to enhance the sensing result.

5.	The SEAL Sensing Server interacts with the 3rd party sensing server to collect third party sensing data measured.

Editor's Note:	Whether the revision of the architecture is needed for the 3rd party sensing server is FFS.

6.	The SEAL Sensing Server consolidates the sensing data/results from different sources in steps 4-5 to generate sensing result (e.g. draft spatial map).

7.	The SEAL Sensing Server sends sensing result response to the SM server with the consolidated sensing results.

8.	The Spatial Map Server creates spatial map based on the consolidated sensing result.

* * * Next Change * * * *

### 6.10.2	Procedures

#### 6.10.2.1	Create spatial map with sensing capabilities

Figure 6.10.2.1-1 below illustrates the procedure for creating a spatial map when sensing capabilities are introduced. For the request from VAL server or SEAL SM client, as a spatial map consumer. SEAL SM server creates a spatial map by utilizing sensing capabilities provided by SEAL Sensing server.

Pre-conditions:

1.	The VAL Server or SEAL SM client is authorized to use APIs provided by the SEAL SM server.

2.	The SEAL SM server can interact with SEAL Sensing server via SEAL-X interface.

3.	The SEAL Sensing server is authorized to use Sensing related capability APIs provided by sensing related function in the 3GPP core network or authorized to use the NEF exposed Sensing related capability.

NOTE:	In this solution, it is assumed that neither the VAL Server nor the SEAL SM Server has sufficient knowledge to understand the detailed use of sensing-related capabilities/parameters provided by the 3GPP core network, whether directly or via the NEF. Alternatively, If the VAL Server/SEAL SM Server knows that sensing service can be provided by SEAL sensing server, the VAL Server/SEAL SM Server may include a sensing indicator in the request. If so, the SEAL SM server will use the sensing indicator to decide whether to utilize sensing capabilities.

.

Figure 6.10.2.1-1: Create spatial map procedure

1.	The requester (e.g., VAL server or SEAL SM client) sends a Create Spatial Map Request message to the SEAL SM server to create a spatial map. The request includes the same IEs as specified in clause 9.3.1.3.1 of 3GPP TS 23.437 [8]. If the VAL Server/SEAL SM Server knows that sensing service can be provided by a SEAL sensing server, the VAL Server/SEAL SM Server may include a sensing service policy as described in clause 6.11.

2.	The SEAL SM server decides to utilize sensing capabilities to assist the creation of the spatial map based on local policy. If SEAL SM server decides to utilize sensing capabilities, it sends a Get Sensing Results Request to SEAL Sensing server. The request includes requester ID, security credentials and three-dimensional area of interest.

Editor's note:	the detailed IEs included in the Get Sensing Result Request is FFS.

3.	The SEAL Sensing server sends a Sensing Service Request either directly to a sensing-related function in the 3GPP core network or to this function via the NEF (e.g., using the Nnef_Sensing_Subscribe service operation). The request includes the Target Sensing Area information. This information is derived from a three-dimensional area of interest and can be handled by 3GPP core network.

Editor's note:	the Sensing Service Request may be used to trigger the sensing service provided by 3GPP core network, or used to request the sensing result. How the SEAL sensing server utilizes the sensing service will be based on SA2 progress and is FFS.

4.	The sensing-related function in the 3GPP core network sends the Sensing Service Response either directly to the SEAL Sensing server or via the NEF. The response includes the sensing result (with or without the sensing contextual information).

Editor's note:	the detailed IEs included in the step 3 and step 4 are based on the output of SA2 and are FFS.

5.	The SEAL Sensing server sends the Get Sensing Result Response to the SEAL SM server. The response includes the List of objects information in the Area of interest obtained from the sensing service.

6.	The SEAL SM server creates the spatial map as per step 2 in clause 9.3.1.2 of 3GPP TS 23.437 [8]. In this step, the processed sensor data may be sourced from step 5 instead of the VAL layer.

7.	The SEAL SM server sends response message to the requester as per step 3 in clause 9.3.1.2 of 3GPP TS 23.437 [8].

8.	If the response in step 7 indicates "in-progress", the SEAL SM server notifies the requester with the spatial map ready notification message as per step 4 in clause 9.3.1.2 of 3GPP TS 23.437 [8].

* * * End of Change * * * *