3GPP TSG-SA WG6 Meeting #70	S6-255099

Dallas , US, 17th – 21st November 2025

Source:	China Telecom

Title:	New solution for KI#2 on AIML service maintenance on satellite

Spec:	3GPP TR 23.700-002 v0.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	Zhou Zhe <zhouz11@chinatelecom.cn>

Xu Xia <xiaxu@chinatelecom.cn>

1. Introduction

When the EDN located on a satellite stores ML models for AIMLE client to download an ML model to support AIML service, the UE would encounter connection problem. The scenario exists that the satellite as an ML model source moves to another place and the UE leaves the satellite’s coverage, leading to the situation that the UE loses the connection and fails to download the satellite deployed ML model. It is worthwhile to expolore ways to ensure onboard AIML service continuity. This solution addresses the following key issue:

Key issue #2: Satellite based AIML service maintenance while losing connection with terrestrial network

This paper proposes a new solution on KI#2, to maintain satellite access based AIML service by leveraging AIMLE server interworking. When the satellite moves to another place, the UE fails to download the satellite deployed ML model. The UE reports to the AIMLE server the failure reason and asks for a new ML model. The AIMLE server has to retrieve a new satellite deployed ML model for the AIML service. If the AIMLE server can’t discover the proper ML model by retrieving in the initial ML repository, the AIMLE server could interwork with another AIMLE server to get another ML repository. When the AIMLE server discovers the proper satellite deployed ML model by retrieving ML model information in another ML repository, it may response to the UE and let the UE continue the ML model downloading.

2. Reason for Change

To support AIML service maintenance on satellite.

3. Conclusions

This paper proposes solution on AIML service maintenance on satellite.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-002 v0.1.0.

* * * First Change * * * *

5.X	Solution#X: AIML service maintenance on satellite while losing connection with terrestrial network

5.X.1	Solution Description

This solution proposes to address the following open issues in KI#2

-	Whether and how to support satellite based AIML service management when losing connection with terrestrial network (e.g. When a satellite is supporting AIML service connection or related data collection, the satellite keeps connection with terrestrial network due to exceeding the available time window).

5.X.1.1 General

Figure 5.X.1.1-1: satellite AIMLE deployment

In this deployment option, the UE with AIMLE client is on the ground or in the air (e.g., ​​a drone​​), requiring satellite deployed ML models from AIMLE server. The AIMLE servers are deployed on the ground with ML repositories. The space AIMLE component is deployed on ​​a​​ satellite, performing as a VAL server in the satellite EDN containing multiple AIML models. The RAN (e.g., gNB) can be deployed on ​​the​​ ground/sea (e.g., on a ship). The 5GS control plane functions (e.g., AMF, SMF) are deployed on the ground, ​​though they are not shown in the figure for simplicity​​. The satellite EDN can provide the information of AIML models located in satellite EDN to AIMLE server via AIML-S on space. Then the AIMLE server stores satellite deployed AIML models information with satellite EDN information in the ML repository.

The AIMLE client in the UE has discovered a proper satellite deployed ML model through AIMLE server and is downloading the ML model from the satellite. When the serving satellite moves away and the UE is not in the satellite coverage any more, the AIMLE client fails to continue downloading the ML model from the satellite and the failure is perceived by the AIMLE server. As a result, the AIMLE server has to retrieve a new ML model which is located on another satellite. However, the new satellite with the new ML model has not yet provided its satellite and ML model infromation to the AIMLE server and the ML repository. The AIMLE server could utilize the interworking between AIMLE servers to obtain extra ML repository information and finally find the proper satellite deployed ML model.

As mentioned above, the AIMLE client loses the connection with the satellite A EDN and fails to continue downloading the ML model via space, and then require new ML model deployed in satellite B EDN from the AIMLE server. To satisfy the requirement from the AIMLE client, the AIMLE server performs interworking with another AIMLE server. The latter AIMLE server offers the information of another ML repository to the former AIMLE server. Then the initial AIMLE server retrieves the proper satellite ML model in the new ML repository. If the initial AIMLE server finally finds the proper satellite ML model deployed in satellite B EDN, it responses to the AIMLE client and makes the AIMLE client continue downloading the proper ML model from the new satellite B EDN.

5.X.1.2	AIML service maintenance on satellite while losing connection with terrestrial network

Figure 5.Y.1.2-1 illustrates the procedure of AIML service maintenance on satellite while losing connection with terrestrial network.

Figure 5.Y.1.2-1: Satellite ML model retrieval

The corresponding procedure reuses the existing procedure for assist a hiearchitical computing process in clause 8.25 in TS 23.482. The procedure in detail is as follow:

- When initial AIMLE server cannot find the proper model by retrieving the ML models in its local ML repository, the AIMLE server sends hierarchical computing assistance request to another AIMLE server, in order to get the remote ML repository information. The request message includes information as described in Table 5.X.1.3.1-1.

- The latter AIMLE server authenticates and authorizes the request from the initial AIMLE server.

- If the request is authorized, the latter AIMLE server performs operations to generate assistance information which is requested in step 1. For retrieving new satellite ML models, according to the information sent in step 1, the latter AIMLE server provides the remote ML repository information.

- The latter AIMLE server sends hierarchical computing assistance response with the generated assistance information (e.g. ML repository information). The response message includes information as described in Table 5.X.1.3.2-1.

5.X.1.3	Information Flows

5.X.1.3.1	Hierarchical computing assistance request

Table 5.X.1.3.1-1 describes information elements of hierarchical computing assistance request from the initial AIMLE server to the latter AIMLE server.

Table 5.X.1.3.1-1: Hierarchical computing assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Original requestor identifier | O | The identifier of the original requestor, e.g. VAL server ID, EAS ID, CAS ID. |
| Role | M | Represents the role of the VAL server in a hiearchitical computing architecture (e.g. root node, sub-root node or leaf node of a hierarchical computing process). |
| Computing task type | M | The type of computing task (e.g. VFL, HFL). |
| Assistance information type | M | Represents the assistance information type, which is used to indicate the assistance information needed. ​​Pre-defined values include but are not limited to:​​ - candidate_execution_node_list: For requesting a list of candidate execution nodes. - computing_preparation_status: For requesting the computing preparation status at an execution node. - ML Repository information: For requesting information about available ML Repositories.​ |
| Execution node(s) | O | Represent one execution node or a list of candidate execution nodes. |

5.X.1.3.2	Hierarchical computing assistance response

Table 5.X.1.3.1-1 describes information elements of hierarchical computing assistance request from the latter AIMLE server to the initial AIMLE server.

Table 5.X.1.3.2-1: Hierarchical computing assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE 1) | Indicates that the assist hierarchical computing request was successful. |
| >Assistance information | M | The assistance information for assisting hierarchical computing process, e.g. candidate execution node list, computing preparation status at an execution node. |
| >>List of candidate execution nodes | O (NOTE 2) | A list of selected candidate execution nodes. |
| >>​​List of ML Repository information | O | A list of discovered ML Repositories that meet the requirements.​ |
| >>Preparation status | O (NOTE 2) | Computing preparation status at the execution node provided in request. |
| Failure response | O (NOTE 1) | Indicates that the assist hierarchical computing request was failure. |
| >Cause | M | Reason for the failure. |
| NOTE 1: One of the IEs shall be present. NOTE 2: At least one of the IEs shall be present. |  |  |

5.Y.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

5.Y.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

5.Y.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

* * * End of Changes * * * *