3GPP TSG-SA WG6 Meeting #70	S6-255116

Dallas, United States 17th – 21st November 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 CAPIF plus SEAL Topology

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

Add CAPIF plus SEAL Topology clause.

2. Reason for Change

Complete clause A.2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.2.0.

* * * First Change * * * *

## A.2	CAPIF+SEAL Topology

### A.2.1	General

This annex describes a deployment in a single PLMN in which the Common API Framework (CAPIF) integrates with the Service Enabler Architecture Layer (SEAL). The scenario illustrates how onboarding, registration, API publication, discovery, authorization, and invocation are carried out with CAPIF security in place. In this deployment, SEAL acts as the common service layer for vertical applications (VAL), while CAPIF provides the common exposure, onboarding, and token issuance functions.

This sample deployment includes the following entities:

-	a CAPIF Core Function (CCF), which provides onboarding, API provider registration, API catalogue, and authorization/token services,

-	a SEAL Server, acting as an API Provider for SEAL service enablers (e.g. group management, configuration, location),

-	one or more VAL Servers, which provide the actual vertical application logic, and which need to invoke SEAL APIs, and

-	one or more UEs hosting both a SEAL Client and a VAL Client, which consume SEAL-enabled services through the network.

The logical topology is shown in Figure A.2.1-1.

Figure A.2.1-1: CAPIF plus SEAL sample topology

### A.2.2 Step 1: SEAL Server Registration into CAPIF

The SEAL Server needs to be provisioned with the CAPIF Root CA, the CAPIF base URL, and bootstrap credentials (e.g. a bootstrap JWT or operator-issued credentials), in line with the CAPIF onboarding procedures (see 3GPP TS 23.222 [2] and security aspects in 3GPP TS 33.122 [7]). Using this information, the SEAL Server establishes a secure TLS connection to the CCF and registers itself as an API Provider. By doing so, the SEAL Server publishes its identity to CAPIF and becomes a trusted source of SEAL-related APIs (e.g. GroupManagement, LocationInfo, ConfigManagement). After successful registration, CAPIF can issue tokens scoped to those APIs, and other network entities can later discover them from the CAPIF API catalogue.

The flow is depicted in Figure A.2.2-1.

Figure A.2.2-1: SEAL Server registration flow

### A.2.3 Step 2: SEAL Server Publishes SEAL APIs

After being registered as an API Provider, the SEAL Server publishes its SEAL service APIs into CAPIF. These typically include SEAL functional APIs such as SS_GroupManagement, SS_LocationInfo, SS_Configuration, and SS_NetworkResourceAdaptation as described in TS 23.434. By publishing them, the SEAL Server enables other entities to discover which SEAL capabilities are available in that PLMN, what versions are supported, and what security methods must be used.

Figure A.2.3-1 shows this publication step.

Figure A.2.3-1: SEAL Server publish API flow

### A.2.4 Step 3: VAL Server Onboarding and Registration into CAPIF

Before consuming SEAL APIs, each VAL Server must first onboard to CAPIF as an API Invoker. For that purpose, the VAL Server is provisioned with the CAPIF Root CA, the base URL of the CCF, and bootstrap credentials. The VAL Server establishes a TLS session towards the CCF, authenticates using the bootstrap credentials, and performs the API Invoker onboarding procedure so that the CCF can issue it invocation tokens later. Depending on operator policy, the same VAL Server can also register as an API Provider (for example, to publish application-level APIs that UEs/VAL Clients may later invoke).

The onboarding and registration flows are depicted in Figure A.2.4-1.

Figure A.2.4-1: VAL Server onboarding and registration flow

### A.2.5 Step 4: VAL Server Discovers SEAL APIs

Once onboarded, the VAL Server needs to discover which SEAL APIs are available in the PLMN. To do that, it queries the CAPIF API catalogue. CAPIF returns the list of available SEAL APIs together with their resource URLs, versions, and security requirements. If the operator mandates OAuth2, the VAL Server then requests an access token from the CCF, indicating the SEAL API scope it wants to invoke.

This step is shown in Figure A.2.5-1.

Figure A.2.5-1: VAL Server discovery flow

### A.2.6 Step 5: VAL Server Creates Security Context

The VAL Server initiates the creation of a Security Context with the CCF, selecting OAuth as the security method. Upon successful mutual authentication, the CCF confirms the establishment of the secure context, which will be used in subsequent token and API invocations.

The combined flow is depicted in Figure A.2.6-1.

Figure A.2.6-1: Val Server Create Security Context Flow

### A.2.7 VAL Server Token Request and SEAL API Invocation

Once the Security Context has been successfully established, the VAL Server proceeds to obtain an OAuth2 access token from the CCF and invoke a SEAL enabler API on the SEAL Server. The token request includes the appropriate scope corresponding to the SEAL service that the VAL Server intends to access.

Upon receiving the request, the CCF validates the credentials and issues a signed JSON Web Token (JWT). The VAL Server then uses this token as a Bearer credential in the Authorization header when invoking the SEAL API.

The SEAL Server verifies the token, executes the requested operation, and returns the response to the VAL Server.

In this phase, the VAL Client simply consumes VAL APIs exposed by the VAL Server. These APIs may internally trigger SEAL API invocations, allowing the VAL Server to obtain group, configuration, or location data from the SEAL Server as needed. The VAL Client does not interact directly with CAPIF or the CCF; instead, all CAPIF-based security and authorization operations are handled by the VAL Server on its behalf.

The combined flow is illustrated in Figure A.2.7-1.

Figure A.2.7-1: VAL Client to VAL Server and SEAL API invocation flow

* * * End of Changes * * * *