3GPP TSG-SA WG6 Meeting #70	S6-255122

Dallas, United States 17th – 21st November 2025	(revision of S6-25xxxx)

Title:	Follow-up reply LS on N6 measurement exposure

Response to:	Reply LS on N6 measurement exposure (S2-2509811/S6-255010)

Release:	3GPP Rel-20

Work Item:	TEI20, EDGEAPP_Ph3

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG2

Cc:	n/a

Contact person:	Walter Featherstone

w_featherstone@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

# 1	Overall description

SA6 thanks SA2 for their reply LS on N6 delay measurement exposure and the clarifications regarding the use of such measurements by the SMF for Local UPF (re)selection.

SA6 have not specified in TS 23.558 EDGEAPP procedures that leverage N6 delay measurements relating to an EAS since such measurements are not currently exposed. However, it is recognized that the core network can perform N6 delay measurements when an AF provides EAS Deployment Information (EDI). Since the EDI may include the DNAI(s) associated with an EAS, SA6 believes that per-DNAI N6 delay measurement information could be valuable to the application enablement layer.

As background, when an EAS registers with an Edge Enabler Server (EES) it provides an EAS Profile, whose information elements are supplied by the Application Service Provider (ASP) and are therefore relatively static. Examples include:

- the “List of EAS DNAI(s)”, which are a subset of the DNAI(s) associated with the Edge Data Network (EDN) where the EAS resides and can be used as Potential Locations of Applications; and

- the “EAS Service KPIs”, which describe the service characteristics provided by EAS and include can maximum response time.

The maximum response time includes the round-trip time of the request and response packet for communication between the Application Client (AC) and EAS, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any.

It is already specified in TS 23.558 that an EES, acting as an AF, may request a specific QoS for an AF session or traffic flow for a UE (or group of UEs). Such a request can include a (UE to PSA UPF) RT Latency Indication (TS 23.502, clause 5.2.6.9.2 and TS 23.503, clause 6.1.3.27.2). Knowledge of the prevailing N6 delay measurement information, as a component of the target end-to-end EAS service latency (maximum response time), could help an EES provide a more accurate RT Latency Indication.

Another potential scenario is EAS discovery, including in support of Application Context Relocation procedures. Depending on the trigger (e.g., EEC/UE, EAS itself, Cloud Application Server (CAS), or another EES), if N6 delay measurement information were available to the EES it could assist in ranking or selecting candidate EAS Profiles to return in the discovery response.

Both these scenario are illustrative examples only; SA6 has not yet specified procedures that directly consume N6 delay measurements.

Regarding SA2’s concern about possible network-topology information exposure, SA6 clarifies that its request concerns N6 delay measurement exposure per DNAI. Such data represents performance information, not topology. The DNAI is an opaque logical label identifying user plane access to a data network in which an EAS can reside, and it does not disclose UPF identifiers, IP addresses, or port numbers. Therefore, exposing performance metrics per DNAI would not reveal network topology, consistent with the topology-hiding requirements.

Based on the above additional information and use-case clarification, SA6 would like to ask SA2 whether they could support exposure of N6 delay measurement information (per DNAI) to SA6-defined entities (considered as AFs by SA2) in Release 20.

# 2	Actions

To SA2:

ACTION: SA6 kindly requests SA2 to consider support for exposure of N6 measurement information.

# 3	Dates of next TSG SA WG 6 meetings

SA6#71                 9th – 13th February 2026 		Goa, India

SA6#72                 13th – 17th April 2025 		Malta