3GPP TSG-SA WG6 Meeting #70	S6-255135

Dallas, USA, 17th – 21th Nov 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	Solution evaluation for Sol#2

Spec:	3GPP TR 23.700-02 v0.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes to provide the solution evaluation for Sol#2.

2. Reason for Change

To complete the Sol#2 and the TR.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-02 v0.2.0.

* * * First Change * * * *

5.3	Solution #2: Support of QoS analysis for services over satellite access

5.3.1	Solution description

This solution addresses the KI#6.

The VAL server or the UE may act as consumers to subscribe the QoS analysis related to the services over satellite access from ADAES. They may ask to provide the preferred or predicted QoS for the requested service over satellite access.

5.3.1.1	Procedure of QoS analysis for services over satellite access

Figure 5.3.1.2-1 illustrates the high-level procedure of QoS analysis for services over satellite access.

Pre-condition:

-	The following NFs (e.g., ADAES) are all deployed on the ground.

- 	 The target UE is accessing the satellite.

Figure 5.3.1.1-1: Procedure of QoS analysis for services over satellite access

1.	Consumers (e.g., the VAL server or UE) initiate a satellite communication analysis subscription request to the ADAES, with the analysis ID (i.e. QoS for the satellite communication), analysis type (statistics or prediction), target UE ID, service ID, target service area, reporting conditions (e.g., periodic reporting or event-triggered reporting), service QoS parameters, etc.

NOTE 1:	The definition of QoS can refer to the clause 5.7 of 3GPP TS 23.501 [10].

2.	The ADAES checks whether the VAL server is authorized to invoke such request.

3.	If the request is authorized, the ADAES enabler sends a satellite communication analysis subscription response to the VAL server.

4.	ADAES may obtain the satellite communication related information for the target UE from the 3GPP CN (i.e., NWDAF), including the location information, RAT type (i.e. satellite), UE mobility analysis information, the DN performance and QoS analysis information (such as the time delay, the traffic data flow and the uplink/downlink QoS parameters when the UE accesses via satellite access) as specified in clause 6.14 and clause 6.23 of 3GPP TS 23.288 [9]).

5.	Upon receiving the satellite communication related information for the target UE from step 4, the ADAES stores and analyses these information based on the service request received in step 1 to generate the performance or service quality (i.e., QoS) statistics and prediction related to satellite access. E.g., the ADAES may provide the preferred satellite and the preferred satellite type (such as GEO, MEO, LEO) when the UE is using the satellite access to meet the requested QoS requirements, or the recommended/predicated QoS parameters for the specific service when the UE is accessing the dedicated satellite.

6.	The ADAES will report the QoS analysis related to satellite communication, including preferred and prediction information (e.g. the preferred satellite and satellite type, recommended/predicated QoS parameters), to the consumer via satellite communication analysis notification message. And the consumers may take actions to minimize the service impact if there is. For example, when the target UE is accessing the GEO satellite, the VAL server may decrease the service QoS (e.g., latency), etc.

NOTE 2:	The actions from the consumer to minimize the service impact are out of scope of this specification.

5.3.2	Architecture Impacts

This solution has no impact on the existing architecture.

5.3.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

5.3.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

This solution addresses the KI#6 to support the QoS analysis for the services over the satellite access in the application enablement layer.

The solution has no impacts on the existing architecture. The ADAES can be enhanced to obtain the satellite communication related information for the target UE from the 3GPP core network and generate the preferred or predicted QoS analysis (e.g. the preferred satellite and satellite type, recommended/predicated QoS parameters, etc.) for the requested services over the satellite access.

NOTE: Whether the procedure 5.2.1.1 of Sol#1 and the procedure 5.3.1.1 of Sol#2 can be combined or not will be discussed in normative phase.

* * * End of Change * * * *