3GPP TSG-SA WG6 Meeting #70	S6-255136

Dallas, USA, 17th – 21th Nov 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	Update Sol#3 to fix the EN

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes to update the Sol#3 to fix the EN.

2. Reason for Change

The contribution mainly proposes to update the Sol#3 to fix the following EN and also makes some clarification and correction for the whole procedure.

Editor's note:	How the ADAES and SEALDD support the sensing related functionalities are FFS.

First, the Sensing Enabler server may interact with SEALDD as specified in clause 9.7 of 3GPP TS 23.433 [23433] to get the e.g., the transmission latency for the target object utilizing the obtained sensing results from the 3GPP CN.

Secondly, the Sensing Enabler server may ask ADAES for sensing accuracy analytics which described in new clause 6.4.1.1.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

6.4	Solution #3: Support of exposure of sensing results

This solution addresses the KI#1.

6.4.1	Procedure of exposure of sensing results

This solution proposes a new enabler (i.e., Sensing Enabler Server) to support the exposure of sensing results to the consumers (e.g., VAL server). The Sensing Enabler server may interact with other SEAL enabler to get the value-added sensing results.

Figure 6.4.1-1 illustrates the high-level procedure of exposure of sensing results.

Figure 6.4.1-1: Procedure of exposure of sensing results

1.	The VAL server sends a sensing service request to the Sensing Enabler server, including the service type (e.g., UAV detection, dynamic HD map, Vehicle tracking, etc.), service ID, sensing target object information (e.g., UAV ID, UE ID, Vehicle ID), sensing service requirements for the target object (e.g., the target area, sensing accuracy, sensing resolution, maximum sensing service latency, etc.), the reporting conditions (e.g., periodic reporting or event-triggered reporting), the time duration for the sensing, etc.

2.	The Sensing Enabler server checks whether the VAL server is authorized to invoke such request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the SM server may invoke the sensing related service request to the 3GPP CN based on the service request received in step 1 and obtain the sensing results from the 3GPP CN.

Editor's note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.	The Sensing Enabler server may interact with other SEAL enablers (e.g., SEALDD, ADAES) to get more sensing results (e.g., the transmission latency, the sensing accuracy analysis) utilizing the obtained sensing results from the 3GPP CN, in case the 3GPP core network will not provide.

-	The Sensing Enabler server may interact with ADAES to get the e.g., sensing accuracy analysis and the prediction analysis for the target object utilizing the obtained sensing results from the 3GPP CN, and the details can refer to the clause 6.4.1.1;

-	The Sensing Enabler server may interact with SEALDD as specified in clause 9.7 of 3GPP TS 23.433 [23433] to get the e.g., the transmission latency for the target object utilizing the obtained sensing results from the 3GPP CN.

5.	The Sensing Enabler may analyse and aggregate the sensing results obtained in step 3 and step 4 to generate the value-added sensing results (e.g. the statistic results in a location/time/direction/height granularity), and then expose to the VAL server if the results can fulfil the requested sensing service requirements.

6.	The Sensing Enabler sends the value-added sensing results to the VAL server via sensing service response message, including all of target objects and the value-added sensing information.

6.4.1.1	Procedure of supporting sensing analysis for ADAES

Figure 6.4.1.1-1 illustrates the high-level procedure when the Sensing enabler, acting as a consumer, asks for sensing accuracy analytics from ADAES.

Pre-conditions:

-	A-ADRF may have stored the historical sensing accuracy results for the requested sensing objects.

Figure 6.4.1.1-1: Procedure of supporting sensing analysis for ADAES

- The consumer (i.e. Sensing enabler) initiates a subscription request to ADAE server for sensing accuracy prediction/stats, including an analytics ID (e.g. "sensing accuracy prediction" or "sensing accuracy sustainability"), an analytics type (e.g. sensing accuracy statistics or prediction for a given location and/or for a given UE). The request may include also the target area, the service ID, the object ID, time validity, accuracy threshold, the obtained sensing results from the 3GPP CN, sensing service requirements (e.g. sensing accuracy, sensing resolution, maximum latency, missed detection probability, false alarm probability, etc.), etc.

- The ADAE server authorizes the service request based on e.g. the pre-configurations or the operator policies.

3.	 If the authorization is approved, the ADAE server sends a sensing accuracy analytics subscription response to the consumer.

4.	If the consumer (i.e. Sensing enabler) didn’t provide the obtained sensing results in step 1, the ADAE server may ask 3GPP CN to subscribe the sensing results or the sensing accuracy analytics based on the service requests in step 1 and obtains the relevant analytics notification.

Editor's note:	How to subscribe the sensing results and sensing accuracy analytics from 3GPP CN is FFS and depends on SA2’s progress.

5.	The ADAE server optionally requests sensing accuracy historical analytics/results from A-ADRF for the corresponding sensing objects or the target service area.

6.	Based on the request, the ADAE server receives the sensing accuracy historical analytics/results from A-ADRF for the corresponding sensing objects or target service area.

7.	The ADAE server utilizes the obtained sensing results from the consumer (i.e. Sensing enabler) and abstracts or correlates the analytics/results from steps 4-6 and then generates the analytics on the sensing accuracy for the requested sensing objects. Depending on the event ID in step 1, the ADAE server can indicate whether the sensing accuracy is sustainable or is predicted to be downgraded or upgraded.

8.	The ADAE server sends the sensing accuracy analytics notification to the consumer.

6.4.2	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 22.137, "Service requirements for Integrated Sensing and Communication; Stage 1".

[4] 		3GPP TS 23.255, "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows;".

[5]	3GPP TS 23.286, "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows; "

[6]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication (Release 19)".

[7]	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication (Release 20)".

[8]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors".

[23433]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications ".

* * * End of Change * * * *