3GPP SA WG6 Meeting #70	S6-255140

# Nov. 17 – 21st, 2025; Dallas, US.

Source: 	China Mobile

Title: 	Solution update of Sol #1: New architecture and functional model for AIoT services

Document for: 	Approval

Agenda Item: 	9.8

Work Item / Spec.:	FS_AmbientIoT_Ph2_APP / TR 23.700-26 v0.1.0

Abstract of the contribution: This document updates the sol#1.

## 1	Introduction

This pCR updates the Sol#1.

## 2	Reason for Change

Updates the Sol#1.

## 3	Proposal

It is proposed to agree and capture the update for the inclusion in the TR 23.700-26 v0.2.0.

# * * * * First change * * * *

## 7.1	Solution #1: New architecture and functional model for AIoT services

### 7.1.1	Solution description

This solution addresses the KI#1. The following procedures introduce the on-network and off-network functional model for AIoT services, also mention the functional entities and reference points in the new architecture. This solution aligns with the 5GS System architecture for AIoT services which is specified in the clause 4.2 of 3GPP TS 23.369 [3].

### 7.1.1.1	On-network Functional model for AIoT services

Figure 7.1.1.1-1 illustrates the generic on-network functional model for AIoT service.

Figure 7.1.1.1-1: On-network functional model for AIoT services

The AIoT management server, acting as an AF, could obtain AIoT data information from the 3GPP core network via NEF (over N33 reference point) which is defined in clause 4.5.4 of 3GPP TS 23.369 [3]. If the AIoT management server is a trusted AF, it may interact with AIOTF directly for AIoT related service as defined in clause 4.5.5 of 3GPP TS 23.369 [3]. The AIoT management server may communicate with other SEAL server(s) over AM-X reference point.

NOTE 1:	The Network interface is either the N33 via NEF (for an untrusted AF), or the interface off the AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF as defined in the 3GPP TS 23.369 [3].

Editor’s note: Whether and how the AIoT management server communicates with other SEAL servers is FFS.

The VAL server(s) could communicate with the AIoT management server over AM-S reference point for requesting the AIoT services and receiving the AIoT data information.

The AIoT management client, acting as a consumer, could also request the AIoT services to the AIoT management server over AM-UU reference point, and provide the support for AIoT related functions to the VAL client(s) over AM-C reference point.

NOTE: In this release, the AIoT management client is not an UE reader, but only acts as an AIoT service consumer.

The VAL client communicates with the VAL server over the VAL-UU reference point which is outside the scope of this document.

### 7.1.1.2	Functional entities description

AMS (AIoT Management Server):

The AMS performs the following functionalities to support AIoT services:

-	Interact with 3GPP core network via NEF to obtain the AIoT device data, acting as an untrusted AF;

-	Interact with AIOTF in 3GPP core network directly to obtain the AIoT device data, acting as a trusted AF;

-	Enrich the processing of AIoT service requests and the collected Ambient IoT related data;

-	Provide the value-added information of AIoT devices to the VAL servers or 3rd parties;

-	Provision the AIoT devices monitoring information;

-	Expose the AIoT device monitoring information to the the VAL servers or 3rd parties;

-	Acting as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8].

NOTE:	The functionalities for the AMS may update according to the approved solution and final conclusions.

AMC (AIoT Management Client):

The AMC performs the following functionalities to support AIoT services:

-	Acting as the application client for consuming the AIoT services;

NOTE:	The specific functionalities for the AMC may update according to the approved solutions and final conclusions.

### 7.1.1.3	Reference points description

AM-S

The interactions related to AIoT functions between the VAL server(s) and the AIoT management servers are supported by AM-S reference point.

AM-S reference point is used by the VAL server to request and receive the AIoT device related information from AIoT management server.

AM-S reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

AM-X

The interactions related to AIoT functions between the AIoT management servers and the other SEAL servers (e.g., LMS, ADAES) are supported by AM-X reference point.

AM-X reference point is used by the AIoT management server to request and receive the AIoT device related information.

AM-C:

The interactions related to AIoT functions between the VAL client(s) and the AIoT management clients within a VAL UE are supported by AM-C reference point.

AM-UU:

The interactions related to AIoT functions between the AIoT management clients and the AIoT management servers are supported by AM-UU reference point.

### 7.1.2	Architecture Impacts

This solution proposes a new architecture and functional mode for AIoT services.

### 7.1.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.1.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *