3GPP TSG-SA WG6 Meeting #70	S6-255145

Dallas, United States 17th – 21st November 2025

Source:	ETRI

Title:	New solution for KI#1 on sensing capability monitoring procedure

Spec:	3GPP TR 23.700-15

Agenda item:	9.13

Document for:	Approval

Contact:	Jong-Hwa Yi <jhyiee@etri.re.kr>

1. Introduction

This pCR proposes a solution for KI#1 on FS_Sensing_APP(Study on utilization of sensing results for vertical applications).

2. Reason for Change

From the VAL server’s perspective, it is necessary to investigate and define a new procedure that enables the continuous collection and evaluation of sensing capability information from multiple sensing servers, as well as the timely notification of any capability changes.

This procedure supports the VAL server in maintaining consistent service quality, selecting optimal sensing sources, and dynamically adapting its service behavior based on the current sensing performance across variousvertical application domains.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v 0.2.0.

* * * First Change * * * *

6	Solutions

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol #xx | x |  |  |  |

* * * First Change * * * *

6.x	Solution #1: Sensing capability monitoring procedure

6.x.1	Solution Description

This solution addresses KI#1 and proposes a new procedure to support sensing capability monitoring for vertical applications.

For some cases, if the VAL server can obtain sensing capability information of sensing server(s), it can be used to evaluate the feasibility of sensing services, to select the most suitable sensing servers, and to configure appropriate triggering conditions and QoS parameters according to the available sensing performance.

In the other hand, some Sensing servers may provide their sensing capability information directly, while others may only provide partial capability information. In such cases, the SEAL Sensing Enabler server may need to collect and analyze these pieces of information to determine the overall sensing capability. Furthermore, The SEAL Sensing Enabler server may notify the VAL server when the sensing capability information of the sensing server(s) changes, so that the VAL server can adapt its service behavior or reconfigure the triggering conditions accordingly.

6.x.2	Procedures

Figure 6.x.2-1: Sensing capability monitoring procedure

- The VAL server sends a Sensing capability monitoring Subscription request to the SEAL Sensing Enabler server. The request may include its sensing requirements (e.g., sensing area, sensing type, accuracy, latency), triggering conditions (e.g., whether sensing capability changes), inotification mode, subscription duration.

- The Sensing Enabler server checks whether the VAL server is suthorized to request the service.

- The SEAL Sensing Enabler server sends a Sensing capability monitoring Subscription response to the VAL server.

- The Sensing Enabler server performs a continuous monitoring process to collect initial and updated sensing capability information from the different sensing servers. This monitoring process continues until the subscription expires or an unsubscribe request is received from the VAL server.

(4a) For this, the Sensing Enabler server interacts with 3rd party Sensing servers (e.g., domain-specific sensing servers) to collect sensing capability information. Some sensing servers may provide complete sensing capability information (e.g., sensing type, accuracy, update rate), while others may provide only partial or limited information related to their sensing performance.

- After collecting these various pieces of capability information described in step 4, the Sensing Enabler server evaluates them and determines the optimal sensing capability that satisfies the VAL server’s requirements.

- The Sensing Enabler server sends a notification to the VAL server including the evaluated sensing capability determined in step 5.

- When the sensing capability of sensing server(s) changes (e.g., accuracy degradation, sensor unavailability, update rate modification), the Sensing server sends updated sensing capability information to the Sensing Enabler server.

- The Sensing Enabler server stores and evaluates updated sensing capability information, and determines the optimal sensing capability that satisfies the VAL server’s requirements.

- The SEAL Sensing Enabler server sends a notification indicating updated evalaued sensing capability to the VAL server.

- When the VAL server decides to stop the sensing capability monitoring service, it sends an unsubscribe request to the Sensing Enabler server.

- The Sensing Enabler server stops getting the sensing capability information and removes the subscription information.

- The Sensing Enabler server sends an unsubscribe response to the VAL server.

* * * End of Changes * * * *