3GPP TSG-SA WG6 Meeting #70	S6-255158

Dallas, United States 17th – 21st November 2025	(revision of S6-254308)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0680 | rev | 2 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC service UE remote management – re-enable procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, UKHO, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP TS 22.280 has requirements related to remote management of MC service UE and which are not fulfilled in stage 2. Below are the requirements [R-6.13.4-005] A temporarily disabled MCX UE, which has limited access capability per Mission Critical Organization policy, shall be able to be re-enabled by the MCX Service Administrator or an authorized MCX User. [R-6.13.4-006] The MCX Service shall provide a mechanism to re-enable a temporarily disabled MCX UE by the MCX Service Administrator or an authorized MCX User. Also paper S6-252045 discussed in details about the need for having a standardized solution to address the stage 1 requirements. This paper provides the solution for re-enabling the temporarily disabled the MC service UE by an authorized MC service user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information flows for the MC service UE enable procedure are added. Procedure for enabling the temporarily disabled MC service UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This is an important feature which has market need and also clear Stage 1 requirements. If not approved, stage 1 requirements are not met which leads to incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (All new) 10.x.2.3, 10.x.2.4,10.X.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.X.2.3	MC service UE re-enable request

Table 10.X.2.3-1 describes the information flow MC service UE re-enable request from MC service client to the MC service server and from MC service server to the target MC service client.

Table 10.X.2.3-1: MC service UE re-enable request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user requesting to enable the MC service UE. |
| MC service UE identifier | M | Unique identifier to identity the target MC service UE that needs to be enabled. |

#### 10.X.2.4	MC service UE re-enable response

Table 10.X.2.4-1 describes the information flow MC service UE re-enable response from MC service server to requesting MC service client and from the target MC service client to the MC service server.

Table 10.X.2.4-1: MC service UE re-enable response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user requested to enable the MC service UE. |
| MC service UE identifier | M | Unique identifier to identity the target MC service UE that needs to be enabled. |
| Result | M | Indicates success or failure of the MC service UE enable request. |

* * * Next Change * * * *

### 10.X.4	Re-enabling MC service UE

Figure 10.X.4-1 below illustrates the procedure of re-enabling the MC service UE by authorized MC service user or MC service administrator which was previously disabled.

Figure 10.X.4-1: Enabling MC service UE by the authorized MC service user

1.	MC service administrator or authorized MC service user sends the MC service UE re-enable request to the MC service server. This request carries the MC service ID of the user requesting to enable the MC service UE and the unique identifier of the MC service UE to be enabled.

2.	On receiving the request to re-enable the MC service UE, the MC service server shall check if the requesting MC service user is authorized to enable the MC service UE and check if the MC service UE is in disabled state.

3.	MC service server shall update the stored information to consider the MC service UE identified by the identifier provided in step 1 as enabled.

4.	MC service server sends MC service UE re-enable request to the target MC service client to indicate that MC service UE is enabled to access MC services.

5.	Target MC service client sends MC service UE re-enable response to the MC service server to acknowledge the receipt of  MC service UE re-enable request.

6.	MC service server sends the MC service UE re-enable response to the requestor containing the status of the request whether it is success or failure.

7.	Target MC service client initiates the MC user service authorization procedure as specified in clause 5.1.1. in 3GPP TS 33.180[25] .