3GPP TSG-SA WG6 Meeting #70	S6- 255191

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Samsung

Title:	Pseudo-CR on Guaranteed QoE for uninterrupted video streaming service

Spec:	3GPP TR 23.947 V0.2.0

Agenda item:	9.14

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR introduces the addition of use case related to Guaranteed QoE for uninterrupted video streaming service.

2. Reason for Change

This use case depicts how the SEAL services can be leveraged to provide Guaranteed QoE for uninterrupted video streaming service.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947 V0.2.0.

* * * First Change * * * *

## 5.x	Guaranteed QoE for uninterrupted video streaming service

### 5.x.1	Introduction

This section describes a use case related to providing improved QoE when using video streaming service. The use case has been selected to illustrate how application developers can leverage the services offered by SEAL framework to offer uninterrupted streaming service when users are on the move and encountering poor network coverage en route.

### 5.x.2	Use case Description

“VQoE” company is developing a video streaming application that leverages the services offered by SEAL framework to offer premium video streaming service even in the poor network condition. The poor network condition could be due to congestion or due to poor signal strength. The application consists of client side application (VAL client) running on a user Equipment (UE) which is integrated with SEAL clients and server application (VAL server) running on the cloud infrastructure. This use case specifically demonstrate the usage of Location management service and Network resource management service offered by the SEAL framework.

Through the Location Management service, the VAL server monitors the UE location and predicts the UE mobility and QoS drop points based on the historical data. When the UE is nearing the QoS drop points, the VAL server through the NRM server requests the core network to increase the GBR facilitating the video streaming application to download more data and keep it in buffer for playing and thereby by avoiding the service interruption.

### 5.x.3	Use case Realisation over Application Enablement Frameworks

Editor’s note: Realisation of this usecase over application enablement frameworks is FFS.

### 5.x.4	Use case Flows

Editor’s note: Flows depicting the usecase realisation is FFS.

* * * End Change * * * *