3GPP TSG-SA WG6 Meeting #70	S6-255208

Dallas, USA, 17th – 21st November 2025	revision of S6-254727

Source:	Ericsson, InterDigital

Title:	New Solution for KI#2 on Energy Monitoring and Exposing in Application Enablement Layer

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #2.

2. Reason for Change

A solution to address the key issue #2 on monitoring and exposure of energy consumption information within the application enablement layer is proposed in this paper.

3. Conclusions

A solution for energy monitoring is proposed to address the key issue #2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol #x |  | X |  |  |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: Monitoring and exposing energy consumption within the application enablement layer

### 6.x.1	Solution Description

#### 6.x.1.0	General

According to key issue #2, there is a need for a service that can monitor and expose the energy information/status (e.g. energy source type change, anomaly energy consumption, etc.). This solution provides energy saving monitoring service, that can be used by the consumer (e.g., VAL server, SEAL client (Energy Saving), SEAL server) to receive notifications regarding energy consumption information/status, which in turn can be useful for energy savings.

#### 6.x.1.1	Energy monitoring procedure

Figure 6.x.1.1-1 illustrates the procedure for a consumer (e.g., VAL server, SEAL client (Energy Saving), SEAL server) to monitor energy information/status.

Figure 6.x.1.1-1: Energy monitoring subscription and notification

1.	The consumer (e.g., VAL server, SEAL client (Energy Saving), other SEAL server) sends an Energy Monitoring Subscription request to the SEAL Server (Energy Saving) with an energy monitoring profile that may contain energy monitoring and reporting requirements of the consumer. The subscription request includes information as defined in Table 6.x.1.2.1-1.

Editor’s Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrates energy saving functionality with an existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

2.	The SEAL server (Energy Saving) checks if the consumer is authorized for the energy monitoring subscription.

3.	The SEAL server (Energy Saving) obtains the energy monitoring and reporting requirements from the energy monitoring profile provided by the consumer in the subscription request. Based on the obtained energy monitoring and reporting requirements, the SEAL server (Energy Saving) determines the data source(s) (e.g., EIF, SEAL server/client) to monitor, and applies for subscription for energy information with the selected data source(s).

4.	The SEAL server (Energy Saving) sends an Energy Monitoring Subscription response to the consumer to indicate the subscription status. The subscription response includes information as defined in Table 6.x.1.2.2-1.

5	The SEAL server (Energy Saving) receives notifications about energy information from the subscribed data source(s).

6.	The SEAL server (Energy Saving) determines whether to send a notification to the consumer based on the consumer energy monitoring and reporting requirements. The reporting can be periodic, or event triggered.

7.	The SEAL server (Energy Saving) sends an Energy Monitoring notification to the consumer by including the subscription identifier and the aggregated energy information for the list of VAL UEs. The notification includes information as defined in Table 6.x.1.2.3-1.

The consumer may adjust the network requirements (e.g., QoS requirements) to the VAL client for energy saving, based on the energy monitoring data provided by the SEAL server (Energy Saving).

#### 6.x.1.2	Energy monitoring Information elements

##### 6.x.1.2.1	Energy monitoring subscription request

Table 6.x.1.2.1-1 shows request sent by the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) to the SEAL server (Energy Saving) for the energy monitoring subscription.

Table 6.x.1.2.1-1: Energy monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (e.g. VAL server, SEAL client (Energy Saving), SEAL server). |
| Target VAL UE ID(s) | M | List of VAL UE(s) for which the energy monitoring applies. |
| Target VAL service ID | O | The identifier of the VAL service for which the energy monitoring applies. |
| Target Slice ID | O | The slice ID (S-NSSAI) for which the energy monitoring applies. |
| Notification endpoint | M | Indicates the address (e.g., IP address, FQDN, URI) where the notifications should be sent. |
| Energy Monitoring profile | M | Represents the energy monitoring profile. Indicates the energy monitoring requirements of the consumer. |
| Reporting requirements | O | The requirements for energy consumption data reporting. Requirements may include, e.g., the type and frequency of reporting (periodic or event triggered), the reporting frequency in case of periodic, and reporting thresholds in case of event triggered. |
| > Reporting periodicity | O | It describes the Frequency of reporting is periodic. |
| > Reporting Event Triggered | O | It describes the event triggered reporting. |
| >> Reporting energy threshold event | O (NOTE) | Indicates the reporting threshold with the corresponding threshold reaching direction (e.g., corresponding energy source type exceeds the limit, slices accumulated energy consumption excess, slice average energy consumption excess, average energy consumption (e.g., per hour) excess, maximum renewable energy excess, slice renewable energy ratio excess). |
| >> Reporting energy anomaly event | O (NOTE) | Indicates the anomaly events for energy consumption or usage (e.g., spike in energy consumptions/usage, high-consumption for a longer interval, black-out). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: Only one of the IEs shall be present. |  |  |

Editor’s Note:	Whether Data Source, Energy Consumption Category or other IEs to be contained in the Energy Monitoring profile is FFS.

##### 6.x.1.2.2	Energy monitoring subscription response

Table 6.x.1.2.2-1 shows response sent by the SEAL server (Energy Saving) to the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) for the energy monitoring subscription procedure.

Table 6.x.1.2.2-1: Energy monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |
| Subscription ID | O | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |

##### 6.x.1.2.3	Energy monitoring notification

Table 6.x.1.2.3-1 shows the notification sent by the SEAL server (Energy Saving) to the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) for the energy monitoring procedure.

Table 6.x.1.2.3-1: Energy monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |
| Slice ID | O | The network slice whose energy monitoring data is provided. |
| Energy monitoring result | M | Energy monitoring result is an aggregation of energy consumption related data collected from the data source. |
| Triggering event | O (NOTE) | The event that triggered the monitoring notification. |
| NOTE: This information element applies to event triggered reporting. |  |  |

### 6.x.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.x.3	Solution evaluation

This solution addresses Key Issue #2 and introduces new application enabler service to support monitoring and exposure of energy consumption information.

This solution is feasible and doesn't introduce any dependency on 3GPP network systems.

* * * End of Changes * * * *