3GPP TSG-SA WG6 Meeting #70	S6-255237

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Lenovo

Title:	Solution on energy-aware split learning operation update

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This contribution provides a new solution to KI 4.

2. Reason for Change

This contribution provides a new solution to KI 4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.x	Solution #x: Support Energy-aware split operation update

### 6.x.1	Solution Description

This solution addresses Key Issue #4.

A Split AI/ML operation pipeline is a workflow for ML model inference in which AI/ML endpoints are organized and collaborate to process ML models in sequential stages, where processing at each stage involves ML model inference on the output of the previous stage. The AIMLE capability for supporting a split AI/ML operation update has been specified in 3GPP TS 23.482 clause 8.14.2.6.

In this solution, the reason for split AI/ML operation pipeline update is the energy criteria/conditions, and particularly the energy credit or budget per VAL UE (or per application or aggregately for the entire pipeline) which is reaching a threshold. Hence, the AI/ML client detects that due to high energy consumption, it needs to modify the pipeline to offload the ongoing split ML operation to a target AI/ML member which can be an edge AIMLE server or another AIMLE client.

Figure 6.x.1-1 illustrates the procedure for an AIMLE client to update an instance of a split operation pipeline at the AIMLE server. The AIMLE client or the VAL client determines that some of the nodes in the split operation pipeline may not be able to perform the required operation (due to VAL UE energy credit/budget starvation/limitation) and decides to modify the pipeline either to add or remove the nodes.

Pre-conditions:

- The AIMLE client has received information related to split operation pipeline profile as specified in clause 8.14.2.3.

- VAL client is aware of the energy credit/budget for the VAL UE.

- AIMLE server has provided in advance to AIMLE clients configuration for reporting trigger events related to energy status changes.

Figure 6.x.1-1: Split operation pipeline update due to energy criteria

- AIMLE client detects an energy trigger event based on step 1 and optionally monitoring the energy status at the VAL UE. This can be done by monitoring the energy data/analytics (e.g. as in Solution 7) per VAL client / AIMLE client, and application traffic schedule or pattern from the VAL application.

Such detection can be also triggered by VAL client providing in advance a VAL UE energy credit/budget report to AIMLE client (via AIMLE-C API) to indicate the current status of the energy credit (remaining) and/or the maximum credit limit for the UE / application.

- The AIMLE client sends an energy-aware split pipeline update trigger message to the AIMLE server. The trigger includes a status update and in particular:

  - An energy credit limit/budget per VAL UE is expected to reach, or

  - high predicted or actual or expected energy usage or credit/budget for VAL UE/application, or

  - high Energy Consumption indicator (based on a pre-defined threshold.

- Upon receiving the trigger message from the AIMLE client, the AIMLE server identifies the split operation pipeline(s) for which the VAL UE is applicable and determines whether the split operation pipeline needs to be updated based on the split operation pipeline identifier and the energy status trigger.

If the split operation pipeline profile is determined, the AIMLE server updates the split operation pipeline profile and notifies the appropriate processing nodes indicated in the request about their inclusion or exclusion in the split operation pipeline as described in clause 8.14.2.5 of TS 23.482. The AIMLE server also notifies the involved nodes about modification of the existing pipeline as described in clause 8.14.2.5 of TS 23.482.

Then, the AIMLE server determines to check alternative entities (AIMLE clients or AIMLE servers) to undertake the task and discovers the entities (AIMLE clients or another AIMLE server using existing discovery capability as described in of TS 23.482.

At the discovery of alternative entities, the energy capability (energy credit limit, energy usage information) of the list of alternative candidate AIMLE clients or AIMLE servers is also fetched by the ML repository.

- Supplementary (for VAL UEs) to the discovery in step 4, the AIMLE server requests ESE Server to provide the energy credit/usage status for the list of alternative VAL UEs with the respective AIMLE clients.

- The AIMLE server receives the energy status (or credit/budget status) information for the list of alternative VAL UEs.

In this step, the AIMLE server can alternatively fetch the energy credit information from a charging function in the operator’s charging domain.

- For the list of AIMLE servers (deployed at the edge DN), AIMLE server may also consume the DN energy analytics service from ADAES (using the corresponding analytics service as in TS 23.436). Additionally, for the AIMLE clients the AIMLE server may also collect analytics based on Solution 7.

- AIMLE server evaluates (by rating or ranking) the alternative candidate entities (AIMLE clients or AIMLE servers) for undertaking the task using the performance for the split operation as well as the energy sustainability factor (if ADAE analytics from Solution 7 are used).

AIMLE server determines one or more entities to be selected or considered as alternative to the AIMLE client of interest for becoming part of the updated split operation pipeline.

- The AIMLE server sends a split operation pipeline update notification message to the AIMLE client. If the AIMLE server has updated an instance of a split operation pipeline profile, the notification includes the corresponding split operation profile.

The notification also includes the list of the alternative entities and their energy rating / credit (as part of the split operation profile or as separate IEs including the entity ID/address (VAL UE ID or AIMLE client ID or AIMLE server ID) the rating/ranking information per entity, the time and area for validity for the availability of the candidate entities and optionally whether this is based on prediction/analytics.

- AIMLE server sends to the ML repository the updated list of AIMLE clients and servers which will be part of the updated pipeline and may also provide the energy status/credit to ML repository to store this information for future updates.

### 6.x.2	Architecture impacts

This clause will provide the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *