3GPP TSG-SA WG6 Meeting #70	S6-255272

Dallas, USA 17th – 21th November 2025	(revision of S6-254xxx)

Source:	Ericsson

Title:	Enhancements to XR data transmission for data burst support

Spec:	3GPP TR 23.700-51 V1.1.0

Agenda item:	9.5

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

This paper provides solution for Key issue #1: Transmission enhancement in application layer

2. Reason for Change

The XR traffic is dynamic in nature like XR application generating large number of packets in short period(burstiness in XR traffic) and also could be periodic burst. 5GS has specified capabilities to use the data burst. The proposed pCR provides solution to KI#1 to use the 5GS capabilities to enhance the XR transmission.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V1.1.0.

* * * First Change * * * *

##### 9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal ata transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4. 7-8. Same as step 8-9 of clause 9.2.2.2. NOTE : If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). Based on the data burst size and time to next burst indication received in the request message in step 1, the SEALDD server performs packetization of the RTP packet to include databurst indication in RTP header extension as defined in 3GPP TS 26.522 [TS26522]), and also configures the 5GS to handle the data burst for the XR traffic as per clause 5.2.6.9.2 TS 23.502[TS23502]. The SEALDD client removes the RTP header extension from the SEALDD-UU traffic and sends the payload to the VAL client. #### 9.12.3.1 SEALDD enabled multi-modal transmission request Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service. Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of specific UE or VAL user. List of requested flows; M; List of the requested multi-modal flows. >SEALDD-S Data transmission connection information for multi-modal XR; M; Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow). >QoS information; O; QoS information provided by VAL server. >Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), RTP header extension for data burst size and time to next burst indication, packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP..

* * * End of Changes * * * *