3GPP TSG-SA WG6 Meeting #70	S6-255296

Dallas, USA 17th – 21st November 2025	(revision of S6-255xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for KI#2 on Sensing UAVs with UE module

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

This pCR provides a solution for KI#2 for TR 23.700-15.

2. Reason for Change

KI#2 proposes to utilize sensing results to assist in detecting UAV, in tracking UAV, in avoiding the collision when there is any object along the UAV trajectory or in triggering warnings when then UAV is entering a No Drone Zone. This assistance is especially relevant when the UAV is without the UE module. or when having the UE module, the 3GPP UE based signalling and connectivity is not available.

In this solution, it is proposed a mechanism where a UAV with a UE module can be identified as a Sensed object, and hence, using the sensing results to assist in continuing tracking the UAV, avoiding the collision with other UAVs or notifying about UAV entering in a No Drone Zone, in those circumstances in which the UAV network location and 3GPP connectivity might not be available, e.g. because NAS signalling is inhibited.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 (v0.2.0)

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol #X |  | X |  |  |

* * * Next Change * * * *

## 6.X	Solution #Y: Sensing UAVs combining location and sensing results

### 6.x.1	Solution description

This solution addresses the KI#2.

In this solution, it is proposed a mechanism where a UAV with a UE module can be identified as a Sensed object, and hence, using the sensing results to assist in continuing tracking the UAV, avoiding the collision with other UAVs or notifying about UAV entering in a No Drone Zone, in those circumstances in which the UAV network location and 3GPP connectivity might not be available, e.g. because NAS signalling is inhibited.
The solution is based on the correlation of the positioning of the UAV with UE module obtained with location services and the positioning of the objects sensed by the sensing network.

#### 6.x.1.1	Procedure for sensing UAVs with a UE module

Figure 6.x.1.1-1 illustrates the high-level procedure for sensing UAVs with a UE module.

Pre-condition:

-	The UAV with the UE module is registered and connected in the CN.

Figure 6.x.1.1-1: Procedure for sensing UAVs with a UE module

1.	The VAL server sends a Sensing request to the Sensing enabler, including the request to detect and identify a UAV with a UE module. The VAL server includes the UE module Id (GPSI), i.e., the UAV UE ID, the sheduled UE trajectory or target area around the UE, and the sensing time duration. If the VAL server wants to receive in step 5 the matching between the provided UAV UE ID (e.g. GPSI) and a sensed object, the VAL server also includes (requested result indication) that the result needs to include the matching between the UAV UE ID and sensed Object description (e.g., position, velocity). The VAL server may include other sensing request parameters related e.g. with the sensing of objects in the proximity of the UAV UE ID.

2.	The sensing enabler invokes the Location service and obtains the network location for the UE module Id (GPSI) of the UAV.

3.	The sensing enabler invokes the sensing related service request to the 3GPP CN and obtains the sensing results from the 3GPP CN.

Editor's note:	The exposure of sensing results from 3GPP CN depends on SA2’s progress. What will be exposed to the consumers is FFS.

4.	The Sensing enabler calculates the distances between the returned sensed objects and the network location(s) (e.g., the Sensing enabler utilizes samples of the sensing results (object(s) positions) and the network location of the UAV UE ID and calculates for each sample the object with the minimum distance between the object position and the network location), and determines with a certain probability, which is the object that corresponds to the UAV UE ID. When the probability that a sensed object is the UAV UE ID is above a certain accuracy level, the Sensing enabler concludes that a sensed object is the UAV with UE module included in step 1.

5.	The Sensing enabler sends to the VAL server the sensing result, including the description of the matched object (e.g., position, velocity) with the UAV UE ID, the accuracy of the matching and the corresponding UAV UE ID.

For subsequent sensing results, when the network location of the UAV UE ID is available, the Sensing Enabler uses the reported UE location to increase accuracy of the sensing poistioning results related to the correlated object. When network location of the UAV UE ID is not available, the Sensing Enabler uses the sensing positioning results in the report. In both cases, the Sensing Enabler indicates to the VAL server whether network location is contributing to the sensing report of the object bound to the UAV UE ID.

### 6.x.2	Architecture Impacts

This solution proposes a new sensing enabler to support the sensing of UAV with UE module. The new architecture for the sensing enabler is described in the solution for KI#1.

### 6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

Editor’s Note:	The evaluation of the solution is FFS.

* * * End of Change * * * *