3GPP TSG-SA WG6 Meeting #70	S6-255298

Dallas, USA, 17th – 21st November 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#2 on Converged Trajectory Tracking

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

As described in Key Issue #2 (TR 23.700-15 clause 4.2), the study aspects on Enhance UAV service utilizing sensing results include:

- How to enhance the UAS application enabler to support enhanced trajectory tracking for UAVs utilizing the sensing results.

Currently, there are two means to for UAE server to track a drone through LM server:

- The drone may periodically report its location (e.g., GPS location) to LM server

- LM server uses network location service to get the locations of the drone

However, the drone may be hacked or malfunctioned so that it stops reporting locations to LM server/UAE, or the drone may be malfunctioned so that the network location service cannot get the location. Sensing can be used as a complementary mean to track the drone’s trajectory.

Since sensing service does not know the UE ID, how to correlate the sensed object to the UE ID is a challenge. The UAE server can compare trajectories generated by the location service and the sensing service, and correlate a sensed drone object with a drone UE ID based on the overlap between the two trajectories. Thus, this pCR proposes a new SEAL Sensing Server service on request sensing for mapping with UAV locations to provide object ID of the target UAV.

2. Reason for Change

To address the study aspects in Key Issue #2, this pCR proposes a new SEAL Sensing Server service on request sensing for mapping with UAV locations to provide object ID of the target UAV.

3. Conclusions

This paper proposes solution on new SEAL Sensing Server service on request sensing for mapping with UAV locations to provide object ID of the target UAV.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol X |  | x |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: Converged Trajectory Tracking

### 6.X.1	Solution Description

#### 6.X.1.1	General

Currently, there are two means to for UAE server to track a drone through LM server:

- The drone may periodically report its location (e.g. GPS location) to LM server

- LM server uses network location service to get the locations of the drone

However, the drone may be hacked or malfunctioned so that it stops reporting locations to LM server/UAE, or the drone may be malfunctioned so that the network location service cannot get the location. Sensing can be used as a complementary mean to track the drone’s trajectory. Since sensing service does not know the UE ID, how to correlate the sensed object to the UE ID is a challenge.

The following clauses specify SEAL Sensing Server service on request sensing for mapping with UAV locations to provide object ID of the target UAV.

#### 6.X.1.2	Procedure

Pre-condition:

- UAE Server requests to LM Server for UAV location reporting.

- The LM Server requests the LM Client (deployed on a UAV) to report location.

- The 5GCN provides the sensed objects.

Figure 6.X.1.1-1: Converged trajectory tracking

- The LM Client (deployed on a UAV) reports the UAV location to LM Server. The LM server provides the UAV location information to the UAE Server. The UAE Server records the trajectory of the UAV.

- The UAE Server sends sensing requests for mapping with UAV locations to the SEAL Sensing Server.

- The SEAL Sensing Server determines the dynamic sensing area based on the UAV locations.

- The SEAL Sensing Server interacts with 5GC NF (NEF) for sensing result.

- After get sensing result from 5GC NF (NEF), the SEAL Sensing Server compares the trajectories and characteristics (e.g. position and velocity) of sensed objects and the UAV trajectory, maps the object with the target UAV.

- The SEAL Sensing Server sends sensing result periodically to the UAE Server with the object of the target UAV.

- The UAE Server sets up/updates the correlation between the object and UAV ID, based on the received information from step 5 and additional information from LMS and 5GC NF (GMLC).

- The UAE Server sends location information requests periodically by object or UAV ID, to various location information sources (e.g. LM Server, SEAL Sensing Server, 5GC NFs (via NEF)), for location information of the UAV.

- The UAE Server aggregates the location information received from the location information sources (e.g. LM Server, SEAL Sensing Server, 5GC NFs (via NEF)) to generate UAV trajectory.

- The UAE Server records the trajectory of the UAV.

#### 6.X.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***