3GPP TSG-SA6 Meeting #69	S6-254421

# Wuhan, China 13th – 17th October 2025               			       		  (revision of S6-254022)

Source:	at&t

Title:	Solution: Recording off-network communications

Spec:	3GPP TR 23.700-39 V0.3.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jerry Shih (jerry.shih@att.com)

1. Introduction

This contribution proposes a solution to key issue for “recording off-network communications.”

2. Reason for Change

No solution exists for key issue for “recording off-network communications.”

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.3.0.

* * * First Change * * * *

## 6.Y	Solution Y: Recording off-network communications

### 6.Y.1	Description

The off-network communications refer to the UE to UE and UE to group communications without the support from the core network. In this case recording can only be done with the MC service client in the UE involved in the off-network communication. Once the UE is connected back to the core network, the MC service client will upload the recorded off-network communications to the recording server as part of overall recording for that specific target, either a MC user or an MC group.

#### 6.Y.1.1	Functional model and reference points

This solution proposes the following enhancements to the functional model and the REC-1 reference point in TS 23.280.

7.3.1.2	Functional model for an MC system

Figure 7.3.1.2-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1.2-1: Functional model for application plane for an MC system

#### 7.5.2.29	Reference point REC-1 (between the replay client and the recording server also between the MC service client and the recording server)

The REC-1 reference point, which exists between replay client and recording server, is used by the replay client to request and receive logs/recordings of metadata and media from the replay server.

The REC-1 reference point, which exists between the MC service client and recording server, is used to upload the off-network communication recording to the mass storage through the recording server.

The requests from the replay client and the MC service client to the recording server shall be validated with the MC access token mechanism, which is specified in TS 33.180 [25]. All other details of the REC-1 reference point are outside the scope of the present document.

#### 6.Y.1.2	Configurations

Clarifications to the existing user profile configuration are needed.

#### 6.Y.1.3	Procedures

Figure 6.Y.1.3-1 illustrates the off-network communication is recorded and stored to the recording server.

Pre-condition:

- MC service client 1 is off network

- MC service client 1 is a target whose communications need to be recorded.

Figure 6.Y.1.3-1: Off-network communication recording

- MC service clients 1 and 2 are in an ongoing off-network communication and MC service client 1 is recording the communication.

- When the off-network communication ends, MC service client 1 stores the recorded communication in local storage.

- MC service client 1 is connected to the network.

- MC service client 1 uploads the recorded off-network communication(s) to recording server. If upload is successful, the MC service client 1 removes the recorded off-network communication(s) from its local storage.

### 6.Y.2	Impacts on existing functional entities and reference points

See clause 6.Y.1.1.

### 6.Y.3	Solution evaluation

TBD.

* * * End of changes * * * *