3GPP TSG-SA WG6 Meeting #70	S6-255674

Dallas, United States 17th – 21st November 2025	(revision of S6-255588)

Source:	Huawei, Hisilicon

Title:	PCR to 3GPP TR 23.949 for NRM introduction

Spec:	3GPP TR 23.949 v0.1.0

Agenda item:	10.7

Document for:	Approval

Contact:	Yang Yanmei yangyanmei@huawei.com

1. Introduction

This contribution is to propose introduction content of NRM service in TR 23.949 v0.1.0.

2. Reason for Change

The same as the above.

3. Proposal

Agree the following PCR into TR 23.949 v0.1.0.

*******************************Begin of the change********************

## 4.x	SEAL Entities

Editor's Note:	This clause will describe the service entities (SEAL servers and SEAL Clients) in SEAL layer.

4.2.x NRM (network resource management) server/client

NRM server provides the value-added services by utilizing capabilities of 3GPP core network (e.g., NEF/PCF/MBSMF). It enables third party‘s application (application server or application client) to request 3GPP system to provide QoS service to improve the performance of applications traffic(s) in an easy-to-use way.

NOTE:	The detailed information about NRM advantages to application developers is described in clause 5.x.

NRM client supports two types of functional role:

1)  Provide assitant information or resource required by NRM server, to enable certain NRM service (e.g., providing MBMS listening status report required by NRM server to determine the data transmission mode), or

2) Trigger (acting asNRM service consumer) NRM server to excute NRM service logic, e.g., request NRM to manage End-to-End QoS service for traffic between application on one UE to application on another UE.

*******************************End of the changes********************