3GPP TSG-SA WG6 Meeting #70	S6-255689

Dallas, United States 17th – 21st November 2025	(revision of S6-255600, S6-255115)

Source:	Telefónica, Apple, Fogus, UMA, Lenovo

Title:	TR 23.947 Example Application Flows: CAPIF + SEAL

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

Add CAPIF+SEAL Application Flow clause within the overall Example Application Flows clause.

2. Reason for Change

Complete clause 6.3.2 and add other headings.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.2.0.

* * * First Change * * * *

## 6.3	Example Application Flows

The following flows will be considered “CAPIF + EDGEAPP”, “CAPIF + SEAL” and “CAPIF + EDGEAPP + SEAL”

### 6.3.1 CAPIF+EDGEAPP Application Flow

### 6.3.2 CAPIF+SEAL Application Flow

#### 6.3.2.1 General

This clause describes the logical flow of interactions between the SEAL Server, VAL Server, and VAL Client when CAPIF is used to enable secure API exposure, discovery, and invocation.

#### 6.3.2.2 SEAL Server Onboarding and Registration

The onboarding procedure allows the SEAL Server to become a trusted entity within the CAPIF ecosystem. By performing this process, the SEAL Server obtains its official API invoker identity.

By the Registration process, the SEAL server obtains its official provider identity, operator and the authorization to later publish its SEAL Enabler APIs (e.g., Group, Configuration, or Location).

These procedures establish the initial trust relationship between the SEAL Server and the CCF, ensuring that subsequent CAPIF operations occur over authenticated and authorized interfaces.

#### 6.3.2.3 SEAL Server Publish APIs

#### 6.3.2.4 VAL Server Onboarding

#### 6.3.2.5 VAL Server Discover

#### 6.3.2.6 VAL Server Security Context

#### 6.3.2.6 VAL Server requesting token

#### 6.3.2.7 VAL Client Requesting VAL Server Service

### 6.3.3 CAPIF+EDGEAPP+SEAL Application Flow

* * * End of Changes * * * *