3GPP TSG-SA WG6 Meeting #69	S6-254389

Wuhan, China 13th – 17th October 2025	(revision of S6-254271)

Source:	Nokia

Title:	Pseudo-CR on Solution#6 evaluation

Spec:	3GPP TR 23.700-83 (v1.0.0)

Agenda item:	9.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@nokia.com)

1. Introduction

This pCR proposes solution evaluation for solution#6.

2. Reason for Change

It is required to provide solution evaluation as per the TR template.

The solution is related to KI#2 and solve open issue 3. In the solution, each AIMLE client evaluates the performance of the received aggregated global model on the local data based on the received Aggregated AI/ML model parameters.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 (v1.0.0).

* * * First Change * * * *

#### 7.6.1.1	Enhancements to ML model performance monitoring

The ML model performance monitoring procedure as specified in clause 8.22 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

### 8.22.2 Procedure Pre-conditions: 1. One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing. Figure 8.22.2-1: ML model performance monitoring 1. The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc. The request also includes the ML model evaluation profile ID which is associated with a ML model evaluation profile containing the baseline validation dataset information which is the baseline reference for evaluation of HFL model after each round of parameter aggregation. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server identifies the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients. The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3. The ML Evaluation profile ID is used to obtain the ML model evaluation profile (e.g., baseline validation dataset) which is used to determine the ML model performance. Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service. In order to get the information, the request is enhanced with following IEs: Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Aggregated AI/ML model parameters; M; Information about the AI/ML model and model parameters for use in HFL training.. Performance Metrics; M; The performance metrics that are used to evaluate the performance of model (e.g. accuracy, latency). ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. Each AIMLE client use the ML evaluation profile ID to obtain the ML model evaluation profile which is used to determine the ML model performance. Each AIMLE client evaluates the performance of the received aggregated global model on the local data based on the received Aggregated AI/ML model parameters, performance metrics and ML model evaluation profile. Each AIMLE client sends the performance report of aggregated global model on the local data as the response message to AILMLE Server. The performance report response message includes information as described in the following table. The response is enhanced with following IEs: Information element; Status; Description. AIMLE Client ID; M; The identifier of the AIMLE Client.. AIMLE service ID; M; The service identifier for the AIMLE HFL training operation.. Performance records; M; The evaluation performance records (e.g. on accuracy, on latency) of received aggregated global model from AIMLE Server on local data.. ML model evaluation profile ID; M; The identifier of the ML model evaluation profile used to perform the performance evaluation.. Upon receiving the performance report from the AIMLE client, the AIMLE server evaluates the parameters based on baseline validation dataset. The AIMLE server further uses the baseline validation dataset to evaluate the global aggregated model also. 5. The AIMLE server detects an expected ML model degradation (e.g., model drift, data drift) based on the deviation of the performance of the AIMLE service as indicated in step 4 . 6. The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement. Further, AIMLE Server sends a configuration request to increase training collaboration to only AIMLE Clients that have degraded performance records. Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model. NOTE: If this action involves re-selecting an AIMLE client for the AIMLE service, this is based on the procedure defined in clause 8.9. 7. The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service. 8.22.3 Information flows 8.22.3.1 ML model performance monitoring subscription request Table 8.22.3.1-1 shows the request sent by a VAL server to an AIMLE server for the ML model performance monitoring. Table 8.22.3.1-1: ML model performance monitoring subscription request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. ML model identifier; M; The identifier of the ML model for which the monitoring applies.. Notification endpoint; M; The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to.. AIML operation information; O; The AIMLE operation (ML model training, HFL, VFL, TL) for which the ML model is used.. > VAL service ID; O; The VAL service identifier of the AIMLE service using the ML model (if known by the requestor).. > AIMLE client ID(s); O; The identifier(s) of the AIMLE client(s) training the ML model (if known by the requestor).. > AIMLE service KPI; O; One or more KPIs for the AIMLE service performance (latency, accuracy, etc).. ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. Monitoring report configuration; M; The reporting configuration for the monitoring service (thresholds for triggering a monitoring event, e.g. minimum accuracy, delay, whether the reporting is one time or periodical or event-triggered).. Area of interest; O; The geographical or service area for which the monitoring applies.. Time validity; O; The time validity for the monitoring subscription.. Trigger Action requirement; O; This requirement identifies policies for triggering an action based on a monitoring event (e.g. if degradation is detected, to train a new model or re-selecting AIMLE clients)..

* * * Next Change * * * *

### 7.6.2	Architecture Impacts

This solution has not identified any architectural impacts.

### 7.6.3	Corresponding APIs

The inputs for the existing API is enhanced as specified in the solution description.

### 7.6.4	Solution evaluation

The solution relates to KI#2 and provides solution for following open issue:

3.	Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

The solution enhances Clause 8.22 and clause 8.12 of 3GPP TS 23.482 [2] for the performance monitoring and its adaptation for HFL training. The solution provides AIMLE enablement layer, the capability of proactively monitoring and reporting the performance of collaborating AIMLE clients with HFL training. The solution has not identified any architecture impacts.

* * * End of Change * * * *