3GPP TSG-SA WG6 Meeting #69	S6-254363

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	KPN N.V

Title:	Update to Solution # 22 Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

Spec:	3GPP TR 23.700-83 1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

This contribution solves editors notes and completes the procedure for solution # 22 sample alignment enablement for VAL servers in vertical federated learning.

2. Reason for Change

Solution # 22 introduced sample alignment enablement for VAL servers in vertical federated learning feature to TR 23.700-83. This contirubtion completes the solution by addressing the editors note.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V1.0.0.

* * * First Change * * * *

## 7.22	Solution #22: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

### 7.22.1	Solution Description

#### 7.22.1.1	General

This solution addresses key issue # 7 which identifies VFL sample alignment among open issues. In a VFL process, a VAL server may act as a VFL server (active party), coordinating training and inference by aggregating results from other VAL servers acting as VFL clients (passive parties). In this context, the sample space consists of user equipment (UEs), each represented by local identifiers specific to the VAL servers.

The VFL process begins by first aligning the samples among VFL participants so that inference and training are performed properly. This is performed without sharing the UE identifiers with other VFL participants. Each UE in the sample space may be identified with different identifications in each of the VFL participants such as different AF specific UE IDs or different public IP address of the UE. Therefore, the process of VFL sample alignment should be able to align the UEs that are involved in a VFL process without sharing the UE identifiers while resolving the potential difference in how the VFL participants identify the UEs.

This solution proposes to enable the AIMLE server to support sample alignment among VFL participants while preserving the privacy of the UEs and while resolving the potential differences in how each VFL participant identifies the UEs.

#### 7.22.1.2	Procedures

The following are described to support VFL inference and training among VAL Servers with sample alignment:

- 	VFL clients registration

-	VFL clients discovery

- 	VFL training and inference

##### 7.22.1.2.1	VFL clients registration

FL member registration is specified in 3GPP TS 23.482 clause 8.4. The FL members can be VAL servers which perform the registration via VAL servers. To support VFL registration the procedure need to be further updated. For this purpose, the following changes are needed in 3GPP TS 23.482 clause 8.4.2 in the corresponding steps.

- For FL regiseteration request, when VAL servers acting as VFL clients register as candidate FL members, they provide additional metadata including supported feature sets, and local UE identifiers in their sample space.

- The AIMLE server before storing the VFL client registration, it first maps each local UE identifier in the sample space to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions and store it along with its registration. This unified UE identifier, however, shall not be accessible to the VAL servers and only used by the AIMLE server for discovery purposes.

- There is no change needed in the FL member registration response.

##### 7.22.1.2.2	VFL clients discovery

Pre-conditions:

- The VAL servers acting as VFL clients and VAL server acting as VFL server have registered with the AIMLE server, providing metadata including supported feature sets, and local UE identifiers.

Figure 7.22.1.2.3-1: VFL clients discovery

1.	The VFL server sends VFL clients discovery request including the desired number of VFL clients, and coorsponding requirements including feature set, and list of local identifiers of the UEs in the sample space.

2.	The AIMLE server discovers VFL clients from the registerd VFL clients which fulfil the requirements such as required feature set and sample space. The UEs in the sample space of the VFL clients and VFL server is matched by using the Core Network functions to resolve local identifiers (e.g. AF specific UE ID, IP addresses) into a common UE identifiers (e.g, GPSI). The VFL clients are identified using the common UE identifiers for matching the sample space.

3.	The AIMLE server sends VFL clients discovery response including the discovered VFL clients.

##### 7.22.1.2.3	VFL training and inference

Assumptions:

- VAL Server acting as VFL server coordinates other VAL servers acting as VFL clients to perform VFL training/inference.

- The sample space is associated with UEs which are locally identified by AF Specific UE IDs, IP addresses, or similar identifiers.

Pre-conditions:

- The VAL servers acting as VFL clients and VAL server acting as VFL server have registered with the AIMLE server as per clause 7.22.1.2.1, providing metadata including supported feature sets, and local UE identifiers.

- The AIMLE server maps each local UE identifier to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions as in clause 7.22.1.2.1.

Figure 7.22.1.2.3-1: VFL training/inference with sample alignment

- The VFL server sends VFL training/inference request including the desired VFL clients or number of VFL clients, feature set, and UEs in the sample space.

- The AIMLE server discovers and selects the desired VFL clients from registered VFL clients fulfilling the required feature set and sample space. The UEs in the sample space of the VFL clients and VFL server is matched by using the Core Network functions to resolve local identifiers (e.g. AF specific UE ID, IP addresses) into a common UE identifiers (e.g, GPSI). The VFL clients are identified using the common UE identifiers for matching the sample space.

- The AIMLE server assigns a sample correlation ID for each UE in the sample space for consistent referencing during training and inference while enabling VFL training/inference using their local identifiers for the UEs.

- The AIMLE server sends VFL training/inference initiation request to the selected VAL servers acting as VFL clients, including the assigned sample correlation IDs to match the correlation IDs in a VFL training/inference task to their corresponding local identifiers of the UEs.

- The VAL servers acting as VFL clients send the initiation response to the AIMLE Server confirming participation to the VFL training/inference.

- The AIME server sends VFL training/inference response to the requesting VFL server, including the sample correlation IDs corresponding to the aligned UEs.

- The VFL server and the VFL clients proceed with the training/inference using the aligned sample space, with sample correlation IDs ensuring consistent sample alignment across particpants.

### 7.22.2	Architecture Impacts

There is no architectural impact from this solution.

### 7.22.3 	Corresponding APIs

Editor's Note: The corresponding APIs are FFS.

### 7.22.4 	Solution Evaluation

Editor's Note: The evaluation of the solution is FFS.

* * * End of Change * * * *