3GPP TSG-SA WG6 Meeting #69	S6-254647

Wuhan, China, 13th – 17th October 2025	(revision of S6-254167)

Source:	China Mobile

Title:	Overall evaluation and conclusion for KI#4

Spec:	3GPP TR 23.700-51

Agenda item:	9.5

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This paper proposes to add overall evaluation and conclusion for KI#4.

2. Reason for Change

Add overall evaluation and conclusion for KI#4.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51.

* * * First Change * * * *

7.3.x	Key issue #4 solutions evaluation

The open issues studied in the Key Issue #4 are as follows:

-	How to enhance to support for SEALDD connection management among tethering device and multiple tethered devices, e,g, based on PINAPP solution, device identifier mechanism.

-	How could the application enabling layer support for QoS differentiation in complex tethered device scenarios, such as when multiple tethered devices are connected to the same 3GPP device.

-	How could the application enabling layer support for policy control, such as policy alignment of Multi-modal flows of multiple tethered devices behind the same tethering device.

Solution #4 addresses the key issue 4.

Solution #4 resolves open issue 1) and open issue 2). This solution enables SEALDD server to identify SEALDD-UU flow of different tethered devices and tethering device based on PIN information during multi-modal data transmission establishment and quality guarantee.

Editor's note: The conclusion and overall evaluation of Key Issue #4 may be updated accordingly if there are other solutions added.

* * * Next Change * * * *

10.3	Conclusions of key issue #4

Solution #4 can be adopted into normative work with the following principles:

-	Solution #4 can be used to multi-modal data transmission establishment and quality guarante procedure. SEALDD server can identify SEALDD-UU flow of different tethered devices and tethering device based on PIN information.

* * * End Change * * * *