3GPP TSG-SA WG6 Meeting #70	S6-255598

Dallas, USA, 17th – 21th November 2025	(revision of S6-255199)

Source:	Ericsson

Title:	Update to Solution #2 on Evaluation

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes the inclusion of updates to solution #2 by adding enhancements on use data drift detection to assist ML model performance monitoring, solution evaluation of the solution #2 on enhance AIMLE data management assistance for data drift detection. Furthermore, the content for corresponding APIs and the architecture impacts sections are added.

2. Reason for Change

The solution evaluation, corresponding APIs, and the architecture impacts clauses of the solution #2 are missing.

3. Conclusions

This paper proposes to update soltuion #2, add content for the solution evaluation, the architectural impacts, and the corresponding APIs clauses of solution #2 on enhance AIMLE data management assistance for data drift detection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

### 7.2.1	Solution Description

As introduced in 3GPP TS 23.482 [2], the AIMLE data management assistance service is the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. In the context of AI/ML, it is important that the data used by the ML model is not significantly different from the data used to train the model (e.g. the distribution of the features have changed over time). If the difference is higher enough, the model's performance tends to decrease. This discrepancy is known as data drift, that is data drift refers to changes in the statistical properties of data over time leading to a decline in model performance.

Therefore, it is essential that the data management system procedure provides the capability to detect whether data drift has occurred to assist in decision-making if the model needs to be updated due to a decrease in its performance.

Data drift detection procedure allows to assist ML model maintenance procedures. To support ML model management (e.g. assist ML model maintenance, assist ML model performance monitoring), the AIMLE data management assistance service need be enhanced by adding drift detection functionality.

ML model evaluation profile ID is provided as part of data drift requirements to allow the AIMLE server with obtaining ML model evaluation information for performing data drift detection.

The following clauses specify the impact to existing AIMLE procedures for the enhancements of AIMLE data management assistance service.

* * * Next Change * * * *

#### 7.2.1.2	Impact to existing ML model performance monitoring procedures

The procedures in 3GPP TS 23.482 [10] on ML model performance monitoring can be enhanced (highlighted in bold italics) as follows.

## 8.22 ML model performance monitoring ### 8.22.1 General The following clauses specify procedures, information flows, and APIs for ML model performance monitoring and potential degradation detection (e.g. data drift detection can be used to support ML model performance monitoring and potential degradation detection). ### 8.22.2 Procedure Pre-conditions: 1. One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing. Figure 8.22.2-1: ML model performance monitoring 1. The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server identifies the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients. The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3. Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients on performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service. 4a. The AIMLE server sends Client data processing trigger requests to AIMLE clients, for data drift detection (as step 4 of procedure in clause 7.2.1.1). This step includes receiving data drift detection results from one or more AIMLE clients for the target ML model (based on steps 5-6 of procedure in clause 7.2.1.1). 5. The AIMLE server detects an expected ML model degradation (e.g., model drift, data drift) based on the deviation of the performance of the AIMLE service and/or the data drift detection results as indicated in step 4. 6. The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement. Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model. NOTE: If this action involves re-selecting an AIMLE client for the AIMLE service, this is based on the procedure defined in clause 8.9. 7. The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service.

* * * Next Change * * * *

### 7.2.2	Architecture Impacts

There were no identified architectural impacts in this solution.

### 7.2.3	Corresponding APIs

API updates have been described in clause 7.2.1.1.

### 7.2.4	Solution evaluation

This solution addresses Key Issue #2: support enhancements for evaluation and determination of ML model performance and correctness within the AIMLE Enablement Layer.

The proposed solution offers data drift detection functionality to the AIMLE service consumer based on the data management operations configuration. Data drift detection is used to determine changes in statistical properties of the input data. Furthermore, it can be used as one of the factors to be considered for detecting ML performance degradation to assist ML model maintenance and ML model performance monitoring.

This solution doesn't introduce any dependency to 3GPP network systems.

* * * End of Change * * * *