3GPP TSG-SA WG6 Meeting #70	S6-255412

Dallas, United States 17th – 21st November 2025	(revision of S6-255220)

Source:	Lenovo

Title:	Pseudo-CR on overall evaluation and conclusion of the Key Issue #3

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This pCR proposes the overall evaluation and conclusion of the Key Issue #3 of the solutions included on TR 23.700-83 v1.1.0.

2. Reason for Change

To include the evaluations and conclusion of the solutions that addresses the Key Issue #3 of TR 23.700-83 v1.1.0.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 10.4	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios

The open issues studied in the Key Issue #3 are as follows:

1.	Identify the use cases for supporting multiple PLMN/NPN, and across MNOs and CSPs/ECSPs scenarios

2.	Whether and how AIMLE services can support VAL UEs that roam into visited PLMNs or NPNs, e.g. registration, discovery, and access of AIMLE in the visited domain, as well as go back to home domain.

3.	Whether and how to ensure service continuity and switching for inference tasks or AI service sessions during UE mobility across MNOs and CSPs/ECSPs.

4.	Whether and how UEs or VAL functions can discover the most suitable AIMLE instance in edge-specific scenarios while UEs move across network boundaries or service areas, considering latency, capability, and service policy.

5.	Whether and how the AIMLE server and/or ML repository are aware of the serving PLMN/network information of the UE associated with the AIML client.

6.	Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes.

There is a total of six solutions that address key issue #3: solutions #7, #10, #11, #16, #18, and #19. Each of these solutions covers different factors of the Key Issue.

Solution #7 covers the deployment of one AIMLE server supporting VAL UEs connected to different PLMNs, and provides an enhancement of the AIMLE client discovery capability to account for AIMLE clients connected to different PLMNs. In additon, Solution #19 provides an enhancement to AIMLE client selection capability for a different deployment where each AIMLE server is deployed per PLMN (hence interactions covering two AIMLE servers of different providers are expected). These solutions are not conflicting since they target different AIMLE server deployments in cross-PLMN scenario.

Solution #10 and #16 support the AIMLE service continuity or migration for UEs while roaming. Solution #10  supports enhancements to AIMLE client registration procedure  to enable the AIMLE server to identify whether the AIMLE client is allowed to use AIMLE services while in a visited network. In addition, Solution #16 provides a mechanism for discovering and selecting a T-AIMLE server for continuing the AIMLE service at the V-PLMN. These solutions are complementary and target different aspects of UE roaming.

Solution #11 supports enhancements in FL member registration capability to allow ML repository being aware of the MNO information for potentially allowing AIMLE servers/clients or VAL servers being considered as candidate FL members for a given FL operation. Such solution is only for scenarios where there might be implication supporting FL members connected (or have service agreements) with different 3GPP network systems.

Solution #18 describes a new capability in AIMLE server to support Federated AI/ML services.

Regarding the mapping of the above solutions to open issues, the solution doesn’t provide alternative, but complementary capabilities, in particular:

- Solution #7 addresses Open Issue 1, 4, 5

- Solution #10 addresses Open Issue 2, 5

- Solution #11 addresses Open Issue 3, 4

- Solution #16 addresses Open Issue 1, 2, 4, 5

- Solution #18 addresses Open Issue 1, 4, 5

- Solution #19 addresses Open Issue 1, 4. 5

In each solution that addresses Key Issue #3, there is AIMLE architectural impact since this requires the introduction of architecture enhancements for multi-operator, UE roaming and support for Federated AIMLE services.

* * * Next Change * * * *

## 11.2	Conclusions of key issue #3

Following solution considerations will be considered for the normative work for KI #3:

- Solution #7 can be considered for normative work to support cross-PLMN AIMLE client discovery, whereas AIMLE server is deployed at ECSP/CSP with agreements to multiple MNOs.

ΝΟΤΕ 1: Whether and how alternative capabilities can be used to determine the VAL UE roaming status will be discussed in the normative phase.

- Solution #10 can be considered for normative work to support AIMLE client registration enhancements for UE roaming scenarios.

- Solution #11 can be considered for normative work to support FL member registration information storage enhancements with MNO information, for enabling the consideration of candidate FL members for a given FL task with restrictions/preferences on the PLMN to be associated with, in cross-PLMN AIMLE scenarios.

ΝΟΤΕ 2: This solution 	doesn’t consider any communication aspects of enabling FL operation over multiple PLMNs and is only applicable to registering information to the ML repository which is initiated from the candidate FL members themselves.

- Solution #16 can be considered for normative work to support AIMLE service continuity for UE roaming scenarios.

- Solution #18 can be considered for normative work to support federated AI/ML services.

- Solution #19 can be considered for normative work to support cross-PLMN AIMLE client discovery assuming AIMLE servers deployed per PLMN.

* * * End of Changes * * * *