3GPP TSG-SA WG6 Meeting #70	S6-255509

Dallas, USA, 17th – 21st November 2025	(revision of S6-255270)

Source:	Ericsson

Title:	New Solution for KI#6 on Optimized Location Reporting based on Energy Saving

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #6.

2. Reason for Change

Key Issue #6 defines potential enhancements to the application enablement layer (e.g. SEALDD, AIMLE, LM, ADAE) to support energy saving for the network resources used by applications. SA6 defines services which may be enhanced to support energy saving capabilities in order to satisfy the request from consumer (e.g. VAL server).

The solution can be used in the use case scenario, for example, where a person with different gadgets such as mobile phone, laptop, smart watch, virtual reality glasses etc., need to send each of their location separately. Here, with the proposed solution, these devices within the same location can delegate the location reporting task to the selected location reporter device based on the energy information of the devices.

A solution is proposed in th is pCR to address the key issue #6 on optimized location reporting when multiple VAL UEs share the same location, based on energy saving.

3. Conclusions

A solution for optimized location reporting when multiple VAL UEs share the same location based on energy saving is proposed to address the key issue #2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol #x |  |  |  |  |  | X |

* * * Next of Change * * * *

## 6.x	Solution #x: Optimized Location Reporting based on Energy Saving

### 6.x.1	Solution Description

#### 6.x.1.1	General

The existing location reporting optimization for different devices sharing the same location is not based on the energy saving criteria. Consider the example scenario where a user with different devices sharing the same location needs to report the location. In this case, a device can be selected as the location reporting device based on the energy information of the different devices and their location range proximity. This will help to save energy saving in location reporting. This solution provides dynamic role switching for the location reporter using the information collected from the SEAL Server (Energy Saving). This enables the location management server to reselect the device for the reporter role based on the information from the energy saving enabler server.

#### 6.x.1.2	Procedure for optimized location reporting based on energy saving

Pre-conditions:

1.	The LMS has the information that all UEs share the same location.

2.	The UEs can be aware of each other's location.

Figure 6.x.1.2-1: Optimized location reporting based on energy saving

1.	The Location Management Server (LMS) sends track energy information subscription request to SEAL Server (Energy Saving) for energy information of the VAL UEs.

2.	The SEAL Server (Energy Saving) collects energy information (e.g. energy consumption for location reporting over period of time) for the VAL UEs (via SEAL client (Energy Saving)) based on the request from the LMS.

NOTE:	The procedures for energy information collection need to take user consent into account.

Editor’s Note:	How SEAL Server (Energy Saving) collects the energy consumption information for location reporting from the VAL UE is FFS.

3.	The SEAL Server (Energy Saving) sends the collected energy information of each VAL UE to the LMS.

4.	Based on the energy information of VAL UE collected from the SEAL Server (Energy Saving), the LMS select VAL UE with less energy consumption for location reporting as a location reporter.

5.	LMS informs the selected VAL UE to report on the location for the other VAL UEs in the same location.

6.	The location reporter VAL UE informs other VAL UEs in the same location via application layer or PC5 communication links, that it enables other VAL UEs to stop reporting location to save energy.

7.	The location reporter VAL UE starts to report on location for the VAL UEs in the same location, the VAL UE list which shares the same location may be included in the report.

8.	The SEAL Server (Energy Saving) sends a new notification for the collected energy information of each VAL UE to the LMS.

9.	LMS detects the change at the location reporter VAL UE for e.g., energy change or location change and re-selects the location reporter VAL UE for location reporting based on energy information and the location information of the VAL UEs.

10.	LMS server informs the new location reporter VAL UE to report on the location of other VAL UEs in the same location, and terminates the previous location reporter VAL UE to stop reporting location.

Editor’s Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

#### 6.x.1.2	Information Flows

##### 6.x.1.2.1	Inform Location reporter VAL UE

Table 6.x.1.2.1-1: Inform Location Reporter VAL UE

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | Indicates the requestor’s identity (i.e., identifier of LMS). |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Identity of VAL UE(s) | M | Indicates the identity of other VAL UE(s) whose location is reported by the reporter UE. |
| Location reporter | M | Indicates the selection of VAL UE to be the location reporter. |

##### 6.x.1.2.2	Terminate Location reporter VAL UE

Table 6.x.1.2.2-1: Terminate Location reporter VAL UE

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | Indicates the requestor’s identity (i.e., identifier of LMS). |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Stop location report | M | Indicates to stop the location reporting and change in the location reporter role. |
| >Reason | O | Indicates the reason for stopping location reporting (e.g., energy change, location change). |

### 6.x.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.x.3	Solution evaluation

Editor’s Note:	The solution evaluation of the solution is FFS.

* * * End of Changes * * * *