3GPP TSG-SA WG6 Meeting #70	    S6-255518

Dallas , US, 17th – 21st November 2025                                        (revision of S6-255098)

Source:	China Telecom

Title:	New solution for KI#1 on AIML model information storage and discovery on satellite

Spec:	3GPP TR 23.700-002 v0.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	Zhou Zhe <zhouz11@chinatelecom.cn>

Xu Xia < xiaxu@chinatelecom.cn >

1. Introduction

Application enabled layer could enhance AIML service by deploying models on satellites. It is necessary to study how to deploy AIMLE on satellite and how to storage AIML models on satellite for AIMLE service. Thus, TR 23.700-02 v0.1.0 identifies the following  key issue to be addressed:

Key issue #1: AIML model storage and deployed on satellite

This paper proposes a new solution on KI#1, to store and discovery ML model information in the EDN on satellite. The information of satellite stored AIML models is stored in the ML repository on satellite. When the disaster like an erathquake or a fire happens, the ML repository on ground might get broken or just could not connect to the AIMLE server successfully. Then as an alternative option, the AIMLE server can ask for the model information from the ML repository on satellite instead of the ML repository on ground.

2. Reason for Change

To support AIML model information storage and discovery on satellite.

3. Conclusions

This paper proposes solution on AIML model storage and deployment on satellite.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-002 v0.1.0.

* * * First Change * * * *

## 5.X	Solution#X: AIML model information storage and discovery on satellite

### 5.X.1	Solution Description

This solution proposes to address the following open issues in KI#1

-	Investigate different deployment options for AIMLAPP when AIML models are deployed on board satellite. Investigate different deployment options for AIMLAPP when AIML models are deployed on satellite and study whether/how to deploy AIMLE on satelllite (e.g. ML repository/ADAES on satellite to perform data processing/interference based on collected remote data).

-	Whether/how could AIML model discovery and storage be optimized.Whether/how to storage AIML models for AIMLE (e.g. Models storaged on remote sensing satellite to process collected remote data, Model transmission and sharing between satellites and terrestrial network).

#### 5.X.1.1 General

Figure 5.X.1.1-1: satellite AIMLE deployment

In this deployment option, ​​the AIMLE server is deployed on the ground with an ML repository. The ML repository onboard the satellite performs as a VAL server in the satellite EDN containing multiple AIML models. The satellite EDN can provide the information of AIML models located in satellite EDN to AIMLE server through AIML-R interface point.

In the proposed scenario, some accidents like earthquake or fire happen and the affected ML repository is down, causing that the AIMLE server couldn’t reach the ML repository. As a result, the AIMLE server has to retrieve in the EDN instead of the ML repository on the ground to discover the proper AIML model. As EDN can be deployed on a satellite and the GEO satellite provides a large coverage, the GEO satellite EDN can store models and the AIMLE server can retrieve the proper AIML model in the GEO satellite EDN.

#### 5.X.1.2	Satellite ML model information storage

Figure 5.Y.1.2-1 illustrates the procedure of satellite ML model information storage.

Pre-conditions:

1.	The AIMLE server has either received application specific model details from AIML consumer or produced analytics model.

Figure 5.Y.1.2-1: AIMLE server-initiated ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. When the AIMLE server is in the coverage of a satellite ML repository, the AIMLE server also sends an ML model information storage request to the satellite ML repository to store an ML model. The ML model included in the request is one trained by the ML repository consumer or its information is provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1. The request message may contain Indication of continuous training and continuous training model parameter.

NOTE:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by ADAE Analytics ID and/or list of the allowed vendors provided within the ML model information attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

5.X.1.3	Satellite ML model information discovery

Figure 5.Y.1.2-1 illustrates the procedure of satellite ML model information discovery.

Pre-conditions:

1.	The AIML models are pre-configured in the satellite EDN. The AIMLE server can retrieve proper models from the satellite EDN.

Figure 5.Y.1.2-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. If the AIMLE server loses connection with the ML repository on ground, then it would request the ML repository on the satellite to discovery ML model information. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, ADAE Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model.

NOTE:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details are indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enablement consumer can rely on model repository to get the model details.

After the AIMLE server receives the response from the ML repository, the AIMLE server determines whether the received ML model satisfy the ML model requirement of the AIMLE server. If not, the AIMLE server needs to continuously train the model to satisfy the ML model requirement based on the AIMLE server capability of supporting continuous training and ML model profile (Indication of continuous model training, continuous training model parameter).

### 5.Y.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 5.Y.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 5.Y.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

* * * End of Changes * * * *