3GPP TSG-SA WG6 Meeting #70	S6-255621

Dallas, United States 17th – 21st November 2025	(revision of S6-255319)

Source:	Lenovo

Title:	Pseudo-CR on overall evaluation and conclusion of the Key Issue #1

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This pCR proposes the overall evaluation and conclusion of the Key Issue #1 of the solutions included on TR 23.700-83 v1.1.0.

2. Reason for Change

To include the evaluations and conclusion of the solutions that addresses the Key Issue #1 of TR 23.700-83 v1.1.0.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 10.2	Key issue #1: Key issue on support for ML model inference

The open issues studied in the Key Issue #1 are as follows:

1.	Identify deployment models for supporting ML model inference in enablement layer.

2.	Whether and how ADAES needs be enhanced or leverage AIMLE for supporting ML model inference for ADAE analytics.

3.	Whether and how AIMLE needs to be enhanced to enable ML model inference to address VAL request (e.g. for VAL automation task or analytics).

4.	Whether and how to enhance AIMLE to support model inference in edge scenarios.

5.	Whether and how to enhance AIMLE architecture and functions to support collaborative inference between AIMLE client and edge AIMLE server.

There is a total of two solutions that address key issue #1: solutions #1 and #17. Each of these solutions covers different factors of the Key Issue.

Solution #1 provides AIMLE support for ML model inference for a VAL request.

Solution #17 enables UE-edge collaborative inference, for the case where a VAL UE (with an AIMLE client) and a nearby edge AIMLE server jointly execute an ML model inference for a VAL request. Both solutions can be seen as complementary since Solution #1 is generic for supporting ML inference while Solution #17 covers the collaborative use case. At the same time, collaborative ML model inference has some commonalities with the concept of AIML operation splitting using pipelines; hence the Solution #17 provides an alternative (or complementary) solution to address the specific scenarios of UE to edge collaboration for jointly performing inference services.

Open issue 2 is not directly mapped to any of the solutions, however the architecture assumptions and analysis as in clause 4 cover that ADAE supports ML model inference for ADAE analytics; hence no solution is enhancing the assumptions.

Regarding the mapping of the above solutions to open issues, the solution doesn’t provide alternative, but complementary capabilities, in particular:

- Solution #1 addresses Open Issue 3

- Solution #17 addresses Open Issue 1, 4, 5

In each solution that addresses Key Issue #1, there is no AIMLE architectural impact.

* * * Next Change * * * *

## 11.2	Conclusions of key issue #1

Following solution considerations will be considered for the normative work for KI #1:

Editor’s Note: Solution #1 and Solution #17 wiill be concluded upon completing the respective solution evaluations.

* * * End of Changes * * * *