# 3GPP TSG-SA WG6 Meeting #69	S6-254623

# Wuhan, China, 13th – 17th Oct 2025	(revision of S6-254175)

Source:	ZTE Corporation, KPN

Title:	Solution for KI#1 on support for sensing results exposure

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Wei Luo <luo.wei11@zte.com.cn>

1. Introduction

This paper proposes a solution to KI#1 on support for sensing results exposure procedure.

2. Reason for Change

This paper proposes a solution to KI# 1 on support for sensing results exposure procedure. This solution addresses the key issue on how to acquire the sensing results from the appropriate sensing service supplier and expose the (enhanced) sensing results to the sensing consumer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

# 6.x	Solution #x: Support the sensing results exposure based on the sensing subscription request

6.x.1	Solution description

This solution aims to address the issues identified in Key Issue 1 and provides a possible procedure to perform the sensing results exposure.

Editor's note: This solution is built on functional architecture described in solution X for key issue 1. The sensing capabilities in the enabler layer may be provided by a separate SEAL sensing server or directly by the specific SEAL/VAL enabler server. The decision on whether a separate SEAL sensing server is needed will be based on the evaluation of the solutions and FFS.

In this solution, after acquiring the sensing requirements from the sensing service consumer, a Sensing enabler server will send sensing request to a 5GC NF(e.g.,NEF). After it receives the sensing results from the 5GC NF, it determines whether the sensing requirements (e.g. the required accuracy of sensing result) are met, if not, it may further select other sensing service supplier(s) to acquire more sensing results, then it may generate enhanced sensing results based on the received sensing results from the 5GC NF and other sensing service supplier(s) and expose them to the sensing service consumer.

In this solution, the sensing service supplier which can provide sensing results can be another 5GC NF or vertical application server.

Editor's note:	Whether the sensing service supplier can be 3rd party AF is FFS.

6.x.2	Procedures

The high-level procedure of sensing service exposure is shown in Figure 6.x.2-1.

Pre-conditions:

- The Sensing enabler server has acquired sensing capability information of one or more Sensing Service Supplier(s).

Figure 6.x.2-1: Sensing service exposure procedure

- The VAL Server/Client acting as sensing consumer performs sensing service subscription procedure by sending sensing service subscription request and receiving sensing service subscription response. During the sensing service subscription procedure, the Sensing enabler server can acquire the sensing requirements of the VAL Server/Client such as Sensing Service Area of interest, Sensing target object type, Accuracy requirements, QoS requirements and so on.

- Sensing enabler server sends sensing request to 5GC NF(e.g.,NEF) and receives sensing response.

Editor's note:	The parameters in the sensing request and response are FFS and should be determined based on SA2’s progress.

- Sensing enabler server receives sensing results from 5GC NF(e.g.,NEF) and determine whether the sensing requirements are met.

- If the sensing requirements are met,  skip to step 8, if not, the Sensing enabler server selects other Sensing Service Supplier(s) according to the sensing capability information of Sensing Service Supplier(s)  and sensing requirement as received in step1 (e.g. by matching sensing capability information of Sensing Service Supplier to the sensing subscription parameters received from the sensing consumer).

- The Sensing enabler server sends sensing request to each selected Sensing Service Supplier and receives sensing response. Different Sensing Service Supplier may be given different sensing request parameters.

Editor's note:	The parameters in the sensing request and response are FFS.

- Each Sensing Service Supplier sends sensing results to the Sensing enabler server based on the sensing request  parameters respectively.

- The Sensing enabler server further generate enhanced sensing results(e.g.,with higher accuracy) based on the received sensing results from the 5GC NF and other Sensing Service Supplier(s).

- The Sensing enabler server sends the (enhanced) sensing results to the VAL Server/Client.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *