3GPP TSG-SA WG6 Meeting #69	S6-254681

Wuhan, China 13th – 17th October 2025	(Revision of S6-254050)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on <Solution for Key issue#2: sensing based tracking dynamic UAVs>

Spec:	3GPP TR 23.700-15 V0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Lei Chen(chenlei79@huawei.com)

1. Introduction

This contribution proposes a solution of sensing based tracking dynamic UAVs for key issue#2.

2. Reason for Change

In TR 23.700-15, there are the following open issues for Key issue #2: Enhance UAV service utilizing sensing results:

-	How to enhance the UAS application enabler to support enhanced trajectory tracking for UAVs utilizing the sensing results.

-	How to enhance the UAS application enabler to support enhanced DAA service/functionalities for UAVs utilizing the sensing results.

This pCR is to handle the first open issue.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15.

* * * First Change * * * *

6.x	Solution #x: sensing based tracking dynamic UAVs

6.x.1	Solution description

This solution intends to solve the first open issue of key issue #2. This solution is to enhance tracking dynamice UAV service by utilizing sensing results.

6.x.2	Procedures

This feature utilizes the following procedures:

-	Step 1: UAS Application Specific Server or the host UAV subscription for host UAV’s dynamic information with UAE server.

-	Step 2: UAE server tracking host UAV’s UE location with support from SEAL’s location management server.

-	Step 3: UAE server management of dynamic area for sensing

-	Step 4: UAE server obtaining dynamic information from sensing results

-	Step 5: UAE server notification of host UAV’s dynamic information to the UAS Application Specific Server and/or to the host UAV.

Note: Step 1 and Step 2 are same to the exsiting procedures defined in clause 7.8.2.1 in 3GPP TS 23.255[4]. Step 5 is enhanced to inlucde UVA dynamic information obtained by sensing results based on the procedure in 7.8.2.4 in 3GPP TS 23.255[4].

6.x.2.1	Subscription for host UAV dynamic information

This procedure intends to obtain the host UAV dynamice information, the procedure in clause 7.8.2.1 in 3GPP TS 23.255[4] is reused.

6.x.2.2	 UAE server management of dynamice area for sensing

Figure 6.x.2.2-1 describes the procedure for management of dynamic area for sensing.

Pre-condition:

-	UAE server 1 has received an updated location of the host UAV as per procedure specified in 3GPP TS 23.434 [5].

Figure 6.x.2.2-1: management of dynamic area for sensing

1. Dynamice area for sensing creation/update is triggered (e.g. notified of the UE location of host UAV or notified a location update) via the step 4 in clause 7.8.2.1 in 3GPP TS 23.255 [4] for the UAV ID of the host UAV.

2. UAE server 1 determines the dynamice area for sensing based on the location of host UAV and the proximity range defined by application by its implementation.

6.x.2.3 	UAE server obtaining dynamic information from sensing results

Figure 6.x.2.3-1 describes the procedure of obtaining dynamice information from sensing results.

Pre-condition:

-	UAE server 1 is configured with NEF information of their supported region of operation.

Figure 6.x.2.3-1: Obtaining dynamic information from sensing results

1. The UAE server 1 determines the NEF(s) operating in the dynamice area for sensing.

2. The UAE server 1 request and obtain sensing results from 5GC.

3. The UAE server 1 obtain dynamic information of UAVs by considering all sensing results from 5GC.

Editor Note: the detail of how to request and obtain sensing results from 5GC depends on SA2

6.x.2.4 	Notification of host UAV dynamic information

Pre-conditions:

-	UAS Application Specific Server has performed subscription as per procedure in clause 7.8.2.1 in 3GPP TS 23.255[4] with UAE server 1.

-	UAE server 1 has prepared the host UAV dynamic information as per procedure in clause 7.8.2.3.3 in 3GPP TS 23.255[4].

Figure 6.x.2.4: Notification for host UAV dynamic information

1.	The UAE server 1 sends notification of host UAV dynamic information to the subscribed entity (i.e. UAS Application Specific Server and/or to the subscribed UAE client of the host UAV). The UAE server can aggregate the dynamic information obtained by sensing and other methos. The notification includes the aggregated information of all the objects in the application defined proximity range of the host UAV and the location of the host UAV.

2.	The UAS Application Specific Server or the UAE client of the host UAV updates the host UAV dynamic information with the host UAV dynamic information received in step 1. The UAE client provides the host UAV dynamic information to the UAS Client.

Editor note: How to define objects in the notification message is FFS. It may be associated to an UAV identity or not.