3GPP TSG-SA WG6 Meeting #69	S6-254716

Wuhan, China 13th – 17th October 2025	(revision of S6-254396, S6-254272)

Source:	Nokia

Title:	Pseudo-CR on Solution#15 evaluation and EN removal

Spec:	3GPP TR 23.700-83 (v1.0.0)

Agenda item:	9.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@nokia.com)

1. Introduction

This pCR provides solution evaluation and resolves the EN.

2. Reason for Change

The solution evaluation is required as per the TR skeleton. The solution is related to KI#7: VFL sample alignment across VAL servers. The solution resolved opein issue 1 “Whether and how to enhance VFL operation considering also VAL servers and AIMLE servers as VFL participants”. The solution proposes to find the same training samples (sample alignment problem) among training participants that have different features of it, in a way that the privacy of the participants and their data is preserved during the training process.

Further, Editor’s note has been removed with following explanation:

- - Consider following examples:

  - you have some sample IDs and features on UE1,

    - street 1,  123456, features: number of vehicles

    - street 2,  123412, features: number of pedestrians

    - ...

    - street 10, 453254, features: traffic incident report

  - you have some sample IDs and features on UE2,

    - street 1,  123456, features: traffic incident report

    - street 2,  123412, features: number of pedestrians

    - ...

    - street 10, 453254, features: number of vehicles

  - If our interested features for the AIML tasks are: traffic incident and number of vehicles.

    - As you can see,

      - For street-1 – UE-1 has number of vehicles, but UE-2 has accident report.

      - For street-10 – UE-1 has accident report, but UE-2 has number of vehicles.

    - So, we need to bind such samples (both stree-1 and street-10) on UE-1 and UE-2 both. So that together they can be used for AIML task.

  - Here, Sample pool is available data for training with each UE, while beinding criteria is the information which is required to bind samples.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 (v1.0.0).

* * * First Change * * * *

## 5.7	Key Issue #7: VFL sample alignment across VAL servers

### 5.7.1	Description

In the Vertical Federated Learning (VFL) process, the active party or coordinating server collects partial results from other participants (passive parties) to obtain a global model. VFL participants can be VAL servers, AIMLE servers, and/or AIMLE clients. VFL participants share the same sample space but have different feature sets. Therefore, to achieve VFL process, these different VFL participants require sample alignment to participate in a VFL operation. In some cases, how these samples are identified by different participants can also differ. Sample alignment is crucial for this process as it allows matching corresponding data entries across different datasets in a privacy-preserving manner.

The AIMLE server, which has the ability to know the identity of VAL UEs, can accomplish sample alignment securely and in a privacy-preserving way without revealing the VAL UE identity.

### 5.7.2	Open Issues

This key issue will study:

1.	Whether and how to enhance VFL operation considering also VAL servers, AIMLE clients and AIMLE servers as VFL participants.

2.	Whether and how to support/enhance sample alignment also considering VAL servers / AIMLE servers / AIMLE clients as VFL participants.

* * * First Change * * * *

## 7.15	Solution #15: Sample alignment for VFL

### 7.15.1	Solution description

The solution enhances the clause 8.18 as follows:

### 8.18.2 Procedure for supporting VFL Pre-conditions: 1. VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server. 2. The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains. 3. The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications. Figure 8.18.2-1: Procedure for supporting VFL 0. An AIMLE client registers with AIMLE server as specified in clause 8.7 of 3GPP TS 23.482 [2]. The registration message includes the list of feature collection capabilities and also the data domains in which features can be collected and processed. The registration message also includes the list of feature ID(s) status its expiration time. NOTE 1: The AIMLE server keeps record of all AIMLE clients with similar feature collection capabilities and data domain. 1. An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. The request includes sample pool, and sample binding criteria. The member selection criteria are enhanced to select the members for VFL training if they have features bound to at least one sample withing the sample pool. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between different data domains), the procedure continues to step 2. The AIMLE server also assigns ML task identity to the determined task. The AIMLE server selects the candidate AIMLE clients for the training based on the corresponding feature collection record of AIMLE clients that are capable of collecting features within the scope of determined VFL task and sample pool. The AIMLE server sends response which includes ML task identity and candidate AIMLE client list. 2. [Optional] The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1. If the retrieved model is already trained and meets the machine learning requirements requested by the consumer, the procedure continues to step 7. 3. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training as described step 4 of the clause 8.13.2. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training. 4. [Optional] The AIMLE Server sends a client feature selection request to some clients asking to activate or de-activate some of their feature ID(s)/ feature list for each data domain. The message includes information as described in following Table. Table 8.18.2-1: Client feature selection request Information element; Status; Description. Requestor identifier; M; The identifier of the AIMLE Server.. Feature ID(s); M; Server requests List of feature ID(s) targeted for activation or deactivation.. > Action type; M; Server specifies via indicators whether the request is for activation or deactivation.. Feature Priority Level indicator; O; Indicates the priority of the feature for VFL training. Minimum expiration Time indicator; O; Server informs the client UE(s) on the minimum expiration time to enforce the requested action (activate /deactivate features). 5. [Optional] AIMLE client sends response indicating the activation or deactivation status of their feature ID(s) or feature list for each data domain. The response message includes information as described in following Table. Based on the response message, the AIMLE server finds common feature ID(s) by comparing features descriptions / feature ID(s) of clients. The AIMLE server constructs the pairwise relation among candidates' datasets considering their shared feature ID(s). Table 8.18.2-2: Client feature selection response Information element; Status; Description. Successful response (NOTE); O; Indicates that the request was successful.. > Feature ID(s); M; List of feature ID(s).. >> Action Type status; M; Status of feature ID(s) activate/deactivate operation. > Feature priority level success rate; O; Indicates the percentage of the available features prioritized for VFL training.. > Feature Minimum expiration Time reliability rate; O; Client (UE)s informs the server about the specified feature set/feature ID(s) enforced expiration Time. In case of conflict of interest between UE and server, UE send remarks about further adjustment in expiration time. Failure response (NOTE); O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: any one IE shall be present.. NOTE 2: The AIMLE server gets the hashed feature values from the candidates for their data samples if the feature ID(s) are subsets of feature space union 6. AIMLE server send a sample binding request to AIMLE clients that are capable of collecting or having features corresponding to the defined VFL task. This request contains a set of samples that are defined to be the sample pool of the VFL task by the VAL server. The message includes information as described in following Table. Table 8.18.2-3: Sample binding request Information element; Status; Description. Requestor identifier; M; The identifier of the AIMLE Server.. VFL Task ID; M; The identifier of the VFL task assigned to AIMLE server by the VAL server. Sample Pool; M; A set of samples and their corresponding IDs that are defined within the scope of VFL task to be trained on them.. Sample binding criteria; M; Identifies the criteria (e.g. common feature IDs) that the available or collecting dataset (features) can be bound to samples within the sample pool.. 7. AIMLE client receives the binding request, and it matches its available dataset (i.e. features) at the UE to a corresponding sample within the sample pool of the VFL task according to the received sample binding criteria. ALMLE client sends the sample binding response to AIMLE server. If the ALMLE client was able to match its dataset (i.e. features) to at least one sample within the sample pool of the VLF task, it sends the successful binding response to ALMLE server to continue the process of VFL sample alignment. Otherwise, it sends a failure response to the sample binding request by AIMLE server. The binding response message includes information as described in following Table. Table 8.18.2-4: Sample binding response Information element; Status; Description. VFL Task ID; M; The identifier of the VFL task assigned to AIMLE server by the VAL server. Binding result; M; The binding result either “success” or “failure”. 8. The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through step 3 or according to the responses received in step 7. The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for each data domain for ML model training capability evaluation as described in clause 8.19.2. 9. The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 4 and parameters received in step 1 (Table 8.3.3.1-1). The AIMLE Server determines VFL members for each data domain. The criteria used by the AIMLE Server include: - Available data and minimum number of data samples for the same sample among the VFL members. - Feature alignment of the sample/datasets with data labels among the VFL members. - Available time of the VFL members for support the VFL training operations. - Capability and minimum number of the VFL members for the VFL training operations. - AIML model information for the VFL members and for the AIMLE Server. 10. The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member for the different data domains. 10a. The AIMLE server may report to the VAL server with the training status, that includes intermediate training results, The VAL server may adjust its request on the ML model training. If the VAL Server is providing data labels to complete the training, the VAL Server sends updated training parameters for the AIMLE Server to distribute to the VFL members. The updated training parameters apply for models of VFL members associated with each data domain. NOTE 4: How AIMLE Server coordinates the selected VFL members to perform VFL training is out of scope for this release. 11. The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 6 are repeated for the next training round.

### 7.15.2	Architecture Impacts

No architecture impacts are identified for this solution.

### 7.15.3	Corresponding APIs

The Client feature selection resquest message includes information elements as specified in Table 8.18.2-1. The Client feature selection response message includes information elements as specified in Table 8.18.2-2.

The Sample binding request message includes information elements as specified in Table 8.18.2-3. The Sample binding response message includes information elements as specified in Table 8.18.2-4.

### 7.15.4	Solution evaluation

The solution is related to KI#7: VFL sample alignment across VAL servers. The solution resolved opein issue 1 “Whether and how to enhance VFL operation considering also VAL servers and AIMLE servers as VFL participants”. The solution proposes to find the same training samples (sample alignment problem) among training participants that have different features of it, in a way that the privacy of the participants and their data is preserved during the training process. This solution addresses the scenarios in which VAL server requests assistance from AI/ML Enablement Server to perform Vertical Federated Learning among AIMLE clients. The AI/ML Enablement Server clusters all AIMLE clients based on their data (feature) collection capability, performs sample alignment (i.e. selecting clients, which have different features of the same sample to be participated in vertical federated learning) for all samples within the scope of the defined VFL task among all different possible AIMLE clients. There is no architecture impact identified based on this solution. Whether binding criteria provided by VAL server to AIMLE server is same provided by AIMLE server to AIMLE client can be considered in normative phase.

* * * End of Change * * * *