3GPP TSG-SA WG6 Meeting #70	S6-255556

# Dallas, USA, 17th – 21st November 2025	                                       revision of S6-255257

Source:	Huawei, HiSilicon

Title:	Update to solution#1 about Flow ID description

Spec:	3GPP TR 23.700-52 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	changlinhui@huawei.com

- Introduction

This pCR is proposed to provide the update to solution#1.

2. Reason for Change

The functional entities description about SEALDD server and client now is incomplete, some of the functionalities are missting, e.g., flow ID allocation, SEALDD enabled data transmission quality measurement and create the dummy SEALDD packet, manage the flow ID packet sequence number for the URLLC case.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-52 v0.4.0.

# * * * First Change * * * *

6.1	Solution #1:	SEALDD-S and SEALDD-UU user plane functionality description and protocol stack

6.1.1	Solution description

6.1.1.1	General

This solution addresses the key issue #2.

Currently, SEALDD layer provide several data delivery services towards the VAL layer with different user plane characteristics as analysed in table 6.1.1.1-1.

Table 6.1.1.1-1: SEALDD delivery service user plane analysis

| Services of SEALDD service | User plane characteristic of SEALDD-S | User plane characteristic of SEALDD-UU |
| --- | --- | --- |
| Regular Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID |
| URLLC Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL), and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID shared by the two paths. |
| Data Storage | Put DL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane | Put UL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane |
| BAT data transfer | Refers to the Data storage | Refers to the Data storage |
| XR Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow IDs and multi-modal flow ID. |
| Transmission Quality Measurement |  | SEALDD UU DL monitoring header, or dummy DL SEALDD packet generated by SEALDD server time stamp: local time T1, T2, T3, T4 |

Editor's note:	This table may be not complete and any further updates is FFS.

6.1.1.2	Protocol stack for SEALDD-UU

6.1.1.2.1	User plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-UU.

Figure 6.1.1.2.1-1: SEALDD-UU user plane stack

SEALDD-UU user plane:	This user plane of the SEALDD-UU layer corresponds to the protocol data unit carried between the SEALDD client and the SEALDD server over the SEALDD-UU flow. The protocol(s) used for this SEALDD-UU may varies due to different SEALDD data delivery service.

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the user plane protocol stacks as described in 3GPP TS 23.501 [23.501].

The user plane of SEALDD-UU layer shall support the functions/characteristics listed in table 6.1.1.1.

6.1.1.2.2	Functional entities description Updates

The functional entities description about SEALDD server and client now is incomplete, some of the functionalities are missing, e.g.,SEALDD enabled data transmission quality measurement and create the dummy SEALDD packet, manage the sequence number for the URLLC case, or specific header handling based on deployment specific requirements.

NOTE:	Whether to carry the SEALDD flow ID and the Multi-modal SEALDD flow ID within the SEALDD-UU packet will be determined in normative phase.

7.3.2 SEAL Data Delivery server The SEAL data delivery server functional entity acts as the application server for the data delivery enablement. The SEALDD server supports the following capabilities: a) Support the signalling interaction with VAL server to negotiate the data delivery aspects including QoS requirement, protocols information, bandwidth settings, delivery policy provisioning, transmission quality measurement, background data transfer. b) Support the signalling interaction with the SEALDD client to management the data delivery between the SEALDD server and the SEALDD client, including the establish/update/release of signalling plane and user plane of SEALDD-UU, bandwidth control, transmission quality measurement. c) Providing the application data/media storage. d) Interacting with 5GC via N33/N5 (i.e. send control plane requirements or receive control plane notification) with usage of capability exposed by 3GPP network. e) Support user plane handlings including obtaining the application data/media and/or application signalling data from VAL server via SEALDD-S, delivering it to the SEALDD client via the SEALDD-UU, and receiving the application data/media and/or application signalling data from SEALDD client and providing it to the VAL server via SEALDD-S, enforce the rate control. f) Allocate the SEALDD flow ID, Multi-modal SEALDD flow ID. g) Manage the packet sequence number for redundant SEALDD-UU flows for the URLLC service. h) Support the transmission quality measurement via in-path packet or dummy packet 7.3.3 SEAL Data Delivery client The SEAL data delivery client functional entity acts as the application client for the data delivery enablement. The SEALDD client supports the following capabilities: a) Interact with the SEALDD server to management the data delivery between the SEALDD server and the SEALDD client, including the establish/update/release of signalling and user plane of SEALDD-UU, bandwidth control, transmission quality measurement. b) Support user plane handlings including obtaining the application data/media and/or application signalling data from VAL client, delivering it to the SEALDD server via the SEALDD -UU, and receiving the application data/media and/or application signalling data from SEALDD client, providing it to the VAL server via the SEALDD-S. c) Allocate the SEALDD flow ID d) Support the transmission quality measurement via in-path packet or dummy packet

6.1.1.2.3	Control plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-UU.

Figure 6.1.1.2.3-1: SEALDD-UU control plane stack

SEALDD-UU control plane: The control plane of the SEALDD-UU layer corresponds to the protocol data unit delivered between the SEALDD client and the SEALDD server. This protocol data unit is used for managing SEALDD-UU user plane (e.g., regular transmission connection establishment, transmission quality measurement, etc.), as per the procedures defined in 3GPP TS 23.433 [2].

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the control plane protocol stacks as described in 3GPP TS 23.501 [23.501].

6.1.1.3	Protocol stack for SEALDD-S

6.1.1.3.1	User plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-S.

Figure 6.1.1.3.1-1: SEALDD-S user plane stack

SEALDD-S user plane:	This user plane of SEALDD-S layer corresponds to the protocol data unit carried between the VAL server and the SEALDD server over the SEALDD-S flow. The protocol(s) used for this SEALDD-S user plane may varies due to different SEALDD data delivery service.

The SEALDD-S-U layer shall support the functions/characteristics listed in table 6.1.1.1.

6.1.1.3.2	Control plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-S.

Figure 6.1.1.3.2-1: SEALDD-S control plane stack

SEALDD-S control plane: The control plane of the SEALDD-S layer corresponds to the protocol data unit delivered between the VAL server and the SEALDD server. This protocol data unit is used for managing SEALDD-S user plane data transmission, etc., as per the procedures defined in 3GPP TS 23.433 [2].

6.1.1.4	User plane protocol stack examples for SEALDD-UU and SEALDD-S

6.1.1.4.1	General

This clause illustrates the recommended protocol(s) for different SEALDD data delivery service related to SEALDD-UU and SEALDD-S user plane.

NOTE:	This clause could serve as the basis for the informative Annex of the TS.

6.1.1.4.2	Recommended protocol(s) for Regular Transmission

Regular transmission supports the data delivery of live streaming. The VAL server (content provider) sends live streaming data to the SEALDD server via RTMP/SRT protocol. The SEALDD server handles all Stream Processing, such as converting the RTMP stream into protocols compatible with various playback devices – including RTMP, HTTP-FLV, or HLS. The SEALDD server may then deliver the data to the VAL UE (player) where the SEALDD client resides, using RTMP, HTTP-FLV, or HLS protocols.

Figure 6.1.1.4.2-1: The recommend protocol(s) for live streaming delivery in SEALDD-UU and SEALDD-S user plane stack

Regular transmission supports the VOD. The VAL server (content provider) could send VOD data to the SEALDD server via HTTP(S) “pull” mode. The SEALDD server handles all Stream Processing, such as converting the HTTP(S) stream into protocols compatible with various playback devices – including HLS, HTTP-FLV, or DASH. The SEALDD server may then deliver the data to the VAL UE (player) where the SEALDD client resides, using HLS, HTTP-FLV, or DASH protocols.

Figure 6.1.1.4.2-2: The recommended protocol(s) for VOD in SEALDD-UU 
and SEALDD-S user plane stack

6.1.1.4.3	Recommended protocol(s) for Data Storage

In the data storage service creation procedure, the VAL server/SEALDD client (content provider) transmits data to be stored to the SEALDD server via HTTP/HTTPS protocol. After authorized verification, the SEALDD server configures the storage service for the request and stores data on the storage server. The SEALDD server then sends a response to the SEALDD client/VAL server via the HTTP/HTTPS protocol.

Figure 6.1.1.4.3-1: The recommended protocol(s) for Data Storage in 
SEALDD-UU and SEALDD-S user plane stack

6.1.1.4.4	Recommended protocol(s) for XR Transmission

XR Transmission supports the data delivery of real-time streaming media. The VAL server (content provider) transmits real-time media data to the SEALDD server via WebRTC protocol. The SEALDD server manages all Stream Processing, including converting the WebRTC data into playback-compatible protocols such as RTMP/HLS/FLV. Subsequently, the SEALDD server delivers processed data to the VAL UE (player) hosting the SEALDD client using the RTMP/HLS/FLV protocol.

Figure 6.1.1.4.4-1: The recommended protocol(s) for XR Transmission 
in SEALDD-UU and SEALDD-S user plane stack

The SEALDD server and client, to provide enhanced e2e functionality, may use protocol extensions, as e.g., the inclusion of specific header information, as e.g., RTP/SRTP extension headers, as defined in 3GPP TS 23.433.

6.1.1.4.5	Recommended protocol(s) for Transmission Quality Measurement

Transmission Quality Measurement supports monitoring of transmission quality in an on-going regular data transmission connection. The monitoring information can be carried in the protocol extension header or in the protocol payload. Accordingly, the recommended Transmission Quality Measurement protocol depends on the protocol employed by the monitored data transmission connection.

6.1.2	Architecture Impacts

This solution has no architecture impacts.

6.1.3	Corresponding APIs

This solution has no new API.

6.1.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * End of Change * * * *