3GPP TSG-SA WG6 Meeting #70	S6-255599

Dallas, United States 17th – 21st November 2025	(revision of S6-255114)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Cross-Framework Integration

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

Add Cross-Framework Integration clause.

2. Reason for Change

Complete clause 6.2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.2.0.

* * * First Change * * * *

## 6.2	Cross-Framework Integration

### 6.2.1 General

This section describes how entities from the EDGEAPP and SEAL frameworks integrate with CAPIF. Each subsection identifies the roles that different entities play within CAPIF, whether as API Providers offering APIs for discovery and consumption or API Invokers discovering and consuming APIs published by others.

Understanding these roles ensures that CAPIF can provide a unified and interoperable service exposure layer across both EDGEAPP and SEAL domains, supporting discovery, authentication, and authorization for API consumption in a consistent manner.

Editor’s Note: 3GPP TS 23.558 and 3GPP TS 23.434 provide clauses on the relationship of EDGEAPP and SEAL to CAPIF respectively and therefore further consideration is required on leveraging those clauses.

### 6.2.2 EDGEAPP Roles in CAPIF

In the context of the overall CAPIF–EDGEAPP integration, it is important to distinguish between the perspectives of the application developer, typically working for an Application Service Provider, and the edge compute service provider operating the EDGEAPP framework itself.

An application developer is mainly concerned with the Application Client (AC) and Edge Application Server (EAS) behaviour and their interactions with the Edge Enabler Client (EEC) and Edge Enabler Server (EES), since these determine how the application runs and scales at the edge.

Conversely, the edge compute service provider manages the Edge Configuration Server (ECS), EES, and associated control functions that expose and orchestrate edge capabilities. The following description therefore focuses on how each EDGEAPP functional entity behaves as API Provider or Invoker within CAPIF, from the viewpoint of the edge compute service provider domain.

In EDGEAPP framework, the ECS acts as API Provider within CAPIF. It exposes control and configuration APIs, such as EES Registration. Publishing these APIs in CAPIF allows other entities to locate them. However, in deployments supporting multi-operator federation or roaming, (see clauses 6.5.9 EDGE-8 and 8.17 of 3GPP TS 23.558 [4]), an ECS can also act as an API Invoker, consuming APIs exposed by peer ECS instances in other administrative domains to exchange configuration, topology, and service availability information.

The EES functions as both an API Provider and an API Invoker, depending on its interactions. As an API Provider, it publishes key service APIs such as EAS Registration, EAS Discovery, and EEC-assistance functions. Registering these APIs in CAPIF allows for standardized discovery, endpoint metadata retrieval, and alignment with security requirements across edge zones. As an API Invoker, the EES consumes APIs exposed by the ECS for registration, configuration synchronization, and policy management. In these cases, the EES must onboard to CAPIF and obtain OAuth tokens to invoke ECS APIs securely.

The EAS acts as API Provider within CAPIF primarily when it exposes enabler-level service APIs such as EAS Registration, EAS Discovery, or other control interfaces towards the EES or other EAS instances. These APIs belong to the EDGEAPP control and management layer and can be published in CAPIF for discovery and secure invocation. The EAS can also act as an API Invoker, particularly when it needs to consume APIs from the EES (for lifecycle or UE monitoring) or from other enablers, such as SEAL. This duality allows EAS instances to integrate context-aware services or network-exposed functions obtained through CAPIF.

The EEC, located on the UE, acts optionally as API Invoker and in typical deployments, the EEC discovers its serving EES through operator-controlled methods such as pre-provisioned configuration, DNS resolution, or network signalling. In EDGEAPP, the EEC also interacts with the ECS via the EDGE-4 interface to obtain the appropriate EES through the service provisioning procedure. However, when a unified CAPIF integration is in place, the EEC can also use CAPIF to discover and authenticate access to EAS application APIs. In that case, the EEC must be onboarded as an API Invoker to retrieve tokens and invoke protected EAS endpoints on behalf of the Application Client. While the EEC exposes local APIs toward the Application Client to enable service registration and discovery, it does not act as a CAPIF API Provider, since those APIs are not published or discoverable through CAPIF.

The AC has no role in CAPIF. It remains abstracted behind the EEC, communicating only through the EDGE-5 interface. All CAPIF-related onboarding, discovery, and token management are handled by the EEC on its behalf. The AC is intentionally kept CAPIF-agnostic to simplify application development and maintain service portability across different network domains.

### 6.2.2 SEAL Roles in CAPIF

In SEAL, the SEAL Server can onboard as a CAPIF API Provider. It can expose enabler APIs such as Group Management, Configuration Management, and Location Management, which provide reusable context and management capabilities to vertical applications. Publishing these APIs in CAPIF enables VAL Servers or other authorized consumers to discover, authorize, and invoke them securely under the CAPIF trust domain. When the SEAL Server utilizes network interfaces, as depicted in Figure 6.2-1 of 3GPP TS 23.434 [6], to retrieve information (e.g., UE location or network context) from core network exposure functions such as the NEF, the SEAL Server can operate as an API Invoker toward network APIs.

The VAL Server acts as a CAPIF API Invoker whenever it consumes SEAL APIs. It must onboard as an Invoker in CAPIF to discover the SEAL Server’s API catalogue, establish a security context, request access tokens with the appropriate scopes and invoke the corresponding APIs.

The SEAL Client, located in the UE, can optionally act as an API Invoker depending on the operator’s security model. In most deployments, the SEAL Client is pre-provisioned with the SEAL Server’s endpoint and performs registration through the SEAL-Uu reference point without requiring CAPIF onboarding. However, in deployments where CAPIF provides unified authentication and authorization for all service invocations, the SEAL Client can be onboarded as an API Invoker, allowing it to obtain CAPIF-issued tokens to access SEAL exposed APIs securely. The SEAL Client never acts as an CAPIF API Provider, as it only consumes SEAL APIs.

The VAL Client communicates with the VAL Server through standard application interfaces and remains CAPIF-agnostic. CAPIF integration and token management are handled entirely by the VAL Server. Only in extended or open-API deployments, where VAL services are exposed directly to external clients, can a VAL Client onboard to CAPIF as an API Invoker to consume VAL Server APIs directly. This scenario is optional and beyond the baseline SEAL architecture defined in 3GPP TS 23.434 [6].

* * * End of Changes * * * *