3GPP TSG-SA WG6 Meeting #70	S6-255631

Dallas, USA, 17th – 21st November 2025	revision of S6-254690, 5211, 5508

Source:	Ericsson

Title:	New Solution for KI#6 on LM Service Enhancements to Location Reporting for Support Energy Saving

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #6 (TR 23.700-44 clause 5.6), the study aspects on SEALDD, LM, ADAE, and other services enhancements for energy saving include:

- Identify whether and how existing application enablement layer services (e.g. SEALDD, LM, ADAE) need be enhanced to support energy savings.

NOTE 1:	The existing application enablement layer services considered in this Key Issue are the ones which are not covered in the other Key Issues.

The existing mechanisms does not support the energy saving requirements for the UE location reporting, e.g., UE is not allowed to change the location reporting configurations based on current battery level. This pCR proposes to enhance the LM service on location reporting to support saving energy of UE.

2. Reason for Change

This pCR proposes a new solution for Key Issue #6, to enhance the LM service on location reporting to support saving energy of UE.

3. Conclusions

This paper proposes solution on enhancements of LM service on location reporting to support saving energy of UE.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol #Z |  |  |  |  |  | X |

* * * Next Change * * * *

## 6.Z	Solution #Z: LM Service Enhancements to Location Reporting for Support Energy Saving

### 6.Z.1	Solution Description

#### 6.Z.1.1	General

Definition of energy saving mode:

- Economy mode: an energy saving mode when the service provider (e.g., SEAL Server, etc.) minimizes energy consumption prioritized over performance.

- Balanced mode: an energy saving mode when the service provider (e.g., SEAL Server, etc.) balances the energy consumption and performance.

- Performance mode: an energy saving mode when the service provider (e.g., SEAL Server, etc.) provides high performance without energy saving constrains.

The services procedures and information flows introduced in 3GPP TS 23.434 [14] for location reporting, can be enhanced to support the energy saving requirements. For the enhancements, the energy saving modes for LM services:

- Economy mode indicates that the consumer (LMC, VAL server) requests to minimize the energy consumption at UE side for location reporting.

- Balanced mode indicates that the consumer (LMC, VAL server) requests to balance the energy consumption at UE side and satisfy the location reporting requirements.

- Performance mode indicates that consumer (LMC, VAL server) requests to provide location reporting from UE side without energy saving constrains.

#### 6.Z.1.2	Enhance location reporting procedures

The procedure for Location reporting as specified in clause 9.3.3.3 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

#### 9.3.3.3 Location reporting Figure 9.3.3.3-1 illustrates the procedure for location reporting. Figure 9.3.3.3-1: Location reporting procedure 1. The location management client sends a location information report to the location management server, containing location information identified by the location management server and available to the location management client. The location management client may include low battery indication if experience the low battery charge. 2. Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client. If low battery indication is received, the location management server may initiate the location reporting adaptation if the adaptive reporting is requested in step 1 of clause 9.3.5.

The procedure for Location reporting as specified in clause 9.3.5 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

### 9.3.5 Client-triggered or VAL server-triggered location reporting procedure Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting. Figure 9.3.5-1: Client-triggered location reporting procedure 1. Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. The location reporting event triggers as specified in table 9.3.2.4-1, e.g. minimum time between consecutive reports, SAI changes, access RAT changes, or ECGI changes for reporting the location of the VAL UE, are included. NOTE: Step 1 can be performed when location management client 2 or VAL server require to update the location reporting trigger corresponding to location management client 1. 2. Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. If the immediate report indicator is included or the triggering criteria is periodical reporting in step 1 and there is valid location report of the target UE (i.e. the timer for the stored location report is not expired), the location management server reuses the stored UE location report and sends the report to the location management client 2 or VAL server. a. The location management server determines the location information source (i.e. location management client 1 and/or 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11])), to satisfy VAL server request in step 1 (e.g. if energy saving mode is set to "Economy mode", the location management server may select 3GPP CN as location information source to save energy at location management client). b. If the adaptive reporting is requested in step 1, the location management server interacts with NWDAF (e.g., UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [34]) to obtain the location management client 1 moving statistic and prediction i. If "DIRECT UPDATE" is requested, the location management server determines the reporting configuration based on VAL service ID, VAL server ID, Energy Saving mode, and the retrieved location management client 1 moving statistic and prediction. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or update location change condition (e.g., to change the location accuracy). ii. If "SUGGESTIVE UPDATE" is requested, the location management server initially uses the configuration provided by VAL server and later dynamically adjusts the reporting configuration to determine adaptive location configuration based on VAL service ID, VAL server ID, Energy Saving mode, and by analysing the retrieved location management client 1 moving statistic and prediction and received location information. In order to adapt the configuration, the location management server may determine, for example, whether to increase or decrease the frequency of the report based on the UE's mobility pattern, or update the location change condition (e.g., to change the location accuracy). c. If the valid location report is not available and 3GPP CN was selected as a source of location data in step 2.a, the location management server requests 3GPP CN to monitor the location management client 1 location change. d. If the valid location report is not available and location management client was selected as a source of location data in step 2.a, depending on the information specified by the location reporting trigger or determined reporting configuration (if "DIRECT UPDATE" is requested), location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. 3. Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server. Further, if adaptive reporting is enabled by the VAL server with "SUGGESTIVE UPDATE", the location management server dynamically adjusts the reporting configuration as specified above (in step 2.a.ii). Once location configuration is adjusted and if it is required to update the triggers to the SEAL client, the SEAL LMS may suggest the adaptive location configuration to the VAL server (or authorized SEAL location management client) as specified in clause 9.3.19. If VAL server accepts the suggested adaptive location configuration, then the location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1. If VAL server rejects the suggested adaptive location configuration, then the location management server will discard the updated configuration.

#### 6.Z.1.3	Enhancements to information flows

The information flow for Location reporting trigger as specified in clause 9.3.2.4 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

#### 9.3.2.4 Location reporting trigger Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure. Table 9.3.2.4-1: Location reporting trigger Information element; Status; Description. Requestor Identity; M (see NOTE 1); Identity of the requesting authorized VAL user or VAL UE or VAL server. Identity; M (see NOTE 1); Identity of the requested VAL user or VAL UE. VAL service ID; O; Identity of the VAL service for which the location reporting trigger is set.. Immediate Report Indicator; O (see NOTE 2); Indicates whether an immediate location report is required. Requested location information; O (see NOTE 2); Identifies what location information is requested. This information element may be represented as VAL service area ID(s) when this information flow from VAL server to the location management server.. Triggering criteria; O (see NOTE 2); Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE as specified in table 9.3.2.4-2.. Minimum time between consecutive reports; O (see NOTE 2); Defaults to 0 if absent otherwise indicates the interval time between consecutive reports. Endpoint information; O; Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required.. Adaptive reporting; O; It indicates the request for an adaptive location reporting by dynamically adjusting the configuration. Following are possible values: - "DIRECT UPDATE", indicates the SEAL LMS to directly update the SEAL LM client once configuration is adjusted; or - "SUGGESTIVE UPDATE", indicates that the VAL Server subscribes to receive the suggested configurations from the SEAL LMS. The related notifications shall be sent by SEAL LMS before updating to the SEAL LM client.. Energy Saving mode; O; It indicates the Energy Saving mode (i.e. Economy mode, Balanced mode, or Performance mode) for the SEAL LM client reporting and corresponding thresholds. - Economy mode indicates that the VAL server requests to constrain the energy consumption at UE side for location reporting providing best efforts to satisfy provided location reporting requirements. - Balanced mode indicates that the VAL server requests the best effort for energy saving at UE side to satisfy the location reporting requirements. - Performance mode indicates that VAL server requests to provide location reporting from UE side without energy saving constrains. The default value is Performance mode if omitted.. NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present.. Editor’s Note: How to use the threshold for energy saving mode is FFS. Table 9.3.2.4-2: Location reporting Triggering criteria Information element; Status; Description. Frequency of reporting; O; It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered.. Reporting periodicity; O; If the Frequency of reporting is periodic, the reporting periodicity shall be provided.. Location change condition; O (see NOTE); It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates, SAI, ECGI, RAT, VAL service area. If the Frequency of reporting is event-triggered, this IE shall be provided.. Inside or outside indication; O (see NOTE); It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations represented by the VAL service area ID(s).. Reporting schedule; O; It indicates the requested reporting schedule, e.g., days of the week and/or time period for the location reporting.. NOTE: Triggering criteria based on the VAL service area represented by the VAL service area ID(s) may be provided when this information flow is from the VAL server to the location management server..

The information flow for Location information report as specified in clause 9.3.2.2 of 3GPP TS 23.434 [14] is enhanced as follows (new text in bold italics):

#### 9.3.2.2 Location information report Table 9.3.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting or from the location management server to the requesting location management client or VAL server to report location information. Table 9.3.2.2-1: Location information report Information element; Status; Description. Set of identities; M; Set of identities of the reporting VAL users or VAL UEs. Triggering event; M; Identity of the event that triggered the sending of the report. The triggering event may be e.g., SAI changes, ECGI changes, RAT changes.. Location Information; M; Location information. For LM-UU reference point, it may include UE location information via non-3GPP positioning technologies (e.g. GNSS, Sensor, TBS, WLAN, Bluetooth) as described in 3GPP TS 37.355 [53].. Velocity information; O (NOTE 1); Velocity information of the reporting VAL users or VAL UEs.. Failure Reason; O (NOTE 2); Indicates the failure reason when the LM Client cannot obtain the requested UE’s location information (e.g. the target VAL UE has moved away, or required positioning method is not supported by target VAL UE). Low battery indication; O; Indicates that battery is low.. Timestamp; O; Timestamp of the location report. NOTE 1: The definition of the velocity refers to 3GPP TS 23.271 [60]. NOTE 2: This element is only applicable when the Location information IE is null..

### 6.Z.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.Z.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***