3GPP TSG-SA WG6 Meeting #69	S6-254515

Wuhan, China, 13th – 17th October 2025	(revision of S6-254169)

Source:	China Mobile

Title:	New Solution of edge application server discovery considering energy consumption

Spec:	3GPP 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liutangqing (liutangqing@chinamobile.com)

lvhuazhang (lvhuazhang@chinamobile.com)

1. Introduction

This pCR proposes new solution for FS_EnergySys_Ph2_APP.

2. Reason for Change

This paper proposes to give the solution to solve the key issue #1 and key issue #3 to enhance the edge application service discovery procedure.

As indicated in TS 28.310[4], the energy consumption and energy efficiency have the following definitions, and these definitions are referred from TS 28.310[4]:

Energy Efficiency (EE): ratio between performance and energy consumption.

NOTE 8: the performance may be measured based on e.g. data volume, latency, number of active users, etc.

Energy Consumption (EC): integral of power consumption over time.

NOTE 9: see ETSI ES 202 706-1 [23].

As indicated in TS 28.310[4], the renewable energy and renewable energy factor have the following definitions:

Renewable energy: energy from renewable non-fossil sources.

NOTE 15:	This definition is taken from 3GPP TS 22.261 [31].

NOTE 16:	Examples of renewable energy sources include wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 17:	A renewable energy source is an energy source not depleted by extraction as it is naturally replenished at a rate faster than it is extracted ISO/IEC 30134-3:2016 [33].

NOTE 18:	Criteria to categorize an energy as renewable can differ among jurisdictions, based on local environmental or other reasons ISO/IEC 30134-3:2016 [33].

Renewable energy factor: ratio of the renewable energy to the total energy.

NOTE 19:	This definition is taken from ISO/IEC 30134-3:2016 [33].

Energy can be classified into two major types: renewable energy and non-renewable energy. Renewable energy sources include solar energy, hydropower, wind energy, biomass energy, wave energy, tidal energy, ocean thermal energy, geothermal energy, etc. They can be recycled and regenerated in nature. It is an inexhaustible and endless energy source that regenerates automatically without human intervention.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #x | x |  | x |  |  |  |
| Sol #2 |  |  |  |  |  |  |

* * * Second Change, all new text * * * *

6.x	Solution #x: Edge Application Server Discovery by Considering Energy Consumption and Energy Efficiency

6.x.1	Solution Description

The proposed solution solves the problem listed in KI#1 and KI#3 and proposes to consider energy efficiency and energy consumption in edge computing.

In Edge Computing deployment, an application service may be served by multiple Edge Application Servers typically deployed in different sites. These multiple Edge Application Servers that host service may use a single IP address (anycast address) or different IP addresses. To start such a service, the UE needs to know the IP address(es) of the Application Server(s) serving the service. The UE may do a discovery to get the IP address(es) of a suitable Edge Application Server (e.g. the closest one), so that the traffic can be locally routed to the Edge Application Server and service latency, traffic routing path and user service experience can be optimized.

But, in the current design of EDGEAPP in SA6, there exists the following issues:

Firstly, during the EAS discovery procedure, that the Energy Efficiency or Energy Consumption of EAS is not considered. If one EAS has the higher Energy Efficiency or lower Energy Consumption, this EAS has the following benefits:

-	Lower energy consumption or higher energy efficiency can improve the response speed and processing capacity of applications, allowing users to enjoy a better experience.

-	The low power consumption and lower heat generation feature of servers enhance the stability and reliability of server operation, effectively reducing service interruptions caused by hardware failures.

-	Lower energy consumption or higher energy efficiency in application server leaves sufficient space for future service expansion and maintaining a high-quality service level, for example, if user performs high energy consumption service like XRM rendering.

Secondly, during the EAS discovery procedure, that the renewable energy of EAS is not considered. In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy. For example, some servers can be powered by electricity converted from renewable energy to reduce the carbon emission. Considering the renewable energy can be recycled and regenerated in nature, if the UE has renewable energy in energy consumption, from operator's policy, the operator can improve the QoS to a certain extent.

Also, the EAS can act as AF to subscribe the energy consumption information of certain UE or certain QoS from 5GC. And if renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

6.x.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of enabling edge applications defined in section 6.2 of TS 23.558[y]. But the following enhancements are added:

-	The EES subscribes the Energy Efficiency and Energy Consumption of EAS. And the EES selects the lower Energy Consumption or higher Energy Efficiency related EAS for EEC.

-	The EES can subscribe to EAS whether renewable is consumed by EAS. If the EAS consumes the renewable energy, for example in the morning time that solar energy can be consumed, the EES may select this EAS to save the energy.

-	The EEC can indicate to the EES to select the lower Energy Consumption or higher Energy Efficiency EAS, or indicate to select the EAS that consumes the renewable energy.

NOTE: As an example, that the lower Energy Consumption or higher Energy Efficiency here, can be that the application client has already have application layer connection with EAS 1 with the value of Energy Consumption is A, and the EEC indicates to select another EAS that the value of Energy Consumption is B. The value of A is larger than B.

-	The EAS can act as AF to subscribe the energy consumption information of certain UE or certain QoS from 5GC. And if renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

Editor’s note: How EAS know the Energy Efficiency and Energy Consumption information is FFS.

6.x.3	Procedure

6.x.3.1	Procedure of subscription Energy Consumption information from EAS

Figure 6.x.3.1-1: EAS energy consumption information subscription

1.	The EES sends an EAS energy consumption information subscription request to the EAS. The EAS energy consumption information subscription request includes the EES ID along with the security credentials, Event ID, time period and energy threshold to subscribe to information about energy consumption information of EAS.

The energy consumption information includes the Energy Efficiency (EE), Energy Consumption (EC) and Renewable energy of EAS. The Renewable energy of EAS includes that either the indication of the EAS consumes renewable energy or not, or the Renewable energy factor of EAS.

The time period indicates the EAS to notify the Energy Consumption information of EAS in the certain time period in the future.

The energy threshold indicates the value of energy consumption information. The energy threshold can be the value of Energy Efficiency (EE), the value of Energy Consumption (EC), the value of Renewable energy factor and etc.

NOTE: For example, the energy consumption of EAS on standby status is 150W. And if the EAS reaches the energy threshold, the EAS notifies the EES.

2.	Upon receiving the request from the EES, the EAS checks if the EEC is authorized to subscribe for information of the requested EAS(s).

3.	If the processing of the request was successful, the EAS sends an EAS energy consumption information subscription response to the EES, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire.

4.	The EAS sends an EAS energy consumption information notification to the EES with the EAS energy consumption information determined in step 1.

If the energy threshold is provided in step 1, the EAS sends the notification to EES according to the comparison between the value of energy threshold and the value of energy consumption information of EAS.

6.x.3.2	Procedure of EAS discovery considering energy consumption information

The procedure listed here are based on the procedure defined in section 8.5.2.2 of TS 23.558[y].

8.5.2.2 Request-response model Pre-conditions: 1. The EEC has received information (e.g. URI, IP address) related to the EES; 2. The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and 3. The EES is configured with ECSP's policy for EAS discovery. NOTE 1: Details of ECSP's policy are out of scope. Figure 8.5.2.2-1: EAS Discovery procedure 1. The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters, EEC service continuity support, and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route, indication and energy threshold. The request may include an EAS selection request indicator. If an e2e tunnel is used in the UE for applications (e.g. user configured tunnel service), and if the tunnel service provider and the ECSP are from the same organization and only if user grants the permission, then the EEC provides the tunnel information for the associated applications in the request to the EES considering user consent. The indication indicates the EES to discover or select the most suitable EAS that has lower energy consumption or higher energy efficiency or support renewable energy. If the indication indicates to select the EAS that support renewable energy, it means that either the EAS supports the renewable energy supply, or the Renewable energy factor of the EAS exceeds the energy threshold. If the indication indicates to select the EAS that lower energy consumption or higher energy efficient, it means that the value of EE exceeds the energy threshold or the EC lower than the energy threshold. NOTE 2: The e2e tunnel case is limited to case where the tunnel service is deployed in home domain in the present release. NOTE: For example, if EAS 1 has higher energy efficient compared to EAS 2, that EAS 1 is selected by EES. Or if the energy efficient exceeds energy threshold, this EAS can be selected as candidate EAS. 2. The same as step 2 in section 8.5.2.2. 3. If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. If the EES perform traffic influence for EAS(s) in step 2, then the discovered EAS(s) is with optimized traffic route. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs and Instantiable EAS Information with EAS instantiation completion time. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. When the discovered EAS is for a certain application group, then the Application Group ID is also included in the response message. If the discovered EAS is registered to another EES, then the EES endpoint of the EES where the discovered EAS is registered is also included in the response message. for the discovered EASs, may satisfy the conditions below: The energy consumption of the EAS is lower than the energy threshold in step 1, and this EAS is selected. The energy efficiency of the EAS is higher than the energy threshold in step 1, and this EAS is selected. The Renewable energy factor of the EAS is higher than the energy threshold in step 1, and this EAS is selected. The EAS starts to consume the renewable energy as energy supply, and this EAS is selected. When the EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. Then the EES determines the remaining EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation in progress ECSP management system and the predicted EAS deployment time. When EEC receives the EAS instantiation in progress indication, the EEC may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the EES for obtaining updated EAS information. If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause. If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required. If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15. If the EAS discovery response message contains the EAS(s) information along with r EAS instantiation completion time which the EAS instantiation in progress, then the EEC determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time/EAS instantiation completion time. If the EEC determines to identify the EAS for which instantiation is in-progress, the EEC may retry to discover the EAS and send the EAS discovery request again after waiting for the time as included in the response message, otherwise, the EAS discovery may fail. NOTE 10: Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command). NOTE 11: The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document. NOTE 12: The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required. If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request. NOTE 13: As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

6.x.3.3	Procedure of QoS update related to renewable energy consumption

The procedure listed here are based on the procedure defined in section 8.6.6.2.3 of TS 23.558[y].

8.6.6.2.3 Update a session Figure 8.6.6.2.3-1 illustrates the session with QoS update request operation between the EAS and the EES. It is used to request a modification of the QoS of the data session between AC and EAS. Figure 8.6.6.2.3-1: Session with QoS API: update operation 1. The EAS requests a modification of the QoS of the data session between the AC and the EAS, e.g. by including a different QoS reference or different bandwidth value. The EAS shall include the Context ID, EAS get information of energy consumption information of certain UE and QoS flow contains renewable energy consumption, from ESE server of 5GC as described in clause 4.29 of TS 23.502. The EAS trigger the session with QoS update request if either of the following conditions are satisfied: - Renewable energy consumption information is included in the energy consumption information of certain UE or QoS flow from 5GC. This may indicate that renewable energy is consumed for certain UE or QoS flow. - Value of renewable energy consumption exceeded the energy threshold. For example the total renewable energy consumption or the ratio of renewable energy consumption in the energy consumption information of certain UE or QoS flow exceeded the energy threshold configured in EAS. The EAS may also request a modification of the event monitoring by subscribing to new events and updating or removing subscriptions to existing events. 2. The EES checks if there is a context for the Context ID available and whether the EAS is authorized for this operation. If yes and the UE IP address is known by the EES, the EES invokes the Policy Authorization Update service or AF Session with QoS Update service with the 3GPP Core Network (PCF or NEF, respectively) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], providing the updated information to the PCF or NEF, respectively. The EES may subscribe to additional events or unsubscribe to certain events, if necessary. 3. If the operation is successful, the EES responds with a Context ID and a Result. If the EAS is not authorized or any other failure happens during the operation, the EES provides a rejection response with cause information. NOTE: The EES will report the resource allocation outcome, e.g. the successful modification of the Service Data Flow(s) related to the data session, with a separate session with QoS notify operation (see 3GPP TS 23.503 [12], clause 6.1.3.18).

6.x.4	Solution evaluation

Editor’s note: This part is for further update.

* * * Third Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Energy Consumption (EC): As defined by the TS 28.310 [4].

Energy Efficiency (EE): As defined by the TS 28.310 [4].

Renewable Energy: As defined by the TS 28.310 [4].

Renewable energy factor: As defined by the TS 28.310 [4].

* * * Fourth Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[y]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[z]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS)".

* * * End of Changes * * * *