3GPP TSG-SA WG6 Meeting #69	S6-254689

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution for support of energy saving for location services

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to support the energy saving for location services.

2. Reason for Change

Propose a new solution to support the energy saving for location services.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * *

6.x	Solution #x: Support of energy saving for location services

6.x.1	Solution description

This solution addresses the KI#6.

The following are the procedures to introduce how LMS subscribes the energy related information analysis from ADAES (clause 6.x.1.1) and how LMS utilizes the obtained energy related information analysis to reduce the energy consumption for the location services (clause 6.x.1.2).

6.x.1.1	Procedure of LMS subscription for the energy related information analysis

Figure 6.x.1.1-1 illustrates the high-level procedure of the LMS subscription for the energy related information analysis.

The LMS may subscribe to the ADAES for the analysis of energy related information.

Figure 6.x.1.1-1: Procedure of LMS subscription for the energy related information analysis

- LMS may subscribe the energy information analysis related to location services from ADAES, including the analysis ID (e.g., energy consumption information), the service ID, the target area, the energy type (e.g., energy consumption), the energy information granularity (e.g., per VAL UE per location service), the filtering information (e.g., maximum energy consumption per VAL UE), the target UE ID, the trigger conditions, the time duration, etc.

- The ADAES check if the LMS is authorized to request the energy information analysis. If the request is authorized, the ADAES responds to the LMS.

- The ADAES sends the energy information analysis response to the LMS.

- To obtain the analysis for the energy information, the ADAES may subscribe the energy related information (e.g., energy consumption information) from 3GPP Core network as described in clause 5.51.2.4 of 3GPP TS 23.501 [23501], with the parameters received in service request in step 1.

- When the trigger condition is met, the 3GPP Core network provides the energy related information (e.g., energy consumption information) to the ADAES per requested granularity (e.g., per VAL UE per location service).

- Upon receiving the energy related information in step 5, the ADAES utilizes and generates the analysis for energy related information per VAL UE per location service, including the energy information statistic (e.g., maximum/average/minimum energy consumption information over a certain period of time, etc.), historical energy information, energy information predication (e.g., the time point of peak energy consumption), etc.

Editor’s note: How the ADAES generates the analysis for energy related information and which information is FFS.

- The ADAES notifies the energy information analysis to the LMS as service requested, including the analysis results generated in step 6.

6.x.1.2	Procedure of LMS adjusting the location reporting configuration to save the energy

Figure 6.x.1.2-1 illustrates the high-level procedure of LMS adjusting the location reporting configuration to save the energy consumption.

Figure 6.x.1.2-1: Procedure of LMS adjusting the location reporting configuration to save the energy

- Similar with step 1 of clause 9.3.5 in 3GPP TS 23.434 [23434], the VAL server sends a location reporting trigger to the LMS to activate a location reporting procedure for obtaining the location information of LMC with the parameters defined in clause 9.3.2.4 of 3GPP TS 23.434 [23434]. The location reporting trigger is periodical and the request also includes the energy saving indicator which means the LMS should consider saving the energy when performing the location services for the target UE.

- Upon receiving the service request, the LMS decides to subscribe the energy information analysis from the ADAES for the target UE, following the procedure as described in clause 6.x.1.1.

The ADAES replies the energy related information analysis for the target UE to the LMS, and may include the energy information statistic (e.g., maximum/average/minimum energy consumption information over a certain period of time), energy information predication (e.g., the time point of peak energy consumption), etc.

- Based on the energy related information analysis received from ADAES and the service request from VAL server, the LMS determines the UE location reporting configuration. For example, the LMS may decrease the reporting frequency when the UE’s energy consumption almost reaches its daily maximum, and vice versa. Or the LMS may pend the location reporting when the time point of peak energy consumption is coming based on the UE's energy information predication.

- The LMS may interact with the VAL server regarding the determined UE location reporting configuration and sends the adaptive location reporting configuration provisioning messages as defined in clause 9.3.20 of 3GPP TS 23.434 [23434].

- If the VAL server approves the suggested location reporting configuration, the LMS sends such configurations to the LMC as defined in clause 9.3.3 of 3GPP TS 23.434 [23434]. If the VAL server rejects the suggested location configuration, the LMS may discard the configuration.

- Considering the received location reporting configuration, the LMC reports the location information to the LMS dynamically when the location reporting trigger is met.

- The LMS reports the received location information for the target UE to the VAL server.

6.x.2	Architecture Impacts

This solution has no impact on the existing architecture.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[23434]	3GPP TS 23434: "Service Enabler Architecture Layer for Verticals (SEAL)".

[23501]	3GPP TS 23501: "System architecture for the 5G System".

* * * End of Change * * * *