3GPP SA WG6 Meeting #69	S6-254725

# Oct. 13 – 17th, 2025; Wuhan, China. 			    (revision of S6-254524, S6-254112)

Source: 	China Mobile, Ericsson

Title: 	KI#1 Solution: Enhance Application enablement layer for Ambient IoT services

Document for: 	Approval

Agenda Item: 	9.8

Work Item / Spec.:	FS_AmbientIoT_Ph2_APP / TR 23.700-26 v0.1.0

Abstract of the contribution: This document proposes a solution to address the KI#1 (TR 23.700-026, i.e., enhance application enablement layer for Ambient IoT services.

## 1	Introduction

This pCR proposes a solution to address the KI#1, i.e., enhance application enablement layer for Ambient IoT services. The solution is based on the current SA2 Ambient IoT architecture as in 3GPP TS 23.369 [3], particularly revolving around the SA2 Rel-19 design of only having RAN reader (Note that the UE reader will be introduced in the SA2 Rel-20). Thanks to the lack of any application enablement layer support at the UE (or client) side, the proposed architecture will concentrate on the server side of the application enablement layer for the support of Ambient IoT services.

## 2	Reason for Change

Ambient power-enabled IoT devices or AIoT devices are of low complexity, small size, lower power consumption, having limited memory capacity, less compute power, and lacking advanced capabilities, which make AIoT devices different from previously defined 3GPP IoT devices. Therefore, AIoT devices cannot accommodate and run directly the VAL functionality component (i.e., VAL client) to support AIoT services, nor can they support the application enablement layer client entity (e.g., SEAL client).

For AmbientIoT_App service, according to the 3GPP TS 23.434[5] clauses 6.2, the generic on-network functional model for SEAL, while there is neither VAL client nor SEAL client providing the service enabler layer support functions from the client side, there should necessarily exist the VAL server (i.e., AIoT application server or the 3rd-party AS/AF) and the SEAL server. The SEAL server communicates with the VAL server via the SEAL-S reference point. However, due to the lack of the corresponding client-side service enabler layer functional entities (e.g., VAL client, SEAL-client), a SEAL server may only communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system, i.e., either the N33 via NEF (for an untrusted AF), or the new interface off the SA2-defined AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF (as in the 3GPP TS 23.369[3]).

Given the unique (one-side) functional model, the application enablement architecture layer can only be enhanced at the server side of the application enabler for Ambient service. In one option, the Ambient IoT services might be supported by enhancing the relevant existing SEAL services (e.g., SEALDD). For which, the AIOTApp is deployed as a functionality component of the SEALDD, then the architecture for SEALDD is applicable to AIOTApp. However, the lack of client-side support of a SEAL service, e.g., the SEALDD service, post challenge to this architectural option.

Thus, the more suitable design shall be defining an independent architecture, for which an AIoTApp server function is defined and acts as a SEAL layer function entity.

## 3	Proposal

It is proposed to agree and capture the solution for the inclusion in the TR 23.700-26 v0.1.0.

# * * * * First change * * * *

## 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| …… |  |  |  |  |
| Sol #X | X |  |  |  |

# * * * * next change * * * *

## 7.x	Solution #x: Enhance application enablement layer for Ambient IoT services

### 7.x.1	Solution Architecture

This solution proposes to define an independent architecture, based on which a new application enabler AIoTApp is defined and acts as a SEAL layer function entity.

### 7.x.2	On-network functional model for AIoTApp Service w/ new application enabler

Ambient power-enabled IoT devices or AIoT devices are of low complexity, small size, lower power consumption, having limited memory capacity, less compute power, and lacking advanced capabilities, which results in AIoT devices not being able to accommodate and run directly the VAL functionality component (i.e., VAL client) to support AIoT services, nor can they support the application enablement layer client entity (e.g., SEAL client).

For AmbientIoT_App service, upon viewing the generic on-network functional model for SEAL as in the 3GPP TS 23.434[5] clauses 6.2, while there is neither VAL client nor SEAL client providing the service enabler layer support functions from the client side, there should necessarily exist the VAL server, i.e., AIoT application server or the 3rd-party AS/AF, and the SEAL layer enabler (or server). The SEAL server communicates with the VAL server via the SEAL-S reference point. However, due to the lack of the corresponding client-side service enabler layer functional entities (e.g., VAL client, SEAL-client), a SEAL server may only communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system, i.e., either the N33 via NEF (for an untrusted AF), or the interface off the SA2-defined AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF (as in the 3GPP TS 23.369[3]).

Figure 7.x.2-1: On-network functional model for AIoTApp service

As shown in the Fig. 7.x.2-1, for the unique (one-side) on-network functional model, the application enablement architecture layer can only be enhanced at the server side (i.e., the right-hand side of the picture) of the application enabler for Ambient IoT service. A new application enabler, i.e., AIoTApp, is introduced to act as a SEAL layer function entity. The AIoTApp enabler function provides services to VAL server over the AIoTApp-S reference point. The detailed 5GC architecture to support the AIoT service is shown in the TS 23.369 [3].

### NOTE 1:	The AIoTApp-N interface is either the N33 via NEF (for an untrusted AF), or the interface off the SA2-defined AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF as defined in the 3GPP TS 23.369 [3].

### 7.x.3	Off-network functional model for AIoTApp Service

The off-network functional model for AmbientIoT application enablement architecture is not supported in this release.

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *