23GPP TSG-SA WG6 Meeting #69	S6-254649

Wuhan, China 13th – 17th October 2025	(revision of S6-254642, S6-254291)

Source:	Samsung, China Mobile

Title:	New Solution for support of traffic identification and differentiated QoS for multiplexed media flows

Spec:	3GPP TR 23.700-51 V1.0.0

Agenda item:	9.5

Document for:	Approval

Contact:	jerry.bae@samsung.com

1. Introduction

This paper introduces a solution for the KI#5 to support of traffic identification and differentiated QoS for multiplexed media flows for XR applications.

2. Reason for Change

This solution addresses on application enabler layer enhancement to support of traffic identification and differentiated QoS for multiplexed media flows for XR applications.

The SEALDD server may receive individual QoS requirements for each individual RTP media flows within the multiplexed data flow for QoS differentiation of the multiplexed media flow at the service request of the VAL server and SEALDD server provides (S)RTP Multiplexed Media Identification Information associated with the individual QoS requirements to the 5GC. In addition, the SEALDD server may receive the (S)RTP Multiplexed Media Identification Information from the VAL server.

The SEALDD server may provides (S)RTP Multiplexed Media Identification Information and individual QoS requirements for supporting QoS differentiation of the multiplexed media flow related to the downlink and uplink to the 5GC and then generate SEALDD traffic descriptor based on the corresponding notification from 5GC.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V1.0.0.

* * * First Change * * * *

5	Solutions

5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |  |
| Sol #2 |  | x |  |  |  |
| Sol #3 |  |  | x |  |  |
| Sol #4 |  |  |  | x |  |
| Sol #5 | x |  |  |  |  |
| Sol #6 | x |  |  |  |  |
| Sol #x |  |  |  |  | x |

* * * Next Change * * * *

5.x	Solution #x: Traffic identification and differentiated QoS for multiplexed media flows for XR applications.

5.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [x].

5.x.2	Solution description

This solution addresses on application enabler layer enhancement to support of traffic identification and differentiated QoS for multiplexed media flows for XR applications.

In Release 19, SA2 has defined that XR and interactive media services can send data traffic of different media components with different QoS requirements and several media flows could be multiplexed on the same end-to-end transport layer connection. In order to uniquely identify each media flow, the IP Packet Filter can further include multiplexed media specific (S)RTP Multiplexed Media Identification Information to differentiate the media flow among multiple media flows that share the same transport layer related elements in the packet filter.

The AF may provide individual QoS requirements for each individual RTP media flow within the multiplexed data flows and for the (S)RTCP that controls the (S)RTP transmission, by including the (S)RTP Multiplexed Media Identification Information associated with the individual QoS requirements within the flow description in the "AF session with the required QoS" procedure specified in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502.

In general, it is assumed that the PSA-UPF support IP Packet Filter with (S)RTP Multiplexed Media Identification Information for for downlink multiplexed media flow, but the differentiated QoS for uplink multiplexed media flows may vary depending on the capability of the UE.

The SEALDD server may receive individual QoS requirements for each individual RTP media flows within the multiplexed data flow for QoS differentiation of the multiplexed media flow at the service request of the VAL server and SEALDD server provides (S)RTP Multiplexed Media Identification Information associated with the individual QoS requirements to the 5GC. In addition, the SEALDD server may receive the (S)RTP Multiplexed Media Identification Information from the VAL server.

The SEALDD server may provides (S)RTP Multiplexed Media Identification Information and individual QoS requirements for supporting QoS differentiation of the multiplexed media flow related to the downlink and uplink to the 5GC and then generate SEALDD traffic descriptor based on the corresponding notification from 5GC.

When the AF request includes media flows with an uplink direction and the UE has not indicated support for (S)RTP Multiplexed Media Identification Information in IP Packet Filters, the PCF may, based on operator configuration, reject the AF request indicating lack of UE support for (S)RTP Multiplexed Media Identification or generate PCC rules and notify the AF that the differentiated QoS handling for multiplexed media flows is not supported by the UE. After receiving the notification that the UE does not support the (S)RTP Multiplexed Media Identification Information from the 5GC, the SEALDD server may determine the descriptor of the uplink traffic based on the policy received from the VAL server. For example, if SEALDD server may receive a notification from the 5GC that the the differentiated QoS handling for multiplexed media flows is not supported by the UE, SEALDD server sends the (S)RTP Multiplexed Media Identification Information in SEALDD traffic descriptors to the SEALDD client for supporting differentiated QoS handling for multiplexed media in the multi-modal transmission connection response. The SEALDD client may perform traffic identification based on (S)RTP multiplexed media identification information and map the identified traffic to each sealdd-uu flow based on SEALDD traffic descriptors.

5.x.2.1	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [x] can be enhanced (highlighted in bold italics) as follows.

9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal ata transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. The VAL server may request QoS differentiation for the multiplexed application flow to the SEALDD server. In order to differentiate the QoS of the multiplexed application flow, the VAL server may include (S)RTP Multiplexed Media Identification Information to differentiate the RTP media (e.g. video or audio) within mulitplexed application traffic, and QoS requirements for each RTP media as in the clause 5.37.11 of TS 23.501[3]. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. The AF request may also include (S)RTP Multiplexed Media Identification Information in the Flow Description and separate QoS requirements per media flow to enalbe differentiated QoS handling for multiplexed media flows. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4. If the VAL server requests QoS differentiation of the multiplexed application flow for UL in step 1 and SEALDD server may receive a notification from the 5GC that the the differentiated QoS handling for multiplexed media flows is not supported by the UE in step 2, SEALDD server sends the (S)RTP Multiplexed Media Identification Information in SEALDD traffic descriptors to the SEALDD client for supporting differentiated QoS handling for multiplexed media in the multi-modal transmission connection response. The SEALDD client may perform traffic identification based on (S)RTP multiplexed media identification information and map the identified traffic to each SEALDD-UU flow. 7-8. Same as step 8-9 of clause 9.2.2.2. NOTE 1: If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). Note 2: Enhance on procedure 9.12.2.1.2 or initiate a new procedure to support traffic identification and differentiated QoS for multiplexed media flows will be addressed in normative work.

* * * Next Change * * * *

5.x.2.2	Enhancement to existing SEALDD information flows

9.12.3.1	SEALDD enabled multi-modal transmission request

Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service.

Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| List of requested flows | M | List of the requested multi-modal flows |
| >SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| >QoS information | O | QoS information provided by VAL server |
| >Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| > Multiplexed media flow info | O | The multiplexed media flow info includes the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement as specified in the clause 5.37.11 of TS 23.501[3]. |

9.12.3.3	SEALDD multi-modal transmission connection establishment request

Table 9.12.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the multi-modal transmission connection establishment.

Table 9.12.3.3-1: SEALDD multi-modal transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flows info | M | Information of the list of SEALDD-UU flows. |
| > SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| > SEALDD traffic descriptor | M | SEALDD traffic descriptor for the flow of the SEALDD client side used to establish SEALDD multi-modal XR connection. |
| >> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >> Transport layer protocol | M | Indicates the transport layer protocol. |
| VAL server ID | O | Identity of the VAL server. |
| VAL service ID | O | Identity of the VAL service. |

9.12.3.4	SEALDD multi-modal transmission connection establishment response

Table 9.12.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the multi-modal transmission connection establishment.

Table 9.12.3.4-1: SEALDD multi-modal transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| Multi-modal SEALDD flow info | M | Information of the Multi-modal SEALDD-UU flow associated with the multi-modal XR connection. |
| > Multi-modal SEALDD flow ID | M | Identity of the Multi-modal SEALDD-UU flow associated with the multi-modal service connection. |
| > SEALDD-UU flows list | M | Indicates the list of SEALDD-UU flows. |
| >> SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| >> SEALDD traffic descriptors | M | SEALDD traffic descriptors for the flows of the SEALDD server side used to establish SEALDD multi-modal connection. |
| >>> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >>> Transport layer protocol | M | Indicates the transport layer protocol. |
| >>> Multiplexed media flow info | O | Indicates the (S)RTP Multiplexed Media Identification Information |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

5.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * END Change * * * *