3GPP TSG-SA WG6 Meeting #69	S6-254739

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254235, 4685

Source:	Ericsson, KPN N.V., ETRI

Title:	New Solution for KI#3 on Use Sensing Results for Creating Spatial Map

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #3 (TR 23.700-15 clause 4.3), the study aspects on use of sensing results for spatial maps include:

- Whether and how the enabler layer can use sensing results for determining the presence of objects (e.g. humans, animals, and vehicles) within an area of interest in a spatial map.

- Whether and how the enabler layer can associate a sensing result with an object within a spatial map.

- Whether and how the enabler layer can localize objects and detect object movements within a spatial map from sensing results.

- How the enabler layer can support SM server with creating SM.

The Spatial Map Server can benefit from 3GPP and non-3GPP sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map. Thus, this pCR proposes to also consider non-3GPP sensing results and sensing application enablement service (SAE, which is introduced in S6-254233) for creating spatial map.

2. Reason for Change

To address the study aspect on “How the enabler layer can support SM server with creating SM” in Key Issue #3, this pCR proposes a new solution on use combination of 3GPP and non-3GPP sensing results for creating spatial map.

3. Conclusions

This paper proposes solution on combination of 3GPP and non-3GPP sensing results for creating spatial map.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X1 |  |  | X |  |

* * * Next Change * * * *

## 6.X1	Solution #X1: Use Sensing Results for Creating Spatial Map

### 6.X1.1	Solution Description

#### 6.X1.1.0	General

The Spatial Map Server can benefit from sensing (including 3rd Party sensing services) data or results to build high-quality Spatial Map.

The following clauses specify use combination of sensing results for creating spatial map.

#### 6.X1.1.1	Procedure

Figure 6.X1.1.1-1: Sensing results for creating spatial map

- The Spatial Map Server sends sensing result request for consolidated sensing results from the SEAL Sensing Server.

- The SEAL Sensing Server authenticates the request, and determines the sensing entities (e.g., 5GC NFs, 3rd party sensing server) for collection of sensing data.

- The SEAL Sensing Server responds to the SM server for the sensing result request.

- The SEAL Sensing Server interacts with the NEF to collect the sensing result measured by the 3GPP network. The SEAL Sensing Server determines whether the sensing result fulfils the request from the consumer provided in step 1, and determines whether additional sensing data is needed to enhance the sensing result.

- The SEAL Sensing Server interacts with the 3rd party sensing server to collect third party sensing data measured.

Editor’s Note:	Whether the revision of the architecture is needed for the 3rd party sensing server is FFS.

- The SEAL Sensing Server consolidates the sensing data/results from different sources in steps 4-5 to generate sensing result (e.g. draft spatial map).

- The SEAL Sensing Server sends sensing result response to the SM server with the consolidated sensing results.

- The Spatial Map Server creates spatial map based on the consolidated sensing result.

#### 6.X1.1.2	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X1.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X1.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***