3GPP TSG-SA WG6 Meeting #69	S6-254512

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254225

Source:	Ericsson

Title:	New Solution for KI#1 on New Service on Support Energy-Related Data Collection

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-44 clause 5.1), the study aspects on enhance application enablement layer architecture and services to support energy saving include:

- Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

According to the use cases and comment functions identified in S6-254224, service on energy-related data collection is needed for supporting energy saving. This pCR proposes new Energy Saving Enablement (ESE) service to support energy-related data collection.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1 by introducing a new Energy Saving Enablement (ESE) service to support energy-related data collection.

3. Conclusions

This paper proposes solution on new Energy Saving Enablement (ESE) service to support energy-related data collection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #X1 | X |  |  |  |  |  |

* * * Next Change * * * *

## 6.X1	Solution #X1: New Service on Support Energy-Related Data Collection

### 6.X1.1	Solution Description

#### 6.X1.1.0	General

The following clauses specify the procedure and information flow of ESE (Energy Saving Enablement) service on energy-related data collection for support energy saving.

Editor’s Note:	Whether a standalone architecture for energy saving enablement or integrate energy saving functionality with the existing SEAL enablers will be decided during solution evaluation and conclusion stage of the study phase.

#### 6.X1.1.1	Procedure

Figure 6.X1.1.1-1: Energy-related data collection

- A consumer (e.g., VAL server, SEAL server/client) sends energy-related data collection request to ESE server. The request includes information as described in Table 6.X1.1.2.2-1.

Editor’s Note:	Whether the ESE server is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

Editor’s Note:	Whether energy-related data collection via new energy saving enablement service/functionality or via existing enablement service (e.g. A-DCCF) is FFS.

- The ESE server authenticates and authorizes the request. If authorized, the ESE server determines the data source (e.g. EIF, ESE client, and/or other network entities (e.g. OAM, 3rd party energy entity)) for collecting the energy-related data. It is assumed that ESE server has a registry with the data sources from which it can collect energy-related data and what kind of data they can provide.

- The ESE sends energy-related data collection response to the consumer. The response includes information as described in Table 6.X1.1.2.3-1.

4.	The ESE server collects energy-related data from the data source.

4a.	The ESE server may interact with 5GC NF for energy-related data collection, by using the EIF service introduced in 3GPP TS 23.501 [3].

4b.	The ESE server may collect energy-related data from ESE client(s).

Editor’s Note:	Whether energy-related information can be collected from other network entities (e.g. OAM, 3rd party energy entity) is FFS.

5.	The ESE server aggregates the collected energy-related data. Aggregation includes combining or performing a statistical operation on the received data. If necessary, steps 4-5 are repeated to complete all the required data collection.

6.	The ESE server notifies the consumer the energy-related data collection result. The notification includes information as described in Table 6.X1.1.2.4-1.

#### 6.X1.1.2	Information Flows

##### 6.X1.1.2.1	General

The following information flows are specified for energy-related data collection.

##### 6.X1.1.2.2	Energy-related data collection request

Table 6.X1.1.2.2-1 shows the request sent by an ESE service consumer to an ESE server for energy-related data collection procedure.

Table 6.X1.1.2.2-1: Energy-related data collection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Target entity ID(s) | M | The identity(ies) of the target data collection entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Data collection requirements | M | Identifies the requirement for energy-related data collection. |
| >Energy consumption | O (NOTE 1) | Identifies the requirement on energy consumption to be collected (e.g. energy consumption per application, energy consumption per task operation). |
| >Energy status | O (NOTE 1) | Identifies the requirement on energy status to be collected (e.g. total remaining energy). |
| >Time of Interest | M | Indicate the interested time for the data collection. |
| Area of Interest | O | The geographical or service area for which the request applies. |
| Reporting requirements | O | It describes the requirements for reporting. This requirement may include e.g. the type of reporting (periodic or one-time report), the reporting periodicity in case of periodic. |
| Data processing requirements | O (NOTE 2) | Indicates the requirements on data processing, as defined in Table 6.X1.1.2.2-2. |
| Additional info | O | Identifies addition requirement of the data to be collected (e.g. time stamp of the data, quality of the data). |
| NOTE 1: At least one of the IEs shall be provided. NOTE 2: The IE shall be provided if requires service on energy-related data combination/aggregation. |  |  |

Table 6.X1.1.2.2-2: Data processing requirements

| Information element | Status | Description |
| --- | --- | --- |
| Processing type | M | Identifies the processing type, e.g. raw-data, aggregation. |
| Aggregation requirements | O | Identifies the requirements for aggregation based on the energy-related data. |
| >Average | O (NOTE) | Indicates that calculate the average value of the energy-related data. |
| >Minimum | O (NOTE) | Indicates that calculate the minimum value of the energy-related data. |
| >Maximum | O (NOTE) | Indicates that calculate the maximum value of the energy-related data. |
| Granularity | M | Identifies the granularity for the processing, e.g. per entity level, or per application level, or per application operation level. |
| NOTE: At least one of the IEs shall be provided if the Processing type is calculation. |  |  |

##### 6.X1.1.2.3	Energy-related data collection response

Table 6.X1.1.2.3-1 shows the response sent by the ESE server to the consumer for energy-related data collection procedure.

Table 6.X1.1.2.3-1: Energy-related data collection response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the Result is positive acknowledgement. |

##### 6.X1.1.2.4	Energy-related data collection notify

Table 6.X1.1.2.4-1 shows the notification sent by the ESE server to the consumer for energy-related data collection procedure.

Table 6.X1.1.2.4-1: Energy-related data collection notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | M (NOTE 1) | Indicates that data collection was successful. |
| >Data collection result | O (NOTE 2) | Provides outputs for data collection result, it may also include the additional information of the collected data based on request. |
| >Aggregated result | O (NOTE 3) | Provides outputs for aggregated result, it may also include the additional information of the collected data and processing information based on request. |
| Failure response | O (NOTE 1) | Indicates that data collection was failed. |
| >Cause | M | Reason for the failure. |
| NOTE 1: One of the information elements shall be provided in the output. NOTE 2: The IE shall be provided if no processing requirements in the Energy-related data collection request. NOTE 3: The IE shall be provided if processing requirements are included in the Energy-related data collection request. |  |  |

### 6.X1.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X1.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***