3GPP TSG-SA WG6 Meeting #69	S6-254729

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254229, 4520

Source:	Ericsson

Title:	New Solution for KI#4 on Energy Saving in AI/ML Task Transfer

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #4.

2. Reason for Change

As described in TR 23.700-44, clause 5.4, there is an open issue on support for AIMLE enhancements for AIML services for energy saving, which includes the study aspects:

- Assistance in AI/ML task transfer

- How to collect and utilize energy consumption data (and which data) from the UE side to support an application layer ML task (or ML model lifecycle / workflow operation).

- How to enhance functionality and architecture of AIMLE to support energy saving.

- Whether and how to use energy consumption information to fulfill FL member selection, AIMLE client selection, split AI/ML operation, etc

As mentioned in the above open issues, there is a lack of solutions to support the AIMLE enhancements for AIML services for energy saving. A solution to address the key issue #4 on support for the AIMLE enhancements in AIML services for energy saving in AI/ML task transfer is proposed in this pCR. The application enablement service on ESE (Energy Saving Enablement) introduced in S6-254224 is considered for the sulution.

3. Conclusions

A solution for energy saving in AI/ML task transfer is proposed to address the key issue #4.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #y |  |  |  | X |  |  |

* * * Next Change * * * *

## 6.y	Solution #y: Energy saving in AI/ML task transfer

This solution corresponds to the key issue #4 described in TR 23.700-44, on energy saving in AI/ML task transfer.

### 6.y.1	Solution Description

#### 6.y.1.0	General

According to the key issue #4, there is a need for AIMLE enhancements for AIML services such as AI/ML task transfer. This solution enables the AIMLE Server to select the target AI/ML member(s) for the AI/ML task transfer, based on energy consumption related information of the target AI/ML member(s) and the energy saving assistance information from the ESE server.

#### 6.y.1.1	Energy saving in AI/ML task transfer procedure

Figure 6.y.1.1-1: Energy saving support for requesting AIMLE server assist AI/ML task transfer

Figure 6.y.1.1-1 illustrates the procedure for requesting AIMLE Server to assist the energy saving for AI/ML task transfer. The detailed corresponding procedure is detailed as follows:

- The source AI/ML member (e.g., an AIMLE Client) sends an AI/ML task transfer assist request to AIMLE Server for assisting with the AI/ML task transfer based on energy saving. To select a target AI/ML member(s), the source AI/ML member set energy consumption criteria for the target AI/ML member(s) such as energy flag, energy consumption level, AIML target traffic volume, and maximum energy cost.

2.	The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the energy criteria and other requirements from the source AI/ML member.

3.	The AIMLE Server interacts with the ESE server for the energy saving assistance information for the list of target AI/ML member(s) that is selected in step 2. The ESE server generates energy saving assistance information for the list of target AI/ML member(s) and provides it to the AIMLE Server.

Editor’s Note:	Whether the ESE server is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

4.	The AIMLE Server collects the energy saving related information for the requested target AI/ML member(s) and selects the most energy efficient AI/ML member as the target AI/ML member based on the collected information from the ESE server.

The AIMLE Server sends request to the selected target AI/ML member(s) for the AI/ML task transfer. Then, the AIMLE Server determines the transfer mode, i.e., transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member. For example, if the task is model training, the AIMLE Server determines the target AI/ML member(s) based on AI/ML member(s)’s capability in the AIMLE client profile, task's AI/ML model training requirement, and the energy status. The task’s AI/ML model training requirement indicates the AI/ML Information.

If target AI/ML member(s) is selected, the AIMLE Server calculate the total energy cost for the AI/ML task transfer, and generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s).

5.	The AIMLE Server sends an AI/ML task transfer assist response to the source AI/ML member with the information on the target AI/ML member(s), the assistance information, and the energy cost generated in step 4.

The information flow for AI/ML task transfer as specified in clause 8.6.3 of 3GPP TS 23.482[x] is enhanced as follows (new text in bold italics):

#### 8.6.3.2 AI/ML task transfer assist request Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server. Table 8.6.3.2-1: AI/ML task transfer assist request Information element; Status; Description. Requestor identity; M; The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE).. VAL service ID; O; The identifier of the VAL service for which the assistance information is requested.. AI/ML Task Type; M; The type of the AI/ML operation (e.g. ML model training).. AI/ML Information Type; M; The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results).. > AI/ML model remaining training requirement (NOTE); O; Indicate the requirement for AI/ML model training including, required remaining training resource, required remaining training number of iterations,. > AI/ML intermediate information (NOTE); O; Indicate the AI/ML intermediate information for intermediate AI/ML operation and result (e.g. ML model training), including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used training number of iterations.. Target AI/ML member energy criterion; M; Specifies the energy consumption criteria (e.g., energy flag, energy consumption level, AIML target traffic volume, max energy cost) of the target AI/ML member.. Time validity; O; The time validity of the request.. NOTE: The IE only present when AI/ML Task Type is ML model training.. #### 8.6.3.3 AI/ML task transfer assist response Table 8.6.3.3-1 describes information elements for the AI/ML task transfer assist response from the AIMLE Server to the source AI/ML member. Table 8.6.3.3-1: AI/ML task transfer assist response Information element; Status; Description. Transfer Mode; O; Indication the transfer mode (e.g. direct transfer).. Target AI/ML member(s); M; The identifier of the target AI/ML member (e.g. identifier of the target AIMLE client or the VAL UE).. Assistance Information; M; Assistance information for the AI/ML task transfer (e.g. time window for the transfer)..

### 6.y.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.y.3	Solution evaluation

Editor’s Note:	The solution evaluation of the solution is FFS.

* * * End of Changes * * * *