3GPP TSG-SA WG6 Meeting #69	S6-254751

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New solution for supporting monitoring requests for AIoT devices

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to support the monitoring requests for AIoT devices.

2. Reason for Change

Some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate the right trajectory/direction.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- 	Monitoring the status (e.g. enable, disable) of requested AIoT devices;

- 	Monitoring the location of requested AIoT devices;

- 	Report the trajectory or direction for the target AIoT devices;

- 	Report to indicate the target AIoT device is deviating or will deviate the right trajectory/direction.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.x	Solution #x: Support of monitoring requests for AIoT devices

7.x.1	Solution description

7.x.1.1	General

This solution addresses the KI#2 and KI#3.

Some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate the right trajectory/direction.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

- 	Monitoring the status (e.g. enable, disable) of requested AIoT devices;

- 	Monitoring the location of requested AIoT devices;

- 	Report the location trajectory for the target AIoT devices;

- 	Report to indicate the target AIoT device is deviating or will deviate the right location trajectory.

7.x.1.2	Procedure of Monitoring requests for AIoT devices

Figure 7.x.1.2-1 illustrates the high-level procedure of monitoring requests for AIoT devices.

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services.

Figure 7.x.1.2-1: Procedure of Monitoring requests for AIoT devices

1.	The VAL server sends an AIoT monitoring subscription request to the AIoT enabler, including the monitoring service type (e.g., location), service ID, target area information, target AIoT device information, triggering conditions (e.g., periodic or event triggered reporting), time intervals for periodic reporting, monitoring time duration, AIoT device group member deviation indicator, the group ID for a group of AIoT devices if the AIoT device group member deviation indicator is included, etc.

The AIoT device group member deviation indicator indicates whether and which AIoT device (or type of AIoT devices) is deviating (or will deviate) from other similar devices in a group.

The information about the target AIoT Device(s) may include Filtering Information as described in clause 5.8 of 3GPP TS 23.369[3].

2.	The AIoT enabler checks whether the VAL server is authorized to request the AIoT monitoring subscription request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the AIoT enabler sends an AIoT monitoring subscription response to the VAL server.

4.	The AIoT enabler invokes Nnef_AIoT_Command request to the 3GPP CN periodically as specified in clause 6.2.3 of 3GPP TS 23.369 [3] to obtain the requested AIoT device data including the location information of AIoT devices. The Command type is Read.

5.	Upon received the AIoT device data and related location information, the AIoT enabler stores and analyses them based on the AIoT monitoring subscription request. If the subscription request includes the location-related UE group analytics indication, the AIoT enabler may interact with ADAES to obtain the predicated location trajectory and the deviation analytics for the requested AIoT devices as specified in clause 8.15 of 3GPP TS 23.436 [23436]. The AIoT enabler will send the obtained AIoT device data to the ADAES and ADAES doesn’t need to sends a data collection subscription request to the Data Producer. Based on the received AIoT device data, the ADAE server sends location-related UE group analytics notifications to the AIoT enabler which may include the predicated location trajectory and the deviation analytics for the group of AIoT devices. The deviation analytics mainly indicate which AIoT device (or type of AIoT devices) is deviating (or will deviate) from other similar devices or the target devices in a group.

6. The AIoT enabler checks if the received location-related analytics in step 5 could meet the service requirements or not. And if not, the step 4 and step 5 may need to be operated repeatedly.

7.	The AIoT enabler reports the monitoring results for the requested AIoT device to the VAL server via sending AIoT monitoring notification message. The report may include the AIoT device ID, the location information for the AIoT device, the location trajectory, the predicted location trajectory for the AIoT device, the deviation analytics, the device availability status, etc.

Editor’s note: Whether and how the procedures of AIoT Inventory subscription and AIoT monitoring subscription can be combined or not is FFS.

7.x.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, please check the related solution for KI#1 for more details.

7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for ambient power-enabled IoT, Stage 1".

[3]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things, Stage 2".

[4]	3GPP TR 23.700-30: "Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2”.

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[23436]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

* * * End of Change * * * *