3GPP TSG-SA WG6 Meeting #69	S6-254778

Wuhan, China 13th – 17th October 2025	(revision of S6-254758)

Source:	Ericsson

Title:	Pseudo-CR on new KIs on Roaming Considerations for Service API Invocation

Spec:	3GPP TR 23.700-43 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides two new key issues for TR 23.700-43 on enhancements required to CAPIF to support gaps identified from use cases and scenarios from SDOs/industry forums, e.g.:

-	how to avoid that a service API invocation is served when the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network; or

-	how to avoid that a service API invocation is served when the UE is out of a designated service area.

2. Reason for Change

Roaming considerations for Service API Invocation

One existing gap in current deployments, detected by field experiences, is how to avoid in a controlled way that a service API invocation is served when, e.g., the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network or indicates that the service should not be delivered over a visiting network (e.g., a QoS boost is not enforced in a roaming network).

Another problem exists when, e.g., the application invoker preferences to access a service API depends on whether the UE is roaming.

To ensure correct handling of Service API invocations, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies and application constraints. To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to become aware of the serving PLMN of the UE. To avoid increased complexity in the API Provider domain exposed service APIs and fragmentation derived from different solutions for roaming handling (e.g. per API Provider domain), the service API should not be forced either to become aware of the serving PLMN of the UE.

A solution in this area would enhance CAPIF to enable the control of service API invocations when the UE is roaming. The solution should consider efficient retrieval of roaming information from the Core Network (e.g. avoiding separated interactions with Core Network per service API invocation) and should keep CAPIF agnostic of the access network (i.e., agnostic of whether the access network is 5G, 4G, etc.).

Service Area considerations for Service API Invocation

Another challenge is how to ensure that a service API is only invoked when the UE is within a designated service area aligned e.g. with the application preferences. Some services are only valid or cost-effective when the UE is physically inside a predefined service area or meets application-specific location preferences. Today, API invokers must monitor UE presence and prevent service API invocations when the UE is outside the service area, which increases complexity and slows time-to-market for application developers.

The key objective of this Key Issue would be to develop an efficient way to determine the UE’s presence within this service area to prevent unnecesary or unsupported service API invocations.

A CAPIF based solution in this area will simplify the integration of service APIs by removing the burden from the API invoker to constantly track user location, and therefore improving operational efficiency.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.1.0)

* * * First Change * * * *

# 4	Key issues

## 4.x	Key issue #Y: Roaming Considerations for Service API Invocation

### 4.x.1	Description

One existing gap in current deployments, detected by field experiences, is how to avoid in a controlled way that a service API invocation is served when, e.g., the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network or indicates that the service should not be delivered over a visiting network.

E.g., a QoS boost is not enforced in a roaming network:

PLMN A may define a strict QoS policy for inbound roamers, to ensure the network capacity is mainly enjoyed by home users. PLMN B, based on the service agreement with the Application Provider X about QoS boost, knows beforehand whether the QoS boost can be applied for its outbound roamers in PLMN A. If PLMN B does not apply roaming control for the QoS boost service API invocation by Application Provider X, the QoS request will be rejected by PLMN A, wasting unnecessarily network resources, having useless inter-PLMN signaling, and having KPIs degradation in both PLMN-A&B since the requests are eventually rejected.

Another problem exists when, e.g., the application invoker preferences to access a service API depends on whether the UE is roaming.

To ensure the correct handling of a Service API invocation, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies and application constraints. To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to be aware of the serving PLMN of the UE. To avoid increased complexity in the API Provider domain exposed service APIs and fragmentation derived from different solutions for roaming handling (e.g. per API Provider domain), the service API should not be forced either to become aware of the serving PLMN of the UE.

A solution in this area would enhance CAPIF to enable the control of service API invocations when the UE is roaming. The solution should consider efficient retrieval of roaming information from the Core Network (e.g. avoiding separated interactions with Core Network per service API invocation) and should keep CAPIF agnostic of the access network (i.e., agnostic of whether the access network is 5G, 4G, etc).

### 4.x.2	Open issues

To support the control of service API invocations when the UE is roaming, the following aspects need to be studied:

-	How to determine and ensure correct handling on Service API invocations, based on:

-	whether roaming agreements and policies between CSPs needed for exposure; and

-	whether application preferences related to roaming of the UE.

NOTE:	This KI ensures to keep CAPIF agnostic of the access network (i.e., agnostic of whether the access network is 5G, 4G, etc.).

Editor's note:	Whether and how handle local access to service APIs in the VPLMN is FFS.

Editor’s Note: Core network roaming aspects in this key issue needs to be updated with the application roaming aspects.

## 4.y	Key issue #Z: Serving Area Considerations for Service API Invocation

### 4.y.1	Description

One existing challenge is how to ensure that a service API is only invoked when the UE is within a designated service area aligned e.g. with the application preferences. Some services are only valid or cost-effective when the UE is physically inside a predefined service area or meets application-specific location preferences. Today, API invokers must monitor UE presence and prevent service API invocations when the UE is outside the service area, which increases complexity and slows time-to-market for application developers.

The key objective of this Key Issue would be to develop an efficient way to determine the UE’s presence within this service area to prevent unnecesary or unsupported service API invocations.

### 4.y.2	Open issues

To support the control of service API invocations when the UE is within or outside a designated service area, the following aspects need to be studied:

-	How to determine and ensure correct handling on Service API invocations based on location of API invoker.

* * * End of Change * * * *

<Proposed change in revision marks>