3GPP TSG-SA WG6 Meeting #69	S6-254521

Wuhan, China 13th – 17th October 2025	(revision of S6-254089)

Source:	InterDigital Inc.

Title:	Pseudo-CR on New solution for AI/ML energy consumption analytics

Spec:	3GPP TR 23.700-44

Agenda item:	9.9

Document for:	Approval

Contact:	Quang Ly <Quang.Ly@InterDigital.com>

1. Introduction

This pCR proposes a new solution for KI#4 on AIMLE enhancements for AI/ML services energy saving.

2. Reason for Change

This contribution proposes a solution to Key Issue #4 – open issue #1 on how to collect and use energy consumption data to support AI/ML energy consumption analytics in ADAE server.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[r23436]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[r26531]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

* * * Next Change * * * *

## 6.x	Solution #x: Support of ADAE analytics for AI/ML energy consumption

### 6.x.1	Solution Description

This solution addresses Key Issue #4 – open issue #1 on how to collect and use energy consumption data. The solution proposes a new ADAE service procedure in 3GPP TS 23.436 [r23436] to support AI/ML energy consumption analytics.

#### 6.x.1.1	Procedure

Figure 6.x.1.1-1 illustrates the procedure for a consumer (e.g., VAL server, AIMLE server) to subscribe for AI/ML energy consumption analytics generated from data collected from VAL UEs while the VAL UEs are performing AI/ML operations.

Pre-conditions:

1.	ADAE Client (ADAEC) is connected to the ADAE Server (ADAES).

2.	Data producers (e.g., VAL Clients) may be pre-configured with data producer profiles for the data they can provide. ADAES and ADAEC have discovered available data producers and their data producer profiles.

Figure 6.x.1.1-1: ADAES support for AI/ML energy consumption analytics

1.	An analytics consumer (e.g. VAL Server, AIMLE Server) sends an AI/ML energy consumption analytics subscription request to an ADAES. The subscription request includes an Analytics ID “AI/ML energy consumption analytics”, ML model identifier, AIML operation, energy consumption metrics, and other information as defined in Table 6.x.1.2.1-1.

2.	The ADAES validates the subscription request and performs authentication and authorization checks to determine if the consumer is authorized to subscribe for the requested analytics. If authorization is successful, the ADAES stores the subscription request information, assigns a subscription ID, and sends a subscription response indicating successful subscription and includes the subscription ID.

3.	The ADAES maps the analytics ID to a list of data collection event identifiers and determines a list of data producer IDs for the data collection event identifiers. The determination of data producer IDs is based on data producer profiles.

4.	The ADAES sends an AI/ML energy consumption data collection subscription request to the ADAECs determined in step 3. The subscription request includes information as defined in Table 6.x.1.2.4-1. Data collection at the VAL UE(s) reuses the EVEX mechanism defined in 3GPP TS 26.531 [r26531].

5.	Each ADAEC configures the data producers (i.e., VAL client or AIMLE client) for collecting data (e.g., duration of AI/ML operation, % resource utilization for AI/ML operation) for AI/ML energy consumption analytics requested in the data collection subscription request.

6.	The ADAEC sends an AI/ML energy consumption data collection subscription response to acknowledge the ADAES. The response includes a status and a subscription ID for the data collection.

7.	The data producers perform AI/ML operations when triggered (e.g., ML model training) using the indicated ML model and collect data for AI/ML energy consumption analytics based on the configured data collection requirements.

8.	At the completion of AI/ML operation, the ADAEC sends an AI/ML energy consumption data collection notification to the ADAES. The data notification includes the data collection subscription ID and the information as defined in Table 6.x.1.2.6-1.

9.	The ADAES performs analytics relevant operations to generate the AI/ML energy consumption analytics based on the data received from the ADAECs. The ADAES uses the data collected by the ADAECs to calculate the AI/ML energy consumption analytics.

NOTE: How the collected data are used to calculate energy consumption is up to implementation.

10.	The ADAES sends an AI/ML energy consumption analytics notification to the consumer. The analytics notification includes the analytics subscription ID and the information as defined in Table 6.x.1.2.3-1.

#### 6.x.1.2	Information flows

##### 6.x.1.2.1	AI/ML energy consumption analytics subscription request

Table 6.x.1.2.1-1 describes the information flow from the consumer (e.g. VAL server, AIMLE server) as a request or update request for AI/ML energy consumption analytics.

Table 6.x.1.2.1-1: AI/ML energy consumption analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer (e.g., VAL server, AIMLE client). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example "AI/ML energy consumption analytics". |
| Analytics type | M | The type of analytics for the event (e.g., statistics or predictions). |
| ML model identifier | M | The identifier of the ML model for which analytics are determined. |
| AIML operation | M | The desired AIML operations (e.g., ML model training, inference) for which analytics are determined. |
| Energy consumption metrics | M | The formula and necessary metrics for calculating the energy consumption based on factors that impact energy consumption for an AIML operation using a particular ML model. |
| Target VAL UE IDs | O | The identifier(s) of VAL UE(s) for which the analytics subscription applies. |
| Target data producer IDs | O | The identifier(s) of VAL clients, AIMLE clients or FL members to target for collecting data for energy consumption analytics. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time validity | O | The time validity of the subscription request. |

##### 6.x.1.2.2	AI/ML energy consumption analytics subscription response

Table 6.x.1.2.2-1 describes the information elements for the AI/ML energy consumption analytics subscription response from the ADAES to the consumer.

Table 6.x.1.2.2-1: AI/ML energy consumption analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics subscription request (positive or negative acknowledgement). |
| Subscription ID | M | An identifier for the subscription. |

##### 6.x.1.2.3	AI/ML energy consumption analytics notification

Table 6.x.1.2.3-1 describes the information flow from the ADAES to the consumer (e.g. VAL Server, AIMLE Server) as a notification for the AI/ML energy consumption analytics.

Table 6.x.1.2.3-1: AI/ML energy consumption analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier for the subscription. |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example " AI/ML energy consumption analytics". |
| ML model identifier | M | The ML model identifier for which analytics are determined. |
| Energy consumption information | M | The range of energy consumption values for a particular ML model with a certain AIML operation and for the indicated VAL UE power profile. The output can be an average, minimum, and/or maximum value for the energy consumption. The output may be a prediction or a statistical value. |
| List of AIML operations | M | The AIML operations for which the analytics are determined. |
| > Energy consumption | M | The range of energy consumption values, per VAL UE power profile (e.g., as defined in Table 6.x.1.2.4-1), for performing an AI/ML operation. |
| >> Maximum consumption | O | The maximum energy consumption value. |
| >> Average consumption | O | The average energy consumption value. |
| >> Minimum consumption | O | The minimum energy consumption value. |
| Target analytics period | O | The time validity of the analytics. |
| Confidence level | O | The level of accuracy for the analytics output (e.g. prediction). |

##### 6.x.1.2.4	AI/ML energy consumption data collection subscription request

Table 6.x.1.2.4-1 describes information elements for the AI/ML energy consumption data collection subscription request from the ADAES to the ADAEC.

Table 6.x.1.2.4-1: AI/ML energy consumption data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the consumer. |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data collection requirements | M | The requirements for data collection and for information to include in the data notification. |
| > ML model identifier | M | The identifier of the ML model for which data is collected. |
| > AIML operations | M | The desired AI/ML operations (e.g., ML model training, inference) for which data is collected. |
| > Power profile | M | A power rating used to indicate the degree of energy consumption a VAL UE utilizes for performing AI/ML operations. The power rating is used to determine the usage rate of energy per unit time that factors into the determination of energy consumption. |
| > AIML operation duration | M | An indication to include the time duration for performing the AI/ML operation. |
| > AIML operation utilization | M | An indication to include the percentage utilization of hardware resources (e.g., compute, memory, storage) used for performing the AI/ML operation. |
| > Time period | M | The time period of data collection. |
| > Location or area information | O | The location or area of the data collection. |

##### 6.x.1.2.5	AI/ML energy consumption data collection subscription response

Table 6.x.1.2.5-1 describes information elements for the AI/ML energy consumption data collection subscription response from the ADAEC to the ADAES.

Table 6.x.1.2.5-1: AI/ML energy consumption data collection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the AI/ML energy consumption data collection subscription request (positive or negative acknowledgement). |
| Subscription ID | M | An identifier for the subscription. |

##### 6.x.1.2.6	AI/ML energy consumption data notification

Table 6.x.1.2.6-1 describes information elements for the AI/ML energy consumption data notification from the ADAEC to the ADAES.

Table 6.x.1.2.6-1: AI/ML energy consumption data collection notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier for the subscription. |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Collected data | M | The collected data. |
| > ML model identifier | M | The identifier of the ML model for which data was collected. |
| > AIML operation | M | The AI/ML operation (e.g., ML model training, inference) for which data was collected. |
| > Power profile | M | The power rating of the VAL UE that represents a VAL UE’s processing capabilities (e.g., entry-level, mid-tier, premium, high performance, high efficiency). The power rating is used to determine the usage rate of energy per unit time that factors into the determination of energy consumption. |
| > Elapse time for AI/ML operation | M | The time duration for performing the AI/ML operation. |
| > % resource utilization for AI/ML operation | M | The percentage utilization of hardware resources (e.g., compute, memory, storage) determined for performing the AI/ML operation. |

### 6.x.2	Architecture impacts

This clause will provide the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *