3GPP TSG RAN 6G Workshop Meeting 	6GWS-250041

Incheon, Republic of Korea, March 10-11th, 2025

Title:	Vision and priorities for 6G system for the support of satellite access

Source: 	Thales

Type:	discussion

Document for:	discussion

Agenda Item:	5 	Vision & priorities for next generation system architecture, core networks and protocols

# Introduction

This document proposes capabilities for 6G system in order to best support satellite access (NTN).

# Recommendations on 6G system for the support of Satellite access (NTN)

## Service

It is assumed that all services targeted by the 6G system may be supported by the 6G NTNs provided that they require round trip delay and bandwidth compatible with the NTN deployment scenario characteristics.

### Broadcasting service

- Technical issue:

  - In NTN, the radio cells may span across country borders while broadcasting services may be subject to national regulations

- Recommendations on 6G radio access capabilities:

  - The 6G system shall provide mechanisms to geo fence a Broadcast service provided by satellite access (i.e. bounded to a targeted geographical area).

## Network architecture

### Mesh connectivity

- Technical issue:

  - service should be available over areas where feeders are not necessarily available (e.g. ocean or when feeder links are not functioning), besides latency should always be optimized especially for co-located UE – UE served by same or adjacent satellites (UE – Satellite – UE).

- Recommendations on 6G radio access capabilities:

  - The 6G system shall support mesh connectivity (UE-SAT-UE) with or without available feeder link while addressing security requirements.

### Network entry/idle mode and connected mode across orbit

- Technical issue:

  - In a multi orbit space segment, it is of interest to serve UEs via GSO and/or NGSO according to RRC connected state, signaling optimization, traffic pattern etc.

- Recommendations on 6G radio access capabilities:

  - The 6G system with a multi orbit satellite network shall support a flexible use of GSO and/or NGSO access to serve UEs.

### Access and backhaul via satellite

- Technical issue:

  - Use the satellite access to also provide backhaul services to local area networks on board aircraft, vessels, trains

- Recommendations on 6G radio access capabilities:

  - The 6G system shall support high capacity backhauled connectivity (up to 1 Gps), including inter-node link management and transport solutions

### Space edge computing

- Technical issue:

  - Be able to provide local services (e.g. content distribution, Integrated Communication And Sensing, etc.) from a specific serving satellite

- Recommendations on 6G radio access capabilities:

  - The 6G system architecture shall support the deployment of edge capabilities on board satellite as well as the transfer of context between successive serving satellites

# Conclusion

Proposal 1: TSG-SA to take into account the above recommendations for the definition of the 6G system.

Proposal 2: TSG-SA to consider in priority the support of Satellite access by the 6G Core Network

Proposal 3: TSG-SA to further discuss in which release of 6G the other recommendations should be addressed

END