3GPP TSG RAN Meeting #109					RP-251960

Beijing, China, Sep 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Ericsson

Title:	Network and UE energy efficiency beyond ITU-R

Document for:	Approval

# 1	Introduction

Energy efficiency is a key requirement for both the base station (BS) and devices (UE). ITU-R WP5D is currently defining minimum requirements for KPIs related to energy efficiency for IMT-2030 radio interfaces [2] (see snapshot copied in Annex A). In this contribution we discuss additional methodologies for 3GPP internal studies to evaluate KPIs on energy efficiency going beyond those defined by ITU-R.

# 2	Discussion

Compared with 5G, 6G will introduce, wider bandwidths, and larger antenna arrays. These factors inevitably increase energy demands in both baseband and RF domains for BS and UE. Within 3GPP, we believe energy efficiency should be prioritized as a quantitative KPI evaluated via system-level simulation (SLS) as the baseline methodology. Rather than solely focusing on user perceived throughput (UPT), such evaluations should consider QoS satisfaction for different service types, using relevant metrics such as UPT, latency, or other KPIs depending on the service and scenario under evaluation. Other evaluation methods such as numerical analysis are not precluded when relevant; however, for each evaluated technique, at least one approved method (e.g. SLS, numerical) should be applied.

Define energy efficiency as a quantitative KPI in 3GPP for both network and UE, evaluated via system-level simulations (SLS). Other evaluation methods, such as numerical, are not precluded when relevant.

In many scenarios, QoS-based energy evaluation (latency, reliability, service continuity) is more relevant than throughput/UPT-only evaluation.

Consider QoS-based energy saving metrics depending on the service and scenario under evaluation, instead of relying solely on throughput-based metrics.

For SLS, we think a relative evaluation model should be used as in NR, where a technique’s energy savings is quantified by comparing against a baseline for the same scenario and over the same duration. The UE/BS energy consumption models should include slot- and symbol-level power consumption, reflecting bandwidth/time occupancy and Tx/Rx direction of symbols in a slot. System-level results can be reported per-slot or averaged across slots, regardless of the detailed symbol-level calculation approach.

Adopt NR’s relative energy saving evaluation methodology, comparing new techniques against baseline scenarios over the same duration.

In NR Releases 16-19, extensive work was carried out in 3GPP to develop UE and BS power models:

- UE power consumption models: TR 38.840 serving as NR baseline UE power consumption model, while extensions for low-power-wake-up receivers, reduced capability devices, and other techniques are captured in TRs 38.869 and 38.875. We believe these should remain the baseline for 6G.

- BS power consumption models: TR 38.864 defining BS power consumption model for two BS categories, Category 1, and Category 2.

3GPP has already developed comprehensive BS/UE power models (TR 38.864, TR 38.840, TR 38.869, TR 38.875) which can be reused as the baseline for 6G evaluations.

Adopt the UE power model from TR 38.840 as the baseline, with refinements from TR 38.869 and TR 38.875 where appropriate.

For BS modelling, Category 1 in TR 38.864 should be the baseline for 6G evaluations. Category 1 reflects future-oriented BSs with faster and deeper transitions into energy-saving states, aligned with advanced and flexible 6G deployments. By contrast, Category 2 assumes slower and less efficient transitions, which are not representative of anticipated 6G HW capabilities.

NR’s Category 1 BS power model (as defined in TR 38.864) represents future-ready, flexible, and energy-efficient BSs. Category 2 is not representative of 6G.

Adopt the future-oriented Category 1 BS model from TR 38.864 as the baseline for 6G BS evaluations.

For both UE and BS, some extensions will however be required for 6G, particularly to support 7-15 GHz operation, where existing parameters may not fully apply and will need updating.

Study updates to BS/UE power parameters to support 7 – 15 GHz operation, building on the 5G baseline models.

# 3 	Text proposal for TR 38.914

Based on the discussion above we propose the following text proposal for TR 38.914 [1]:

### 5.1.15	Energy efficiency

Energy efficiency is an important KPI for sustainability and shall be considered as a basic principle in the 6G design. KPIs and methodologies for evaluating them are defined for both network (BS) and device (UE).

#### 5.1.15.1	Network Energy Efficiency

Network energy efficiency is a quantitative KPI evaluated by system simulation. The KPI is defined as the relative energy savings of a given technique quantified by comparing against operation without the technique for the same scenario and over the same time duration. [Details of the relative measure are FFS].

The KPI is defined based on quality of service (QoS) satisfaction at the UE for a given service type and scenario under evaluation. [Details of QoS definition are FFS]

For base station energy consumption modeling, the relative power and transition time/energy values for BS Cateogry 1 in 3GPP TR 38.864 are used as baseline for 6G evaluations. [Possible updates of the modeling for 6G, e.g., for new frequency ranges supported for 6G, are FFS].

#### 5.1.15.2 	UE Energy Efficiency

UE energy efficiency is a quantitative KPI evaluated by system simulation. The KPI is defined as the relative energy savings of a given technique quantified by comparing against operation without the technique for the same scenario and over the same time duration. [Details of the relative measure are FFS].

The KPI is defined based on quality of service (QoS) satisfaction at the UE for a given service type and scenario under evaluation. [Details of QoS definition are FFS]

For UE energy consumption, the power models and methodology in 3GPP TR 38.840 and 3GPP TR 38.875 are used as baseline for 6G evaluations. For evaluation of LP-WUS/WUR techniques, the models in TR 38.869 are used as baseline. [Possible updates of the modeling for 6G are FFS].

# 4	Conclusion

In this contribution we propose to add the text proposal in Section 3 to TR 38.914 Clause 5.1.15.

# References

3GPP TSG RAN TR 38.914 v.0.1.0, “Study on 6G Scenarios and requirements”, June 2025.

ITU-R, WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT 
NEW REPORT ITU-R M.[IMT-2030.TECH PERF REQ], “Minimum requirements related to technical performance for IMT-2030 radio interface(s),” Annex 5.14 to Document 5D/792-E, July 11, 2025.

# Annex A: Snapshot of ITU-R minimum requirements related to energy efficiency for IMT-2030 radio interface(s) [2]

## 4.15	Energy efficiency for sustainability

Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics].

The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

The minimum requirements for Energy efficiency are summarized in Tables below.

[Editor’s note: WP 5D #49 meeting agreed that Network and UE Energy efficiency in unloaded case compared to fully loaded case is evaluated analytically using power models. On the evaluation of partial loaded case(s) (low load, light load and/or medium load), there are divergent views including mandatory, optional or not included, as well as the evaluation method of analysis or simulations. Proponents are encouraged to submit more details including load level (i.e. A/B/C in tables), requirement value (i.e. X/Y in tables) and the associated evaluation method, to facilitate the discussion at WP 5D #50 meeting.]

Table X

The minimum requirements for network energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy [saving/consumption] (%) | [Editor notes: Mandatory, optional or not included – this is not part of the final document |
| --- | --- | --- | --- | --- |
| N. A | empty load | Fully loaded case* | [X1%] | Mandatory |
| [Dense urban-IC]** | [A%] |  | [X2%] | TBD |
| [Rural-IC]** | [B%] |  | [X3%] | TBD |
| * Empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100. ** Needed only in case of simulation. |  |  |  |  |

Table X

The minimum requirements for device energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy [saving/consumption] (%) | [Editor notes: Mandatory, optional or not included - this is not part of the final document] |
| --- | --- | --- | --- | --- |
| N. A | empty load | Fully loaded case | [Y1%] | Mandatory |
| [Dense urban-IC]** | [C%] |  | [Y2%] | TBD |
| * Empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100. ** Needed only in case of simulation. |  |  |  |  |

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].